| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0493 |
| --- | --- |

| Title: | AHG12: On independent regions and output sub-picture sets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Sühring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to add a new SEI message to VVC that carries nested SEI messages with the scope of one or more Output Sub-Picture Sets.

The contribution asserts that such a signaling mechanism is required to allow conformance checking of independently coded regions such as sub pictures as well as for replacement of SEI messages when such sub pictures are extracted.

# Introduction

In VVC, independently coded region within a picture are currently only supported through encoder-side constraints in VTM without bitstream indication. However, a CE at the N meeting investigated boundary padding for increased coding efficiency with the respective notes stating the following: “The group was inclined to adopt [tile boundary padding], but felt that VTM software should be available before action”. While carriage of independent regions is in principle also possible via multiple independently coded layers (another option in the current VVC draft), such a solution is undesirable for ROI scalability applications wherein a composed picture is to be output by the decoder.

In HEVC, the Motion-constrained tile sets extraction information sets (MCTS EIS) SEI message and the Motion-constrained tile sets extraction information nesting (MCTS EIN) SEI message are specified to enable extraction and conformance testing of MCTS.

In document JVET-O0141, and similar in spirit to document JVET-N0354, it is proposed to add a output sub-picture sets (OSPS) concept allowing identification of independent regions that can serve as sub-bitstreams without the need to replace parameter sets in the style of the HEVC specification. The OSPS would obsolete the MCTS EIS SEI message and allow to transmit correct HRD parameters for independent regions.

However, when OSPS are used to avoid Parameter Set replacement, there still exist the issue of wrongful SEI messages remaining in the bitstream. For instance, the VVC specification currently contains HRD-related SEI messages such as the Buffering Period SEI message or non-HRD related SEI messages such as the Decoded Picture Hash SEI message.

This document proposes to add an SEI message similar in spirit to MCTS EIN SEI message that allows carriage of nested SEI message associated with an OSPS instead of an MCTS (or sets thereof) as in HEVC.

# OSPS nesting SEI message

    - OSPS nesting SEI message syntax

| osps_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| all_osps_flag | u(1) |
| if( !all_osps_flag ) { |  |
| num_associated_osps_minus1 | ue(v) |
| for( i = 0; i <= num_associated_osps_minus1; i++ ) |  |
| idx_of_associated_osps[ i ] | ue(v) |
| } |  |
| num_sei_messages_in_osps_nesting_minus1 | ue(v) |
| while( !byte_aligned( ) ) |  |
| osps_nesting_zero_bit /* equal to 0 */ | u(1) |
| for( i = 0; i <= num_sei_messages_in_osps_nesting_minus1; i++ ) |  |
| sei_message( ) |  |
| } |  |

The OSPS nesting SEI message provides a mechanism to carry and associate the SEI messages with bitstream subsets corresponding to one or more OSPSs. An SEI message contained in an OSPS nesting SEI message is referred to as OSPS-nested or an OSPS-nested SEI message, and an SEI message that is not contained in an OSPS nesting SEI message is referred to as a non-OSPS-nested SEI message.

In the OSPS sub-bitstream extraction process, the OSPS-nested SEI messages applicable to the OSPSs in an OSPS set in an access unit can be included in the corresponding access unit of the extracted sub-bitstream as non-OSPS-nested SEI messages.

[Ed. (RS): The sub-bitstream extraction of independelty coded regions remains to be defined in this contribution]

An SEI NAL unit containing an OSPS nesting SEI message shall not contain any other SEI message that is not OSPS-nested in the OSPS nesting SEI message.

NOTE 1 – A "smart" bitstream extractor might include appropriate non-OSPS-nested SEI messages in the bitstream, provided that the SEI messages applicable to the OSPS sub-bitstream were present as OSPS-nested SEI messages in the original bitstream.

all_osps_flag equal to 0 specifies that the list nesting_ospsid is set to consist of the OSPS indices indicated by all instances of idx_of_associated_osps[ i ] for i from 0 to num_associated_osps_minus1, inclusive. all_osps_flag equal to 1 specifies that the list nesting_ospsid consists of the OSPS indices of all the OSPSs indicated by the active Sequence Parameter Set.

The OSPS-nested SEI messages apply to all OSPSs for which the OSPS indices are included in the list nesting_ospsid.

num_associated_osps_minus1 plus 1 specifies the number of the following OSPS indices. The value of num_associated_osps_minus1 shall be in the range of 0 to 511, inclusive.

idx_of_associated_osps[ i ] indicates the OSPS index of the i-th OSPS associated with the following OSPS-nested SEI messages. The value of idx_of_associated_osps[ i ] shall be in the range of 0 to 511, inclusive.

num_sei_messages_in_osps_nesting_minus1 plus 1 specifies the number of the following OSPS-nested SEI messages.

osps_nesting_zero_bit shall be equal to 0.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).