| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0495 |
| --- | --- |

| Title: | AHG17: On HRD for Open GOP and DRAP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin Karsten Sühring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes changes to the VVC HRD timing model for the case where AUs are dropped from the original bitstream which happens when dropping RASL pictures, for instance  during splicing or using DRAP for random access.

The proposal is twofold:

- Indicating only the initial CPB removal delay for the original/complete bitstream in the Buffering period SEI message but adding indication of the presence of alternative removal times to following Picture Timing SEI messages (similar to irap_cpb_params_present_flag in HEVC) and and indication whether the removal of pictures has happened (same as use_alt_cpb_params_flag in HEVC).

- Opposed to the HEVC alternative timing design, an indication of offsets to original CPB removal times in picture timing SEI messages that applies when AUs are removed from the IRAP up to the AU associated to the picture timing SEI message.

# Introduction

HEVC defines alternative CPB removal syntax elements for open GOP RAPs. More concretely, it specifies initial CPB removal delays and delay offsets that can be used if RASL pictures are dropped either at CRAs or BLAs. If such RASL pictures are removed before transmission, obviously, the timing information, i.e. initial arrival time and removal time of pictures changes when the bitstream is modified compared to the original bitstream.

Therefore, 2 types of values are included into the Buffering Period SEI message in HEVC:

- nal/vcl_initial_cpb_removal_delay and nal/vcl_initial_cpb_removal_offset for the original bitstream

- nal/vcl_initial_alt_cpb_removal_delay and nal/vcl_initial_alt_cpb_removal_offset for the bitstream once the RASL pictures are removed

It is asserted that following the same principle for VVC is not desirable.

First, setting the alternative removal times requires either waiting until all RASL pictures are encoded before deriving the proper values to be written in the Buffering Period SEI (with a delay in sending the associated RAP with the Buffering SEI) or setting initially a value, which needs to be taken into account later at the rate controller, and which could lead to a suboptimal solution.

Second, VVC has already specified DRAPs which potentially leads to a bitstream where dropped AUs are missing and for which it is desirable to verify conformance. Note that the DASH subgroup in systems did not add support for DRAP in DASH as no conformance was defined in video for HEVC [m38198]. For convenience, part of the meeting notes is copied below:

“The video group needs to define signaling and conformance for it when not the entire stream is decoded, i.e the decoder only decodes previous IRAP and then start decoding DRAP.”

Following the same principle for DRAPs as done currently for open GOP RAPs in HEVC would lead to several alternative timings at the Buffering Period SEI message and the drawback related to the delay involved in determining the alternative timings would be even more problematic.

Therefore, this document proposes a harmonized approach that covers both cases for conformance checking after AU dropping (RASL and DRAP) and that solves the described issue related to the delay.

# Proposal

The proposal described below consists of:

- Indicating only the initial CPB removal delay for the original/complete bitstream in the Buffering period SEI message but adding indication of the presence of alternative removal times to following Picture Timing SEI messages (similar to irap_cpb_params_present_flag in HEVC) and and indication whether the removal of pictures has happened (same as use_alt_cpb_params_flag in HEVC).

- Opposed to the HEVC alternative timing design, an indication of offsets to original CPB removal times in picture timing SEI messages that applies when AUs are removed from the IRAP up to the AU associated to the picture timing SEI message.

Buffering period SEI message:

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| bp_seq_parameter_set_id | ue(v) |
| bp_nal_hrd_parameters_present_flag | u(1) |
| bp_vcl_hrd_parameters_present_flag | u(1) |
| if( bp_nal_hrd_parameters_present_flag ¦ ¦ bp_ vcl_hrd_parameters_present_flag ) { |  |
| initial_cpb_removal_delay_length_minus1 | u(5) |
| cpb_removal_delay_length_minus1 | u(5) |
| dpb_output_delay_length_minus1 | u(5) |
| bp_alt_cpb_params_present_flag | u(1) |
| } |  |
| concatenation_flag | u(1) |
| cpb_removal_delay_delta_minus1 | u(v) |
| cpb_removal_delay_deltas_present_flag | u(1) |
| if( cpb_removal_delay_deltas_present_flag ) { |  |
| num_cpb_removal_delay_deltas_minus1 | ue(v) |
| for( i = 0; i <= num_cpb_removal_delay_deltas_minus1; i++ ) |  |
| cpb_removal_delay_delta[ i ] | u(v) |
| } |  |
| bp_cpb_cnt_minus1 | ue(v) |
| if( bp_nal_hrd_parameters_present_flag ) |  |
| for( i = 0; i < bp_cpb_cnt_minus1 + 1; i++ ) { |  |
| nal_initial_cpb_removal_delay[ i ] | u(v) |
| nal_initial_cpb_removal_offset[ i ] | u(v) |
| } |  |
| if( bp_vcl_hrd_parameters_present_flag ) |  |
| for( i = 0; i < bp_cpb_cnt_minus1 + 1; i++ ) { |  |
| vcl_initial_cpb_removal_delay[ i ] | u(v) |
| vcl_initial_cpb_removal_offset[ i ] | u(v) |
| } |  |
| if( bp_alt_cpb_params_present_flag ) |  |
| use_alt_cpb_params_flag | u(1) |
| } |  |

bp_alt_cpb_params_present_flag equal to 1 specifies the presence of the syntax element use_alt_cpb_params_flag in the buffering period SEI. When not present, the value of bp_alt_cpb_params_present_flag is inferred to be equal to 0. When the associated picture is neither a CRA picture nor an IDR picture, the value of bp_alt_cpb_params_present_flag shall be equal to 0.

use_alt_cpb_params_flag may be used to derive the value of UseAltCpbParamsFlag. When use_alt_cpb_params_flag is not present, it is inferred to be equal to 0.

When one or more of the following applies, UseAltCpbParamsFlag is set to 1:

–	use_alt_cpb_params_flag is equal to 1

–	If some external means not specified in this Specification is available and a value equal to 1 is provided by the external means.

Picture Timing SEI message:

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| frame_field_info_present_flag | u(1) |
| if( frame_field_info_present_flag ) { |  |
| pic_struct | u(4) |
| source_scan_type | u(2) |
| duplicate_flag | u(1) |
| } |  |
| if( CpbDpbDelaysPresentFlag ) { |  |
| cpb_removal_delay_minus1[ sps_max_sub_layers_minus1 ] | u(v) |
| if(bp_alt_cpb_params_present_flag) { |  |
| cpb_alt_timing_info_present_flag | u(1) |
| if(cpb_alt_timing_info_present_flag) { |  |
| for( i = 0; i < bp_cpb_cnt_minus1 + 1; i++ ) { |  |
| cpb_alt_initial_cpb_removal_delay_delta | u(v) |
| cpb_alt_initial_cpb_removal_offset_delta | u(v) |
| } |  |
| cpb_delay_offset | u(v) |
| dpb_delay_offset | u(v) |
| } |  |
| } |  |
| for( i = TemporalId; i < sps_max_sub_layers_minus1; i++ ) { |  |
| sub_layer_delays_present_flag[ i ] | u(1) |
| if( sub_layer_delays_present_flag[ i ] ) { |  |
| cpb_removal_delay_delta_enabled_flag[ i ] | u(1) |
| if( cpb_removal_delay_delta_enabled_flag[ i ] ) |  |
| cpb_removal_delay_delta_idx[ i ] | u(v) |
| else |  |
| cpb_removal_delay_minus1[ i ] | u(v) |
| } |  |
| } |  |
| dpb_output_delay | u(v) |
| } |  |
| } |  |

cpb_alt_timing_info_present_flag equal to 1 specifies the presence of the syntax elements cpb_alt_initial_cpb_removal_delay_delta, cpb_delay_offset and dpb_delay_offset. When the associated picture is a RASL picture, the value of cpb_alt_timing_info_present_flag shall be equal to 0.

NOTE X – The value of  cpb_alt_timing_info_present_flag might be equal to 1 for more than one access unit. Only the alternative times of the first access unit following in decoding order the CRA or IDR apply.

cpb_alt_initial_cpb_removal_delay_delta[ i ] specifies the alternative initial CPB removal delay delta for the i-th CPB. The lenght of this syntax element is initial_cpb_removal_delay_length_minus1 + 1 bits.

cpb_alt_initial_cpb_removal_offset_delta[ i ] specifies the alternative initial CPB removal offset delta for the i-th CPB. The lenght of this syntax element is initial_cpb_removal_delay_length_minus1 + 1 bits

cpb_delay_offset specifies an offset to be used in the derivation of the nominal CPB removal times of the access unit associated with the picture timing SEI message and of the access units following in decoding order, when the access unit associated with the picture timing SEI message directly follows in decoding order the access unit associated with the buffering period SEI message. The syntax element has a length in bits given by au_cpb_removal_delay_length_minus1 + 1. When not present, the value of cpb_delay_offset is inferred to be equal to 0.

dpb_delay_offset specifies an offset to be used in the derivation of the DPB output times of the CRA or IDR access unit associated with the buffering period SEI message when the access unit associated with the picture timing SEI message directly follows in decoding order the CRA or IDR associated with the buffering period SEI message. The syntax element has a length in bits given by dpb_output_delay_length_minus1 + 1. When not present, the value of dpb_delay_offset is inferred to be equal to 0.

## Changes to the HRD text.

(C.1)

The syntax elements of non-VCL NAL units (or their default values for some of the syntax elements), required for the HRD, are specified in the semantic clauses of clause 7, Annexes D and E.

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled through the hrd_parameters( ) syntax structure, which may be part of the SPS syntax structure.

Two sets of bitstream conformance tests are needed for checking the conformance of a bitstream, which is referred to as the entire bitstream, denoted as entireBitstream. The first set of bitstream conformance tests are for testing the conformance of the entire bitstream and its temporal subsets.

For each test, the following ordered steps apply in the order listed, followed by the processes described after these steps in this clause:

- An operation point under test, denoted as TargetOp, is selected by selecting a target highest TemporalId value OpTid. The value of OpTid is in the range of 0 to sps_max_sub_layers_minus1, inclusive. The value of OpTid is such that the sub-bitstream BitstreamToDecode that is the output by invoking the sub-bitstream extraction process as specified in clause 10 with entireBitstream and OpTid as inputs satisify the following condition:

–	There is at least one VCL NAL unit with TemporalId equal to OpTid in BitstreamToDecode.

- HighestTid is set equal to OpTid of TargetOp.

- The hrd_parameters( ) syntax structure and the sub_layer_hrd_parameters( ) syntax structure applicable to TargetOp are selected. The hrd_parameters( ) syntax structure in the active SPS (or provided through an external means not specified in this Specification) is selected. Within the selected hrd_parameters( ) syntax structure, if BitstreamToDecode is a Type I bitstream, the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( vcl_hrd_parameters_present_flag )" is selected and the variable NalHrdModeFlag is set equal to 0; otherwise (BitstreamToDecode is a Type II bitstream), the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows either the condition "if( vcl_hrd_parameters_present_flag )" (in this case the variable NalHrdModeFlag is set equal to 0) or the condition "if( nal_hrd_parameters_present_flag )" (in this case the variable NalHrdModeFlag is set equal to 1) is selected. When BitstreamToDecode is a Type II bitstream and NalHrdModeFlag is equal to 0, all non-VCL NAL units except filler data NAL units, and all  leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex B), when present, are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

- An access unit associated with a buffering period SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) applicable to TargetOp is selected as the HRD initialization point and referred to as access unit 0.

- For each access unit in BitstreamToDecode starting from access unit 0, the buffering period SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the access unit and applies to TargetOp is selected, and the picture timing SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the access unit and applies to TargetOp is selected. The selected buffering period and picture timing SEI messages shall be either SEI messages or provided by external means.

- A value of SchedSelIdx is selected. The selected SchedSelIdx shall be in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where cpb_cnt_minus1[ HighestTid ] is found in the hrd_parameters( ) syntax structure as selected above.

- When the coded buffering period SEI message associated with access unit 0 has cpb_alt_timing_info_present_flag equal to 1, either of the following applies for selection of the initial CPB removal delay and delay offset:

–	If NalHrdModeFlag is equal to 1, the default initial CPB removal delay and delay offset represented by nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. Otherwise, the default initial CPB removal delay and delay offset represented by vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. The variable DefaultInitCpbParamsFlag is set equal to 1.

–	If NalHrdModeFlag is equal to 1, the alternative initial CPB removal delay and delay offset represented by nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message and cpb_alt_initial_removal_delay_delta[ SchedSelIdx ] and cpb_alt_initial_removal_offset_delta[ SchedSelIdx ], respectively, in the picture timing SEI message associated with the access unit following access unit 0 in decoding order are selected. Otherwise, the alternative initial CPB removal delay and delay offset represented by vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message and cpb_alt_initial_removal_delay_delta[ SchedSelIdx ] and cpb_alt_initial_removal_offset_delta[ SchedSelIdx ], respectively, in the picture timing SEI message associated with the access unit following access unit 0 in decoding order are selected. The variable DefaultInitCpbParamsFlag is set equal to 0, and one of the following applies:

–	the RASL access units associated with access unit 0 are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

–	all access units associated following access unit 0 up to an associated DRAP are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

Each conformance test consists of a combination of one option in each of the above steps. When there is more than one option for a step, for any particular conformance test only one option is chosen. All possible combinations of all the steps form the entire set of conformance tests. For each operation point under test, the number of bitstream conformance tests to be performed is equal to n0 * n1 * ( n2 * 2 + n3 + n4 ), where the values of n0, n1, n2, n3 and n4 are specified as follows:

–	n0 is derived as follows:

–	If BitstreamToDecode is a Type I bitstream, n0 is equal to 1.

–	Otherwise (BitstreamToDecode is a Type II bitstream), n0 is equal to 2.

–	n1 is equal to cpb_cnt_minus1[ HighestTid ] + 1.

–	n2 is the number of access units in BitstreamToDecode that each is associated with a buffering period SEI message applicable to TargetOp and for which all of the following conditions are true:

–	nal_unit_type is equal to CRA_NUT.

–	The associated buffering period SEI message has bp_alt_cpb_params_present_flag equal to 1

–	There is at least one RASL picture asscociated to the access unit.

–	n3 is the number of IRAP access units without associated RASL pictures in BitstreamToDecode that each is associated with a buffering period SEI message applicable to TargetOp.

–	n4 is the number of access units in BitstreamToDecode that each is associated with a DRAP SEI message applicable to TargetOp and for each of which the associated picture timing SEI message has cpb_alt_timing_info_present_flag equal to 1.

…

(C.2.2)

The process specified in the remainder of this clause is invoked for derivation of the initial and final CPB arrival times for access unit n.

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are derived as follows:

–	If one or more of the following conditions are true, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ] minus the values of the picture timing SEI message syntax elements cpb_alt_initial_removal_delay_delta and cpb_alt_initial_removal_offset_delta of access unit 1, respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ] minus the values of the picture timing SEI message syntax elements cpb_alt_initial_removal_delay_delta and cpb_alt_initial_removal_offset_delta of access unit 1, respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message containing the syntax elements is selected as specified in clause C.1:

–	UseAltCpbParamsFlag for access unit 0 is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	Otherwise, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message syntax elements are selected as specified in clause C.1.

The variables DpbDelayOffset and CpbDelayOffset are derived as follows with k being the access unit associated with the buffering period SEI message:

–	If one or more of the following conditions are true, DpbDelayOffset is set equal to the value of the picture timing SEI message syntax element dpb_delay_offset of access unit k + 1, and CpbDelayOffset is set equal to the value of the picture timing SEI message syntax element cpb_delay_offset of access unit k + 1, where the picture timing SEI message containing the syntax elements is selected as specified in clause C.1:

–	UseAltCpbParamsFlag for access unit 0 is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	Otherwise, DpbDelayOffset and CpbDelayOffset are set to 0.

The time at which the first bit of access unit m begins to enter the CPB is referred to as the initial arrival time initArrivalTime[ m ].

In C-9 the following should be added:

NominalRemovalTime[ n ] = baseTime + ClockTick * ( tmpCpbRemovalDelay − CpbDelayOffset)

where NominalRemovalTime[ firstPicInPrevBuffPeriod ] is the nominal removal time of the first access unit of the previous buffering period, NominalRemovalTime[ prevNonDiscardablePic ] is the nominal removal time of the preceding picture in decoding order with TemporalId equal to 0 that is not a RASL or RADL picture, CpbRemovalDelayVal is the value of CpbRemovalDelayVal[ OpTid ] derived according to cpb_removal_delay_minus1[ OpTid ] and cpb_removal_delay_delta_idx[ OpTid ] in the picture timing SEI message, and cpb_removal_delay_delta[ cpb_removal_delay_delta_idx[ OpTid ] ] in the buffering period SEI message, selected as specified in clause C.1, associated with access unit n and concatenationFlag and CpbRemovalDelayDeltaMinus1 are the values of the syntax elements concatenation_flag and cpb_removal_delay_delta_minus1, respectively, in the buffering period SEI message, selected as specified in clause C.1, associated with access unit n.

In C-12 the following should be added:

When picture n has PicOutputFlag equal to 1, its DPB output time DpbOutputTime[ n ] is derived as follows, where the variable firstPicInBufferingPeriodFlag is equal to 1 if access unit n is the first access unit of a buffering period and 0 otherwise:

DpbOutputTime[ n ] = CpbRemovalTime[ n ] + ClockTick * ( picDpbOutputDelay – 	(C-12)
				picDpbOutputDelta[ OpTid ] )
	if( firstPicInBufferingPeriodFlag )
		DpbOutputTime[ n ]  −=  ClockTick * DpbDelayOffset

Note: The marks highlited in magenta are a fix to the current draft.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).