| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0507 |
| --- | --- |

| Title: | Non-CE3 : Intra-sub-partition coding mode only performing on non-square blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woowoen Gwun Wonjun Lee Dae Young Lee Tea Hyeon Kim Gwang Hoon Park KHU, 1732, Deogyeong-daero, Giheung-gu, Yongin-si, Gyeonggi-do 17104, KOREA | Tel: Email: | +82-31-201-3680 woo_gwun@khu.ac.kr dldnjs1013@khu.ac.kr daeyung2002@khu.ac.kr taehyun@khu.ac.kr ghpark@khu.ac.kr |
|  | Gun Bang Woong Lim Hui Young Kim ETRI, 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-6791 gbang@etri.re.kr |
| Source: | KHU (Kyung Hee University), ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

Intra sub-partition (ISP) mode can be performed on both NxN (square) or NxM (non-square) luma blocks. As a result of some experiments, we were able to find that the coding gain differences between performing ISP mode for both square and non-square blocks and performing ISP mode only on non-square blocks are insignificant. Thus, this contribution proposes method which allow ISP mode only on non-square blocks. This restriction can far reduce the computational complexity for ISP coding mode with minimum coding loss. Encoding and decoding times are reduced up to 2% and 8% in AI configuration, and 4% and 0% in RA configuration, respectively, while maintaining almost same coding efficiencies.

Problem Definition

In WD5 [2], the current ISP mode is restricted when block’s width is bigger than MaxTbSizeX, or heght is bigger than MaxTbSizeY, or block’s size is 4x4. So, ISP mode is performed on various form of square blocks. In CE3-1 [3][4], the various tests were performed in ISP, but its’ complexity issue was not handled thoroughly. The various sizes of blocks are tested in terms of gain and complexity for ISP mode, and our tests can be observed that ISP mode only on a non-square block results in negligible gain drop compared to current ISP mode while the computational complexity reduction makes more sense by proposed solution.

Proposed Solutions

In this contribution, the restriction of ISP mode to be used on only non-square block is proposed. The proposed method checks whether or not current block is square block to determine the use of ISP mode along side with other existing ISP restrictions. As shown in Figure 1, the proposed method is suggested that the determination part is just added whether the ISP is possible based on the block type to the existing ISP coding mode. If the current block is non-square type, ISP mode is performed with ISP split flag and MPM signaling. The non-square type can be determined by the comparison of width and height of the block. As a results, the computational complexity can be reduced because the proposed ISP mode is limited in only non-square blocks.

Figure 1: Flow diagram of the proposed ISP coding mode

For the purpose of comparison, the existing rule of splitting for different block sizes in ISP is listed in Table 1, while the proposed splitting rule is listed in Table 2. The difference from existing ISP coding tool is highlighted with yellow marking in Table 2.

Table 1: Existing rule of splitting in ISP

| Block Size | Number of Sub-Partitions | Partition direction |
| --- | --- | --- |
| 4×4 | Not divided |  |
| 4×8 | 2 | Allow both horizontal and vertical |
| 4×N (N>8) | 4 | Allow both horizontal and vertical |
| 8×4 | 2 | Allow both horizontal and vertical |
| 8×N (N>4) | 4 | Allow both horizontal and vertical |
| All other cases allowed in ISP | 4 | Allow both horizontal and vertical |

Table 2: Proposed rule of splitting in ISP

| Block Size | Number of Sub-Partitions | Partition direction |
| --- | --- | --- |
| N×N | Not divided |  |
| 4×8 | 2 | Allow both horizontal and vertical |
| 4×N (N>8) | 4 | Allow both horizontal and vertical |
| 8×4 | 2 | Allow both horizontal and vertical |
| 8×N (N>4) | 4 | Allow both horizontal and vertical |
| All other cases allowed in ISP | 4 | Allow both horizontal and vertical |

Experimental Results

The simulations are performed according to the common test conditions (CTC) as described in [1]. AI and RA results are reported.

Simulation result for proposal (to-be-completed)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | 0.03% | 98% | 97% |
| Class A2 | 0.03% | 0.04% | -0.02% | 99% | 97% |
| Class B | 0.05% | -0.01% | 0.06% | 98% | 93% |
| Class C | 0.04% | -0.07% | 0.01% | 97% | 86% |
| Class E | 0.05% | 0.04% | -0.04% | 99% | 88% |
| Overall | 0.03% | -0.02% | 0.01% | 98% | 92% |
| Class D | 0.06% | 0.20% | -0.01% | 91% | 94% |
| Class F | 0.04% | 0.12% | 0.17% | 98% | 90% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.16% | 0.17% | 91% | 99% |
| Class A2 | 0.02% | 0.07% | 0.02% | 91% | 102% |
| Class B | 0.04% | 0.00% | -0.15% | 99% | 100% |
| Class C | -0.02% | -0.19% | -0.17% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.07% | -0.06% | 96% | 100% |
| Class D | 0.06% | 0.24% | -0.07% | 100% | 99% |
| Class F | 0.03% | -0.07% | 0.03% | 98% | 100% |

Conclusion

In this contribution, intra-sub-partition coding mode only performed on non-square block is proposed. Both in AI and RA configurations, the encoding and decoding times are reduced to 2% and 8% and reduced to 4% and 0%, respectively, in comparison with the results of VTM5.0, while maintaining almost same coding efficiencies.

Patent rights declaration(s)

KHU and ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Draft Specification Changes on top of [2]

Changes of Proposal

7.3.7.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| … |  |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| … |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| … |  |
| if( intra_bdpcm_flag[ x0 ][ y0 ] ) |  |
| intra_bdpcm_dir_flag[ x0 ][ y0 ] | ae(v) |
| else { |  |
| … |  |
| if( intra_mip_flag[ x0 ][ y0 ] ) { |  |
| … |  |
| } else { |  |
| … |  |
| if( cbWidth != cbHeight ){ |  |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY )) { |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

References

- “JVET common test conditions and software reference configurations”, JVET-N1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v7, B. Bross, J. Chen, and S. Liu

- “CE3: Summary Report on Intra Prediction and Mode Coding”, JVET-N0023, G. Van der Auwera, L. Li, A. Filippov

- “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding”, JVET-N1023, G. Van der Auwera, L. Li, A. Filippov