| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0509 |
| --- | --- |

| Title: | Non-CE3 : MPM flag signaling first |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dae Young Lee Woowoen Gwun Wonjun Lee Tea Hyeon Kim Gwang Hoon Park KHU, 1732, Deogyeong-daero, Giheung-gu, Yongin-si, Gyeonggi-do 17104, KOREA | Tel: Email: | +82-31-201-3680 daeyung2002@khu.ac.kr woo_gwun@khu.ac.kr dldnjs1013@khu.ac.kr taehyun@khu.ac.kr ghpark@khu.ac.kr |
|  | Jinho Lee Jungwon Kang ETRI, 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr jungwon@etri.re.kr |
| Source: | KHU (Kyung Hee University) and ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This paper proposes two signaling modification methods which related to MPM flag signaling, MRL index signaling, ISP mode flag signaling, and Not-planar flag. The first method is to clean up intra mode coding structure by sending MPM flag signaling before MRL index signaling and ISP mode flag signaling. The second method is to configure Not-planar flag signaling in the first method before MRL index signaling and ISP mode flag signaling. In the first method, when the MPM flag is 0, the MRL index and the ISP mode flag are informed by the decoder without signaling. In the second method, in addition to the first method, when the MPM flag is 1, and if the Not-planar flag is 0, the MRL index is informed by the decoder without signaling. It is reported that the average luma BD-rates for AI and RA cases are 0.01% and 0.00% in method1, 0.00% and 0.01% in method2, in comparison to VTM 5.0, respectively.

Introduction

In VVC, as shown in Figure 1, the information of mode, index, or flag is sent first, and then MPM flag is signaled next in the process of intra prediction mode signaling. There is a skip condition for the MPM flag according to the prediction related signaling value because the prediction related signaling is performed first and the MPM flag is signaled next. Therefore, MPM flag has the structure as shown in Figure 1 and MPM flag signaling condition check reconfiguration is needed when future prediction related tools are added. Also, if signaling omission condition exists between each prediction related signaling, duplicated condition check can occur as shown in Figure 1. This is not a clean design in terms of system scalability and efficiency.

Figure 1. Example of signaling relationship between different modes and MPM flags.

Problem Statement

Intra subblock partitioning (ISP) and multiple reference line (MRL) were adopted to VVC at the last meeting. In VVC, the ISP always uses MPM mode. Also, the MPM is always used when the MRL index is larger than 0. So for the each case the MPM flag is always set to 1 and the signaling is skipped. Therefore, there are dependency between MPM flag, ISP mode flag and MRL index in VVC. Since there is a dependency between ISP mode flag and MRL index in VVC, dependency also exists between MPM flag, ISP mode flag and MRL index as shown in Figure 1, and also it results MPM flag to be signaled later than ISP mode flag and MRL index. The existing structure where most used MPM mode is signaled behind other mode and index gives sense of untidiness. Also, when various other modes using MPM are added, the structure in which the MPM flag is signaled last is not good in terms of system scalability.

Figure 2 shows part of the signaling structure in VVC, and Table 1 is part of the syntax table matching Figure 2. As shown in Figure 2 and Table 1, VVC performs MRL index signaling and ISP mode flag before MPM flag in intra prediction. Since the ISP mode flag is 1 only when the MRL index is 0, a condition check is performed to determine whether the MRL index is 0 before ISP mode flag signaling. Since the MPM flag is always 1 when the ISP mode flag is 1 or the MRL index is larger than 0, the MPM flag signaling is performed only when the ISP mode flag is 0 and the MRL index is 0. In addition, when the MRL index is larger than 0, the Not-planar flag is 1 because Planar mode is not used. Therefore, Not-planar flag signaling is performed only when the MRL index is 0. As a result, the condition for checking MRL index value is duplicated in the signaling structure of the VVC from the MRL index signaling to the intra prediction mode restoration process, which is an inefficient method. This is shown in Figure 2 and Table 1.

Figure 2. Part of the VVC signaling structure.

Table 1. Part of the VVC syntax structure

| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |

Proposal

Proposed methods change some of the ordering and structure of MPM flag, MRL index, ISP mode flag, and Not-planar flag signaling, as described as below;

- Method 1: MPM flag signaling is performed prior to MRL index signaling and ISP mode flag signaling

- Method 2: Method 1 + Not-planar flag signaling is performed prior to MRL index signaling and ISP mode flag signaling

The signaling flow for Method 1 is shown in Figure 3 and its syntax is shown in Table 2. Changes in the syntax of Table 2 were highlighted in yellow, and the deletion was crossed out.

In Method 1, if the MPM flag is 0, the MRL index and ISP mode flag signaling are skipped, the MRL index is set to 0, and the ISP mode flag is set to 0.

The signaling flow and syntax for Method 2 are shown in Figure 4 and Table 3.

Addition to the Method 1, if the MPM flag is 1, Not-planar flag is signaled prior to the MRL index, and if the Not-planar flag is 0, a condition for skipping the MRL index signaling and MRL index is set to 0.

In Method 1 and Method 2, redundancy of the condition check for MRL index is reduced than existing VVC signaling structure.

Figure 3. Flow chart of the Method 1.

Figure 4. Flow chart of the Method 2.

Table 2. Part of the syntax structure of Method 1.

| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { | Descriptor |
| --- | --- |
| … |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ){ |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |

Table 3. Part of the syntax structure of Method 2.

| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { | Descriptor |
| --- | --- |
| … |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ){ |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) { |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |

Experimental results

Experiments were performed based on VTM 5.0 software [1] under JVET common test condition [2].

Tables 4 and 5 show the experimental results for Method 1, and Method 2, respectively.

Table 4. Results of the Method 1 over VTM 5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | -0.07% | -0.03% | 99% | 100% |
| Class B | 0.01% | -0.06% | 0.00% | 99% | 100% |
| Class C | 0.01% | -0.09% | 0.00% | 100% | 100% |
| Class E | 0.01% | 0.06% | 0.01% | 100% | 100% |
| Overall | 0.01% | -0.04% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.24% | 0.01% | 100% | 100% |
| Class F | 0.02% | 0.04% | 0.06% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.15% | 0.06% | 100% | 100% |
| Class A2 | 0.01% | 0.00% | 0.07% | 100% | 100% |
| Class B | 0.00% | -0.14% | -0.06% | 100% | 100% |
| Class C | 0.00% | -0.20% | -0.14% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.13% | -0.03% | 100% | 100% |
| Class D | 0.06% | -0.02% | 0.14% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.07% | 100% | 100% |

Table 5. Results of the Method 2 over VTM 5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.01% | 99% | 101% |
| Class A2 | 0.01% | -0.03% | -0.04% | 100% | 101% |
| Class B | 0.00% | -0.01% | 0.05% | 100% | 100% |
| Class C | -0.01% | -0.01% | 0.02% | 99% | 99% |
| Class E | 0.00% | -0.11% | -0.01% | 99% | 100% |
| Overall | 0.00% | -0.04% | 0.01% | 99% | 100% |
| Class D | 0.02% | 0.21% | -0.08% | 100% | 101% |
| Class F | 0.01% | -0.08% | -0.07% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.13% | -0.02% | 99% | 100% |
| Class A2 | 0.01% | 0.08% | 0.03% | 100% | 99% |
| Class B | 0.00% | -0.10% | -0.19% | 99% | 100% |
| Class C | 0.00% | 0.02% | -0.22% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.04% | -0.12% | 99% | 100% |
| Class D | 0.02% | 0.14% | 0.11% | 100% | 99% |
| Class F | 0.01% | -0.05% | 0.14% | 100% | 100% |

Reference

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

Patent rights declaration(s)

Kyung Hee university and ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).