| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0510 |
| --- | --- |

| Title: | CE3-related: Extensions to CCLM reference sample filtering in CE3-3.2.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Keating Karl Sharman | Email: | steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

CE3-3.2.1 tests filtering of chroma reference samples used in generating the CCLM parameters alpha and beta.  This, in itself, has been found to reduce coding efficiency.  By restricting the use of this filtering and by also performing similar filtering on the luma reference samples any loss in coding efficiency is eliminated (-0.01% Y, -0.03% U, +0.01% V).

# Introduction

The premise behind CE3-3.2.1 is that low-pass filtering the reference samples should reduce outliers and make CCLM less sensitive to noise.  Only the chroma samples are filtered as it is assumed that the 6-tap filter and down-sampling that is applied to the luma samples already has a low-pass effect.  This is not the case as the 6-tap filter does not reduce the bandwidth much below a half and so once sub-sampled there is very little softening effect.  Also, when reference samples are spatially close any low-pass filtering would tend to merge the values together and reduce the distinctiveness of the selected samples which could be detrimental in the calculation of alpha and beta.

# Proposed Implementation

It is proposed to match the low-pass filtering of the chroma samples by also filtering the luma samples, this could either be done by extending the existing 6-tap luma filter or by adding a low-pass filter after the luma has been sub-sampled, this added filter being identical to the chroma low-pass filter.

Filtering is also disabled when reference samples are within 2 samples apart in order to avoid merging of the samples values.

# Results – All Intra

The results of the proposed implementation are shown below:

(note that run-times have not been included as they were not reliable)

# Conclusion

An extension to CE3-3.2.1 has been proposed which eliminates any loss in coding efficiency whilst still allowing some filtering of reference samples.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, B. Bross, J. Chen, and S. Liu