| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0512 |
| --- | --- |

| Title: | Non-CE3: CCLM Coding bug fix |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Keating Karl Sharman | Email: | steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution describes a modified coding method for the intra_chroma_pred_mode.  The current coding method shares a context between the MDLM_on flag and the MDLM_direction flag. It is proposed that the MDLM_direction flag should instead be coded as an EP bin. The implementation tested gives 0.00% change in luma BD-rate and a BD-rate gain of 0.08% in each chroma component over VTM5.0 in AI.

# Introduction

LM Chroma can use samples from above and to the left of the block (MDLM=off) or purely samples from above (MDLM=T) or purely samples from the left (MDLM=L).  Two symbols are currently used to code this:  MDLM_on and MDLM_direction. These two symbols are currently unary coded with a single context whereas the probability of MDLM being on is not related to the direction of MDLM chosen.

# Proposed Implementation

It is proposed to code the MDLM_direction flag independently with an EP bin as this is the simplest alternative to the current method.  Experimental results show that coding the MDLM_direction with its own CABAC context gives no improvement in coding performance.

# Results – All Intra

The results of the proposed implementation are shown below:

The results for coding the flag with CABAC are shown below:

# Conclusion

An alternative coding method for the MDLM_direction flag is proposed which avoids the use of a shared context with the MDLM_on flag.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Specification

The changes are relative to JVET-N1001-v8 [2]

9.5.4.2.1	General

Input to this process is the position of the current bin within the bin string, binIdx.

Outputs of this process are ctxTable, ctxIdx and bypassFlag.

The values of ctxTable, ctxIdx and bypassFlag are derived as follows based on the entries for binIdx of the corresponding syntax element in Table 9-17:

- If the entry in Table 9-17 is not equal to "bypass", "terminate" or "na", the values of binIdx are decoded by invoking the DecodeDecision process as specified in clause 9.5.4.3.2 and the following applies:

- ctxTable is specified in Table 9-5

- The variable ctxInc is specified by the corresponding entry in Table 9-17 and when more than one value is listed in Table 9-17 for a binIdx, the assignment process for ctxInc for that binIdx is further specified in the clauses given in parenthesis.

- The variable ctxIdxOffset is specified in Table 9-5 depending on the current value of initType.

- ctxIdx is set equal to the sum of ctxInc and ctxIdxOffset.

- bypassFlag is set equal to 0.

- Otherwise, if the entry in Table 9-17 is equal to "bypass", the values of binIdx are decoded by invoking the DecodeBypass process as specified in clause 9.5.4.3.4 and the following applies:

- ctxTable is set equal to 0.

- ctxIdx is set equal to 0.

- bypassFlag is set equal to 1.a

- Otherwise, if the entry in Table 9-17 is equal to "terminate", the values of binIdx are decoded by invoking the DecodeTerminate process as specified in clause 9.5.4.3.5 and the following applies:

- ctxTable is set equal to 0.

- ctxIdx is set equal to 0.

- bypassFlag is set equal to 0.

- Otherwise (the entry in Table 9-17 is equal to "na"), the values of binIdx do not occur for the corresponding syntax element.

| Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … |  |  |  |  |  |  |
| … |  |  |  |  |  |  |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 | bypass | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 | 0 | 1 | 2 | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 | 0 | 1 | 2 | 2bypass | na | na |
| ... |  |  |  |  |  |  |

# References

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, B. Bross, J. Chen, and S. Liu