| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0513 |
| --- | --- |

| Title: | Non-CE4: Adaptive blending filtering for TPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Han Gao, Anand Meher Kotra, Biao Wang, Zhijie Zhao, Elena Alshina (Huawei), Max Bläser, Johannes Sauer (RWTH Aachen) | Tel: Email: | semih.esenlik@huawei.com elena.alshina@huawei.com blaeser@ient.rwth-aachen.de |
| Source: | Huawei Technologies Co., LTD, RWTH Aachen University |  |  |

_____________________________

# Abstract

It is asserted that the blending filtering of TPM causes over-smoothing when the content is known to include sharp edges, hence rendering TPM less useful for screen content application. It is proposed to include a control flag in tile group header or picture parameter set to enabling turning blending filtering of TPM off. The goal of the proposed modification is to retain sharp details that are common especially in SCC material. For TGM class, the simulation results demonstrate 0.59% and %X luma BD-rate reduction for RA and LDB configurations respectively.

# Introduction

Triangle Partition Mode (TPM) coding was adopted to VVC draft at JVET-L meeting in Macao [1]. According to AhG-13 test results in VTM5.0 triangle inter block coding provides 0.35% BD-rate gain in random access and 0.88% in low-delay B coding scenario.

According to the TPM, 2 uni-prediction results are combined via a blending filter to obtain the final prediction of a coding block. The blending filter that is utilized has a size of 7, i.e. 7 samples along the diagonal of a square block is obtained by weighted combination of two predictions.

# Proposal

Proposed solution further extends flexibility of Triangle Partition Mode by introducing a control flag to disable the blending filtering operation. When the blending filtering is disabled, the sample weight for weighted combination can either be 0 or 8, the implementation in the draft specification is as follows:

- if slice_triangle_blending_enabled_flag is equal to 1, the variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows: wValue = ( cIdx = = 0 ) ? Clip3( 0, 8, wIdx ) : Clip3( 0, 8, wIdx * 2 ) (8-826) - otherwise (slice_triangle_blending_enabled_flag is equal to 0), the variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows: wValue = ( cIdx = = 0 ) ? (wIdx < 4 ? 0 : 8) : (wIdx * 2 < 4 ? 0 : 8) (8-7)

It is asserted that the contribution provides flexibility in the case of SCC, when the over-smoothing of the sharp edges cannot be avoided with the current TPM blending filter.

The slice_triangle_blending_enabled_flag is suggested to be signalled in tile or tile group header in order to support mixed content, where parts of the picture is natural content whereas other part is screen content.

## Experimental results

Test results vs VTM-5.0 anchor were obtained for RA and LDB test conditions (LDB results to be updated) are presented in Table 1.

Table 1: Experimental results for switching the blending filter of TPM off, RA configuration.

# Conclusion

It is proposed to include a control flag to control the blending operation of the TPM. The goal of the proposed modification is to retain sharp details that are common especially in SCC material. For TGM class, the simulation results demonstrate 0.59% and %X luma BD-rate reduction for RA and LDB configurations respectively. It is proposed to adopt proposed modification into next VVC draft specification.

# References

- R.-L. Liao, C. S. Lim, “CE10.3.1.b: Triangular prediction unit mode”, JVET-L0124, Macao, CN, 3–12 October 2018

- B. Bross, J. Chen, S. Liu “Versatile Video Coding (Draft 5)”, JVET-N1001, Geneva, CH, 19–27 March 2019

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.