| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0514 |
| --- | --- |

| Title: | Non-CE3: Analysis of Matrix-based Intra Prediction (Informative) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | For Information |  |  |
| Author(s) or Contact(s): | Steve Keating Karl Sharman | Email: | steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution contains some analysis of Matrix-based Intra Prediction (MIP) and how it differs from conventional angular prediction methods.

# Introduction

Matrix-based Intra Prediction (MIP) was adopted at the 14th JVET meeting.  It offers a different approach to luma prediction from conventional angular prediction and provides some coding gain albeit at the cost of extra complexity.  This contribution aims to show how this prediction method differs from angular prediction and why it improves coding efficiency.

MIP can be split into two parts for analysis purposes: the prediction method and the coding method.

# Prediction Method

Instead of projecting a row/column of reference samples to make the predicted block, the row/column of reference samples is multiplied by a 2-D matrix to generate a 2-D block, which then has another 2-D matrix added to it to make the final predicted block.  This allows a prediction which varies in more than one direction, which is not possible with angular prediction (although planar prediction does achieve this).

Another feature of the prediction method is that, in order to limit the size of the matrices, the reference samples for blocks larger than 4x4, are filtered and down-sampled and the output data up-sampled to provide the predicted block; blocks larger than 16x16 are not processed.  Either 2 or 4 reference samples per side (left, above) are used depending on the size of the block.  Also, only directly neighbouring samples are used as reference samples – left and above, whereas angular prediction may also use samples up to a block height/ width further away – below-left and above-right.

## Comparison with Angular Prediction

In order to visualise the predictions obtained with MIP the matrices were displayed as a series of 4x4 or 8x8 grey-scale images, examples of which are shown below for the largest matrices (MipSizeId=2).  The sequence of images is arranged to correspond with the relevant reference samples (in this case 4 above and 4 left):

MIP mode 0						MIP mode 1

MIP mode 2						MIP mode 3

MIP mode 4						MIP mode 5

It can be seen that these predictions are similar to those of angular prediction although generally softer and sometimes with a stronger PDPC-like effect.  On noticing this, an experiment was done which replaced each MIP prediction with an equivalent angular prediction but with stronger filtering and/or PDPC as appropriate. The stronger filtering is to mimic the softer predictions and down/up sampling feature of MIP; the PDPC is to mimic the impact of reference samples on the opposite edges of the prediction.  This was done in an attempt to remove the complexity of the down/up-sampling and matrix multiplication, by instead re-purposing the other existing tools.  The experiment, although showing some improvement over VTM without MIP, failed to achieve the same coding gain as MIP, indicating that these similar-looking angular predictions were not sufficiently close to the MIP predictions or at least not as useful.

# Coding Method

Another notable feature of MIP is that its predictions are coded separately from the conventional prediction modes using a second Most Probable Mode (MPM) list which is limited to 3 modes compared to the 6 mode MPM list used in conventional prediction.  It is known that the MPM list can significantly affect coding gain so this might be responsible for some of the coding gain achieved by MIP.  In order to test the effect of the coding another experiment was done which used the non-MIP coding (with 6 MPMs) for both non-MIP and MIP modes.  Each MIP mode number was mapped to an equivalent mode number, this mapping being based on Table 8-1 in [2].  The MIP flag was coded only for modes which had such a mapping (as there are fewer MIP modes than conventional modes).  The experiment gave a reduction in coding gain, dropping from 0.36% to 0.25% (in VTM5.0), this may be due in part to the mapping used but does indicate that the coding method may indeed be responsible for some of the coding gain achieved by MIP.  Other proponents have simplified the coding method by removing the second MPM list with negligible loss [3], indicating that the coding method does not significantly affect coding gain.

# Resources

MIP currently requires the following storage for the matrices:

| Size | MipSizeId | Number of distinct matrices | Size of multiply matrix (in coefficients) M | Size of additive matrix (in coefficients) A | Total number of coefficients M+A |
| --- | --- | --- | --- | --- | --- |
| 4×4 | 0 | 18 | 18×42×4 | 18×16 | 1440 (23%) |
| 8×N, N×8 (N≤8) | 1 | 10 | 10×42×8 | 10×16 | 1440 (23%) |
| 16×N, N×16 (N≤16) | 2 | 6 | 6×82×8 | 6×82 | 3456 (55%) |
| All |  |  |  |  | 6336 |

In addition, there is some storage to apply a variable shift following each matrix multiply / matrix add stage. However, the values have little variance, and are in comparison, negligible.

There is also some storage for the maps used to convert between MIP modes and conventional modes for the MPM lists:

3x35 + 3x35 = 210 entries

In order to assess the contribution from each of the different size matrices some experiments were done to disable MIP for various blocks sizes.  Note that these were simple encoder-only changes so the MIP flag was still encoded in the bit-stream even if MIP could not be selected for that block size.  The results are shown below compared to the VTM5.0 anchor and to VTM5.0 with MIP disabled in the configuration:

Disabling MIP (encoder-only) for MipSizeId=2

Disabling MIP (encoder-only) for MipSizeId<2

From these results it can be seen that the MipSizeId=2 case (>8x8 blocks) seems to be responsible for much of the overall gain.  Another experiment tested the contribution for MipSizeId=0 (4x4 blocks):

Disabling MIP (encoder-only) for MipSizeId=0

It can be seen from this that the 4x4 blocks contribute very little to the overall gain.

# Conclusion

A brief analysis of Matrix-based Intra Prediction has been done including comparison with conventional angular prediction.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, J. Pfaff, B. Stallenberger, M. Schäfer, P. Merkle, P. Helle, T. Hinz, H. Schwarz, D. Marpe, T. Wiegand

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, B. Bross, J. Chen, and S. Liu

- “CE3 Related: No MPM Derivation for Matrix Based Intra Prediction (MIP)”, JVET-O0169, Salehifar, M., Seung-Hwan Kim