| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0517 |
| --- | --- |

| Title: | Slice/tile level CABAC zero-word constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This document describes a modified coding method for padding with cabac_zero_word.  The current coding method adds padding at the end of a slice based upon picture-level information.  It is proposed that the padding is moved to the end of each slice/tile (or the appropriate independently decodable region) and base the calculation on the sub-region information.  Implementation of this modification would allow any picture created by composition of different slices/tiles/sub-regions from different pictures to meet the specification, removing the need for counting bins in the resulting bit-stream.

# Introduction

Section 7.4.3.11 of the specification [1] states the following:

Let NumBytesInVclNalUnits be the sum of the values of NumBytesInNalUnit for all VCL NAL units of a coded picture.

Let BinCountsInNalUnits be the number of times that the parsing process function DecodeBin( ), specified in clause TBD, is invoked to decode the contents of all VCL NAL units of a coded picture.

Let the variable RawMinCuBits be derived as follows:

RawMinCuBits = MinCbSizeY * MinCbSizeY *
				( BitDepthY + 2 * BitDepthC / ( SubWidthC * SubHeightC ) )	(7-39)

The value of BinCountsInNalUnits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  32.

NOTE – The constraint on the maximum number of bins resulting from decoding the contents of the coded slice NAL units can be met by inserting a number of cabac_zero_word syntax elements to increase the value of NumBytesInVclNalUnits. Each cabac_zero_word is represented in a NAL unit by the three-byte sequence 0x000003 (as a result of the constraints on NAL unit contents that result in requiring inclusion of an emulation_prevention_three_byte for each cabac_zero_word).

In section 7.3.2.11 of the specification it states that cabac_zero_word may be added at the end of each slice to maintain a ratio of bits to bins, such that a decoder has a limit with which to work.  The limits are based upon picture level constraints.

# Proposed Implementation

It is proposed that the calculation of the number of cabac_zero_word is updated based upon sub-picture level constraints.  The level of signalling may be at slice or tile level, or perhaps at a finer granularity: it ideally would be placed at the finest granularity that a picture can be independently decoded.  Such a change will ensure that if independent regions from different coded sequences are composited together, the constraints will continue to apply to the resulting bit stream: the compositor would not need to parse the bitstream to calculate the number of coded bins.

An example application is where a sequence consists of multiple VR tiles, where only a subset should be sent to a conformant decoder.  In the current design, if selecting only some tiles, it is not known whether cabac zero words are required for a particular combination of tiles.  In the proposed design, the tiles themselves would be consistent.

The constraint / requirement of cabac_zero_word is identical, only it is based upon a tile-level calculation, and therefore the area of the tile (rather than the area of the picture).

# Results

The proposed implementation was tested in three configurations so that the impact could be objectively assessed:

- Quad tile: each picture is split into 2x2 tiles

- Oct tile: each picture is split into 4x2 tiles

- Per CTU: in which each CTU within a picture is a tile.

Anchors for each configuration were created using VTM 5.0.  HashME was disabled for class F as this was not decodable in multiple tile configurations in VTM 5.0.

## Quad Tile

The quad tile anchor gave a loss of +0.77% (AI) / 1.12% (RA) against CTC.

The results for the proposed implementation when compared with this anchor are given below:

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0_TileQuad_Anchor-Benchmark-VTM-JVET-M1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.08% | 0.05% | 0.05% | 102% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 102% |
|  | Random Access |  |  |  |  |
|  | Over VTM5.0_TileQuad_Anchor-Benchmark-VTM-JVET-M1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class A2 | 0.02% | 0.02% | 0.02% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

## Oct Tile

The oct tile anchor gave a loss of +1.29% (AI) / 1.91% (RA) against CTC.

The results for the proposed implementation when compared with this anchor are given below:

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0_TileOct_Anchor-Benchmark-VTM-JVET-M1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class A2 | 0.09% | 0.06% | 0.06% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.02% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random Access |  |  |  |  |
|  | Over VTM5.0_TileOct_Anchor-Benchmark-VTM-JVET-M1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class A2 | 0.02% | 0.03% | 0.03% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

## Tile Per CTU

The tile per CTU anchor gave a loss of +7.92% (AI) / 15.89% (RA) against CTC.

The results for the proposed implementation when compared with this anchor are given below:

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0_TileCTU_Anchor-Benchmark-VTM-JVET-M1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | 0.01% | 0.01% | 0.01% | 103% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 104% | 105% |
| Class E | 0.00% | 0.00% | 0.00% | 103% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 101% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0_TileCTU_Anchor-Benchmark-VTM-JVET-M1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.02% | 0.02% | 100% | 100% |
| Class A2 | 0.01% | 0.01% | 0.01% | 101% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.01% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 101% |

# Conclusion

A modified padding cabac_zero_word padding method is proposed which changes the calculation to be based upon finer picture granularity information, rather than the picture itself.  The result would allow compositors to mix independently decodable regions into single streams while maintaining the bin/bit level constraints offered by the cabac_zero_word mechanism, but without needing to parse the bit-stream.  This results in very small overall losses whilst offering increased flexibility.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, B. Bross, J. Chen, and S. Liu