| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET- O0521 |
| --- | --- |

| Title: | Non-CE6: Removing LFNST when MTS larger than zero |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gosala Kulupana Saverio Blasi Andre Seixas Dias 201 Wood Lane, The Lighthouse W12 7TQ, London, UK | Tel: Email: | gosala.kulupana@bbc.co.uk saverio.blasi@bbc.co.uk andre.seixasdias@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution presents a modification to the Low Frequency Non-Separable Transform (LFNST) usage in VVC. In the current VVC draft specification, LFNST is used regardless of the primary transform type. This contribution proposes to remove the usage of LFNST when the MTS index is different than 0 (transform different than DCT2-DCT2). This allows shorter encoding time as well as removal of one context model in the current VVC draft specification. Consequently the average BD-rates of 0.07% (Y), 0.06% (U) and 0.10% (V) are reported for AI with 93% average encoding time and 99% decoding time with respect to VTM 5.0.

# Introduction

In the current VVC draft specification, LFNST may be used on intra-coded primary transform coefficients to further compact the energy of the residual signal. An index is signalled in the bitstream to identify whetner LFNST was used in the current block or not. LFNST is enabled for all MTS options apart from MTS 1 (corresponding to Transform skip mode).

The benefits of using LFNST in blocks that use MTS different than 0 are very limited. In this contribution we propose to remove the usage of LFNST in blocks that use MTS different than 0. This allows for faster encoding time as the encoder does not need search for the optimal LFNST flag in case MTS is different than 0. Additionally, the context model used in the current VVC draft specification to encode the lfsnt_idx depending on the MTS index can also be removed.

Table 1: Changes in the VVC draft specification

| numSigCoeff = 0 |  |
| --- | --- |
| numZeroOutSigCoeff = 0 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] && tu_mts_idx[ x0 ][ y0 ] == 0 ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 ) |  |
| if( treeType = = DUAL_TREE ¦¦ tu_cbf_luma[ x0 ][ y0 ]) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Performance results

The approach was tested on top of VTM5.0 [1] using the Common Test Conditions [2]. The software modifications were made on top of VTM 5.0, compiled with gcc 4.8.2. All experiments were run using AVX, with parallel encoding/decoding used for RA.

Table 1: Performance of the proposed approach

|  |  |  |  |  |  | AI |  |  |  |  |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  | EncT |  | DecT |  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.06% | 0.15% |  |  | 0.20% |  | 92% |  | 100% |  | xx | xx |  | xx |  | 69% |  | #NUM! |  |
| Class A2 | 0.03% | 0.11% |  |  | 0.05% |  | 93% |  | 101% |  | xx | xx |  | xx |  | 27% |  | #NUM! |  |
| Class B | 0.07% | 0.08% |  |  | 0.16% |  | 92% |  | 98% |  | -0.01% | -0.01% |  | -0.17% |  | 100% |  | 99% |  |
| Class C | 0.08% | -0.04% |  |  | 0.09% |  | 93% |  | 97% |  | -0.02% | -0.09% |  | -0.06% |  | 100% |  | 98% |  |
| Class E | 0.08% | 0.04% |  |  | -0.01% |  | 94% |  | 99% |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.07% | 0.06% |  |  | 0.10% |  | 93% |  | 99% |  | xx | xx |  | xx |  | 72% |  | #NUM! |  |
| Class D | 0.08% | 0.20% |  |  | -0.10% |  | 93% |  | 98% |  | 0.07% | 0.05% |  | 0.47% |  | 100% |  | 97% |  |
| Class F | 0.04% | 0.07% |  |  | 0.12% |  | 94% |  | 99% |  | 0.06% | -0.07% |  | 0.04% |  | 101% |  | 103% |  |

# Conclusions

In this contribution, a restriction to the usage of LFNST is proposed in cases where a block uses MTS index different than 0. The performance of the proposed method when implemented in VTM 5.0 demonstrates an average BD-rates of 0.07% (Y), 0.06% (U) and 0.10% (V) are reported for AI with 93% average encoding time and 9x% decoding time with respect to VTM 5.0.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” JVET-N1001-v5, June 2019

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” in JVET-M1010, January 2019

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).