| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0523 |
| --- | --- |

| Title: | CE3-related: simplification of Matrix Intra Prediction (MIP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thierry Dumas, Franck Galpin, Gagan Rath, Fabien Racape | Tel: Email: | +33.2.56.85.62.76 thierry.dumas@InterDigital.com franck.galpin@InterDigital.com gagan.rath@InterDigital.com fabrice.leleannec@InterDigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This contribution reports the result of using a single mode, called Simplified Matrix Intra Prediction (SMIP) mode. This mode consists in keeping only 1 matrix per block-size. It decreases the memory footprint from

~8 kilobytes to 1 kilobyte. It also simplifies the decoding process by removing the need of an extra MPM list derivation as well as a mapping table between the conventional modes and the MIP modes. A loss of 0.xx% in AI, and a loss of 0.xx% in RA is reported.

# Introduction

In the JVET meeting held in Geneva in March 2019, it was agreed to use Matrix Intra Prediction (MIP) [1] as additional intra prediction tool.

In this contribution, we propose to use only 1 mode instead of the 35 MIP modes in the case of 4x4 blocks to be predicted, 19 MIP modes in the case of 8x4, 4x8, and 8x8 blocks to be predicted, and 11 MIP modes in the other cases.

# Proposed solution

In this contribution, all the MIP modes are replaced by a single mode, called Simplified Matrix Intra Prediction (SMIP) mode. Section 2 describes the number of parameters in the matrices of weights and the vector of biases of SMIP. Sections 3 and 4 explains the process for signaling SMIP.

For predicting the current block from decoded reference samples via the SMIP mode, the averaging of the boundary, the affine transformation, and the linear interpolation are identical to those proposed in [1]. However, the signalization of SMIP is simpler than that of MIP as, in the case of SMIP, there is no need to create a list of MPMs and the mapping between each MIP mode and one of the 67 regular intra prediction modes in H.266/VVC is replaced by a mapping between SMIP and planar.

For MIP in VTM-5.0, the memory footprint of the weights and biases amounts to 6336 coefficients to be coded, which corresponds to 7.92 kilobytes when stored as 10-bit values, whereas, for SMIP, the memory footprint of the weights and biases is 800 coefficients to be coded, which corresponds to 1 kilobytes when stored as 10-bit values. For MIP in VTM-5.0, the memory footprint of the mapping between each MIP mode and one of the 67 regular intra prediction modes in H.266/VVC amounts to 170 coefficients to be coded, i.e. 0.17 kilobytes when stored as 8-bit values, which disappears for SMIP. Compared to MIP in VTM-5.0, 0.xx% coding loss in AI, with xx% encoding and xx% decoding time is observed.

## Number of parameters of SMIP

As a direct consequence of replacing all the MIP modes by a single mode, all the above-mentioned MIP matrices of weights and biases are replaced by a matrix of weights of size 16x4 and a vector of biases of size 16 for predicting 4x4 blocks, a matrix of weights of size 16x8 and a vector of biases of size 16 for predicting 8x4, 4x8, and 8x8 blocks, and a matrix of weights of size 64x8 and a vector of biases of size 64 for predicting the other blocks. This means that, in SMIP, 800 coefficients are to be coded, which amounts to 1 kilobyte of memory consumption when storing these coefficients as 10-bit values. In comparison, in MIP, 6336 coefficients are to be coded, which corresponds to 7.92 kilobytes of memory consumption when storing them as 10-bit values. The memory footprint due to the storage of the parameters of the matrix intra prediction is thus divided by 7.92.

## Signalization of SMIP

The signalization of SMIP is simpler than that of MIP as it involves no MPM specific to SMIP. In details, the flag for signaling MIP is removed and a flag for signaling SMIP, called smipFlag, is placed after the flag signaling the first MPM mode. On the encoder side, let us say that the current intra prediction is a MPM and the flag signaling a MPM with value 1 has already been written to the bitstream. If the current intra prediction mode is the first MPM, the flag signaling the first MPM mode with value 0 is written to the bitstream, and smipFlag is not written to the bitstream. If the current intra prediction mode is SMIP, the flag signaling the first MPM mode with value 1 is written to the bitstream, and smipFlag with value 1 is written to the bitstream. If the current intra prediction mode is neither the first MPM nor SMIP but it corresponds to one of the MPMs, the flag signaling the first MPM mode with value 1 and smipFlag with value 0 are written to the bitstream, then the remaining flags signaling the subsequent MPM modes are written to the bitstream. In summary, the complete codeword of the MPMs and SMIP are given in Table 1.

Table 1: codeword for each MPM and SMIP

| Candidate Index | code |
| --- | --- |
| MPM[0] | 10 |
| SMIP | 111 |
| MPM[1] | 1100 |
| MPM[2] | 11010 |
| MPM[3] | 110110 |
| MPM[4] | 1101110 |
| MPM[5] | 1101111 |

## Mapping from SMIP to planar when deriving the list of MPMs

When deriving the list of MPMs, instead of using a mapping of 65 coefficients for associating each MIP mode to a one of the 67 conventional intra prediction mode in H.266/VVC, SMIP is mapped to planar. Moreover, as said in Section 3, there is no MPM for signaling SMIP. This means SMIP involves no mapping from the 67 conventional intra prediction modes in H.266/VVC to SMIP whereas the mapping from the 67 conventional modes to MIP modes involves 105 coefficients. Therefore, the simplification of the mapping in SMIP saves 170 coefficients, i.e. 0.17 kilobytes of memory consumption when coding the coefficients as 8-bit values.

# Experimental Results

Simulations were performed according to CTC described in [2]. AI and RA results are reported below.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.14% | 0.02% | 0.06% | 97% | 100% |
| Class C | 0.13% | 0.12% | 0.18% | 94% | 97% |
| Class E | 0.18% | 0.13% | 0.11% | 97% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.20% | 0.21% | -0.07% | 97% | 101% |
| Class F | 0.08% | 0.21% | 0.04% | 103% | 107% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.05% | 0.10% | 0.12% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.07% | -0.09% | 0.27% | 100% | 100% |
| Class F | 0.01% | -0.02% | 0.19% | 104% | 102% |

# Conclusion

This contribution document reports the results of the reduction of the MIP modes to a single mode. This incurs a reduction of the memory footprint of 7.09 kilobytes for a loss of 0.xx% in AI and xx% in RA.

# References

- Jonathan Pfaff, Björn Stallenberger, Michael Schäfer, Philipp Merkle, Philipp Helle, Tobias Hinz, Heiko Schwarz, Detlev Marpe, Thomas Wiegand. JVET-N0217: “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”. Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 14th meeting, Geneva, CH, 19-27 March 2019.

- JVET-M1010: “JVET common test conditions and software reference configurations”. Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 13th meeting, Marrakech, MA, 9-18 January 2019.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).