| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0524 |
| --- | --- |

| Title: | Non-CE2: alternative solutions for reducing the luma-chroma latency |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tangi Poirier, Edouard François, Fabrice Le Léannec | Email: | tangi.poirier@interdigital.com edouard.francois@interdigital.com fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes partitioning restrictions to reduce the latency due to chroma-to-luma dependency of CCLM and Chroma Residual Coding (CRS) coding tools. The proposal consists in enabling BT and TT partitioning only for luma blocks of size lower than or equal to 32x32 samples, and for chroma blocks of size lower than or equal to 16x16 samples. Above those dimensions, only NoSplit or QT split are enabled. These restrictions reportedly reduce the chroma reconstruction latency from one 64x64 luma block to one 32x32 luma block.

Three tests are reported. In Test1, the partitioning restrictions are applied without other change to VTM5. In Test2, the partitioning restrictions are applied, and CRS is activated in dual tree with the CRS factor derived from collocated reconstructed luma samples. In Test3, the partitioning restrictions are applied, and CRS is activated in dual tree with the CRS factor derived from reconstructed luma samples neighboring the luma block colocated with the chroma block.

For SDR content, reported BD-rate PSNR-Y, U, and V variations are summarized in the table below.

|  |  | AI |  |  | RA |  |  | LDB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| SDR test1 | 0.17% | 2.35% | 2.30% | 0.04% | 0.77% | 0.78% | 0.04% | 0.57% | 0.50% |
| SDR test2 | 0.18% | 2.11% | 1.86% | 0.07% | 0.15% | 0.18% | 0.03% | 0.04% | 0.28% |
| SDR test3 | 0.18% | 2.09% | 1.84% | 0.07% | 0.15% | 0.17% | 0.01% | 0.03% | 0.06% |

For HDR PQ content, the CTC anchors do not use LMCS. One additional test is reported with LMCS on and without using CRS in DualTree (“HDR LMCS”). Reported BD-rate DE100, PSNR-L100, wPSNR-Y, U and V variations for HDR PQ are summarized in the table below. Using CRS in DualTree (Test2 and Test3) is reported to be beneficial for the luma / chroma balance when LMCS is activated.

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL | wY | wU | wV | DE100 | PSNRL | wY | wU | wV |
| HDR Test1 | 5.82% | 0.24% | 0.23% | 5.89% | 8.70% | 4.70% | 0.48% | 0.13% | 3.21% | 4.73% |
| HDR LMCS | 14.36% | -1.84% | -1.41% | 20.50% | 17.61% | 10.94% | -1.69% | -1.51% | 17.77% | 11.81% |
| HDR Test2 | 6.42% | -1.48% | -1.03% | 9.70% | 4.40% | 4.54% | -1.46% | -1.24% | 8.18% | 1.70% |
| HDR Test3 | 6.52% | -1.47% | -1.01% | 9.72% | 4.72% | 4.79% | -1.45% | -1.23% | 8.16% | 1.80% |

# Introduction

In current VVC draft specification, two chroma coding tools, CCLM and Chroma Residual Coding (CRS), require for processing a chroma block to access co-located luma samples. This generates a chroma reconstruction latency. The latency can be measured as the amount of luma data needed to be reconstructed before the chroma can be reconstructed without interruption afterwards. In the current VTM5 design, the latency is one 64x64 luma block.

CE2 aims at addressing this issue, but from discussions with hardware experts, the solutions investigated in CE2 do not solve properly or completely the problem. The proposal investigates an alternative solution, resulting in a latency of one 32x32 luma block.

# Proposal

From those discussions, it was suggested to investigate an alternative solution, resulting in a latency of one 32x32 luma block. The solution consists in applying the following partitioning restrictions:

- BT and TT split are disabled for luma blocks larger than 32x32 samples.

- BT and TT split are disabled for chroma blocks larger than 16x16 samples.

Therefore, at VDPU (64x64 luma / 32x32 chroma) level, only the four following configurations are enabled:

- Luma has no split and chroma have no split

- Luma has no split but chroma has QT split

- Luma has QT split and chroma has QT split

- Luma has QT split but chroma has no split

In any of those 4 cases, the luma reconstruction process for a 64x64 block can be performed with four 32x32 blocks in serial, and therefore four 16x16 chroma can generate CCLM prediction (or CRS) without large latency.

Note that in current CTCs, luma cannot be split by BT or TT when the block size is 64x64. But chroma can be split by BT or TT when s the block size is 32x32. Thus, the proposed restrictions removes this latest possibility.

# Simulation results

Three tests are reported in this contribution. In all cases, the proposed restrictions are applied in intra and inter tile groups.

- Test1: VTM5 + partitioning restrictions

  - The partitioning restrictions are applied without other change to VTM5.

  - In intra tile groups (Dual Tree), CRS is not activated.

  - In inter tile groups, CRS is activated, and the CRS factor of a chroma TU is derived from the collocated prediction luma samples.

- Test2: VTM5 + partitioning restrictions + CRS in dual tree using coloc reco luma samples

  - The partitioning restrictions are applied.

  - In intra tile groups (Dual Tree), CRS is activated, and the CRS factor of a chroma TU is derived from the collocated reconstructed luma samples.

  - In inter tile groups, CRS is activated, and the CRS factor of a chroma TU is derived from the collocated prediction luma samples.

- Test3: VTM5 + partitioning restrictions + CRS in dual tree using neighboring reco luma samples

  - The partitioning restrictions are applied.

  - In intra tile groups (Dual Tree), CRS is activated, and the CRS factor of a chroma TU is derived from reconstructed luma samples neighboring the luma block collocated with the top-left sample position of the chroma block.

  - In inter tile groups, CRS is activated, and the CRS factor of a chroma TU is derived from the collocated prediction luma samples.

The table below summarizes the settings for the three tests.

|  | CRS in dual tree | CRS in single tree |
| --- | --- | --- |
| Test1 | No | Using coloc pred luma samples |
| Test2 | Using coloc reco luma samples | Using coloc pred luma samples |
| Test3 | Using coloc neighbor luma samples | Using coloc pred luma samples |

Tests are reported for SDR and HDR PQ content.

## SDR results

### SDR test1 – Partitioning restrictions

All Intra

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.27% | 3.86% | 3.09% | 88% | 99% |
| Class A2 | 0.18% | 2.39% | 1.41% | 92% | 100% |
| Class B | 0.15% | 2.60% | 3.27% | 93% | 99% |
| Class C | 0.09% | 0.46% | 0.62% | 95% | 96% |
| Class E | 0.18% | 2.91% | 3.03% | 94% | 97% |
| Overall | 0.17% | 2.35% | 2.30% | 92% | 98% |
| Class D | 0.03% | 0.52% | 0.27% | 94% | 98% |
| Class F | 0.15% | 1.22% | 1.27% | 96% | 97% |

Random Access

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.07% | 0.84% | 0.77% | 100% | 101% |
| Class A2 | 0.09% | 0.95% | 0.70% | 99% | 102% |
| Class B | 0.05% | 1.20% | 1.32% | 99% | 102% |
| Class C | -0.03% | 0.06% | 0.18% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.77% | 0.78% | 99% | 101% |
| Class D | 0.03% | 0.11% | 0.27% | 100% | 101% |
| Class F | 0.07% | 0.27% | 0.62% | 71% | 97% |

Low delay B

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.65% | 0.42% | 102% | 106% |
| Class C | 0.00% | 0.01% | 0.03% | 102% | 109% |
| Class E | 0.15% | 1.18% | 1.26% | 102% | 108% |
| Overall | 0.04% | 0.57% | 0.50% | 102% | 107% |
| Class D | 0.03% | 0.45% | 0.54% | 101% | 103% |
| Class F | 0.03% | -0.18% | 0.08% | 102% | 108% |

### SDR test2 – Partitioning restrictions + CRS dualTree using collocated luma samples

All Intra

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.13% | 4.43% | 3.56% | 90% | 100% |
| Class A2 | 0.03% | 3.63% | 2.73% | 93% | 101% |
| Class B | 0.17% | 2.53% | 3.07% | 94% | 104% |
| Class C | 0.27% | -0.97% | -1.17% | 96% | 103% |
| Class E | 0.30% | 1.67% | 1.32% | 94% | 101% |
| Overall | 0.18% | 2.11% | 1.86% | 94% | 102% |
| Class D | 0.17% | -0.62% | -1.03% | 97% | 102% |
| Class F | 0.43% | -0.17% | -0.26% | 97% | 102% |

Random Access

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.07% | 0.84% | 0.77% | 101% | 101% |
| Class A2 | 0.09% | 0.95% | 0.70% | 100% | 102% |
| Class B | 0.05% | 0.81% | 0.95% | 100% | 103% |
| Class C | 0.07% | -1.79% | -1.60% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.15% | 0.18% | 100% | 102% |
| Class D | 0.06% | -0.66% | -1.12% | 100% | 101% |
| Class F | 0.30% | -0.76% | -0.66% | 100% | 102% |

Low delay B

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.48% | 0.23% | 102% | 106% |
| Class C | 0.00% | -0.25% | -0.12% | 101% | 110% |
| Class E | 0.09% | -0.31% | 0.89% | 102% | 108% |
| Overall | 0.03% | 0.04% | 0.28% | 102% | 108% |
| Class D | -0.04% | 0.26% | 0.65% | 101% | 103% |
| Class F | 0.06% | -0.60% | -0.38% | 102% | 109% |

### SDR test3 – Partitioning restrictions + CRS dualTree using luma samples neighboring the collocated luma block

All Intra

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.13% | 4.51% | 3.58% | 90% | 100% |
| Class A2 | 0.03% | 3.65% | 2.70% | 93% | 101% |
| Class B | 0.18% | 2.53% | 3.03% | 94% | 103% |
| Class C | 0.25% | -1.06% | -1.11% | 97% | 103% |
| Class E | 0.30% | 1.57% | 1.19% | 94% | 101% |
| Overall | 0.18% | 2.09% | 1.84% | 94% | 102% |
| Class D | 0.17% | -0.49% | -0.95% | 97% | 103% |
| Class F | 0.44% | -0.17% | -0.27% | 97% | 103% |

Random Access

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.07% | 0.84% | 0.77% | 100% | 101% |
| Class A2 | 0.09% | 0.95% | 0.70% | 100% | 102% |
| Class B | 0.05% | 0.79% | 0.95% | 100% | 102% |
| Class C | 0.08% | -1.76% | -1.66% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.15% | 0.17% | 100% | 101% |
| Class D | 0.10% | -0.97% | -0.91% | 100% | 100% |
| Class F | 0.32% | -0.85% | -0.73% | 100% | 102% |

Low delay B

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.36% | -0.06% | 102% | 106% |
| Class C | 0.03% | -0.10% | -0.22% | 101% | 109% |
| Class E | 0.00% | -0.36% | 0.63% | 102% | 108% |
| Overall | 0.01% | 0.03% | 0.06% | 102% | 108% |
| Class D | 0.03% | -0.13% | 0.49% | 101% | 103% |
| Class F | 0.11% | -0.41% | -0.23% | 102% | 108% |

## HDR PQ content

As a reminder, the official HDR CTC anchor use Adaptive luma QP on, LMCS off.

In the tables below, “wY”, wU”, “wV” stand for wPSNRY, wPSNRU, wPSNRV.

For information, deactivation of CCLM for HDR PQ leads to the following performance:

All Intra

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | wY | wU | wV |
| Overall | 25.87% | 2.51% | 2.36% | 69.67% | 93.56% |

Random Access

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | wY | wU | wV |
| Overall | 19.40% | 1.28% | 1.18% | 61.37% | 71.16% |

### HDR Test1 – Partitioning restrictions

For HDR Test1, the anchor is the HDR CTC anchor, that is, with Adaptive luma QP on, but LMCS off. Test1 uses the same settings, with in addition the partitioning restrictions.

All Intra

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | wY | wU | wV |
| Overall | 5.82% | 0.24% | 0.23% | 5.89% | 8.70% |

Random Access

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | wY | wU | wV |
| Overall | 4.70% | 0.48% | 0.13% | 3.21% | 4.73% |

### HDR LMCS – Partitioning restrictions + LQP off LMCS on

In HDR LMCS, Adaptive luma QP is deactivated and LMCS is activated. CRS is not activated in Dual Tree.

All Intra

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | wY | wU | wV |
| Overall | 14.36% | -1.84% | -1.41% | 20.50% | 17.61% |

Random Access

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | wY | wU | wV |
| Overall | 10.94% | -1.69% | -1.51% | 17.77% | 11.81% |

### HDR Test2 – Partitioning restrictions + LQP off LMCS on + CRS dualTree using collocated luma samples

In HDR Test2, Adaptive luma QP is deactivated and LMCS is activated. In addition, CRS is activated in Dual Tree, and in Dual tree the CRS factor is derived from the collocated reconstructed luma samples.

All Intra

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | wY | wU | wV |
| Overall | 6.42% | -1.48% | -1.03% | 9.70% | 4.40% |

Random Access

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | wY | wU | wV |
| Overall | 4.54% | -1.46% | -1.24% | 8.18% | 1.70% |

### HDR Test3 – Partitioning restrictions + LQP off LMCS on + CRS dualTree using luma samples neighboring the collocated luma block

In HDR Test3, Adaptive luma QP is deactivated and LMCS is activated. In addition, CRS is activated in Dual Tree, and in Dual tree the CRS factor is derived from the reconstructed luma samples neighboring the luma block collocated with the chroma block.

All Intra

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | wY | wU | wV |
| Overall | 6.52% | -1.47% | -1.01% | 9.72% | 4.72% |

Random Access

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | wY | wU | wV |
| Overall | 4.79% | -1.45% | -1.23% | 8.16% | 1.80% |

### HDR Tests summary

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | wY | wU | wV |
| noCclm | 25.87% | 2.51% | 2.36% | 69.67% | 93.56% |
| HDR Test1 | 5.82% | 0.24% | 0.23% | 5.89% | 8.70% |
| HDR LMCS | 14.36% | -1.84% | -1.41% | 20.50% | 17.61% |
| HDR Test2 | 6.42% | -1.48% | -1.03% | 9.70% | 4.40% |
| HDR Test3 | 6.52% | -1.47% | -1.01% | 9.72% | 4.72% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | wY | wU | wV |
| noCclm | 19.40% | 1.28% | 1.18% | 61.37% | 71.16% |
| HDR Test1 | 4.70% | 0.48% | 0.13% | 3.21% | 4.73% |
| HDR LMCS | 10.94% | -1.69% | -1.51% | 17.77% | 11.81% |
| HDR Test2 | 4.54% | -1.46% | -1.24% | 8.18% | 1.70% |
| HDR Test3 | 4.79% | -1.45% | -1.23% | 8.16% | 1.80% |

# Conclusion

The proposed restrictions reduce chroma reconstruction latency from one 64x64 luma block to one 32x32 luma block, with reasonable impact on the coding efficiency, especially in inter configurations.

Acknowledgments.

Thanks to Broadcom, RealTek and MediaTek for their fruitful inputs helping to clarify the definition of the luma-chroma latency problem and proposing possible solutions.

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).