| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0525-v1 |
| --- | --- |

| Title: | AHG16: Removal of PCM mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman | Email: | tim.hellman@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a claimed simplification to VVC by the removal of PCM mode.  It contends that PCM mode has been simply carried over since the H.264/AVC standard without any evidence of its benefit.  It further claims that PCM support is an implementation and testing burden to decoder implementations, and that its continued inclusion adds unnecessary complication to the standard.  The proposal recommends removing PCM mode in its entirety from the text.

# Description

PCM mode allows a CU’s pixel data to be transmitted directly in a stream without any processing or entropy coding.  It was first introduced in the H.264/AVC standard.  The justification given for its inclusion was to prevent negative compression for very low Qp and lossless streams.  However, its subsequent use has been for niche applications, if at all.  High-volume commercial encoders do not use it, and in general it’s only encountered in test streams.

PCM support in decoder implementations does not come for free.  There are buffering and throughput considerations in the arithmetic decoder and throughout the pixel pipeline.  Additionally, there are special-case filtering conditions in all the filter stages that require line-buffer storage of PCM state and extensive corner-case testing.  The mode even adds complexity to the spec text – there are over 100 lines of text needed to specify all the details of PCM mode.

Note that the VVC standard already includes lossless coding tools, such as quantization bypass and BDPCM.  There does not appear to be any data that shows any additional benefit derived from PCM mode to justify its inclusion in the standard.

# Experimental Results

No experimental results are provided as the CTC specifies that PCM mode is always disabled, therefore removing it has no coding efficiency impact.

# Conclusions

Producing a new video standard gives the opportunity to prune existing tools as well as to specify new ones.  H.265/HEVC, for example, eliminated the complexity of H.264/AVC’s interlaced coding tools.  Removing the seldom-used PCM mode would help to offset the added complexity of VVC’s many new coding tools.

This proposal recommends its removal.  Short of that, further study of the empirical benefit of PCM mode is encouraged.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001-v7, the 14th JVET meeting, March 2019, Geneva, CH

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.

# Text changes

Text changes are provided in a separate document.