| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0526-v1 |
| --- | --- |

| Title: | AHG16: Setting the minimum CTU size to 32x32 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman Minhua Zhou | Email: | tim.hellman@broadcom.com minhua.zhou@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution claims that the present minimum CTU size of 16x16 represents a burden to decoder implementation without any corresponding benefit.  Reported VTM-5.0 test results show a 33.67% RA coding loss when using a 16x16 CTU size in place of the 128x128 CTU size in the CTC.  It is alleged that such a coding loss makes the use of the 16x16 CTU size impractical.

This contribution recommends setting the minimum CTU size to 32x32, and using a more efficient encoding of CTU size in the SPS header.

# Introduction

VTM-5.0 allows 4 different CTU sizes: 16x16, 32x32, 64x64 and 128x128, expanding on the H.265/HEVC set by one (128x128).  However, it is asserted that the smallest CTU size (16x16) represents a significant burden on decoder implementations, because decoding a CTU requires a certain amount of overhead, and that overhead becomes significant when the CTU is so small.  This is claimed to be true even for software implementations, as can be seen by the large increases in decoder runtime shown in section 3.1.  Additionally, a 16x16 CTU corresponds to an 8x8 chroma CTB in the 4:2:0 color space.  This represents an outlier for hardware blocks, like loop filters, which are often designed to operate efficiently on 16x16 block sizes.

Moreover, it is asserted that the 16x16 CTU size provides no benefit to encoders due to the significant loss in coding efficiency that results from its use.  Results in section 3 show a 37% average RA coding loss, with losses as high as 63% for class 1.  It’s unlikely that any encoder implementation could be successful with such a large coding loss.

H.265/HEVC implements a level restriction such that 16x16 CTU sizes are not permitted at level 5 and above.  That solves the problem for large image sizes, but still forces decoders to implement and test that CTU size.  Additionally, 16x16 throughput is still an issue for multi-decode implementations, such as quad HD decoding in a 4K system.

The CTU size is encoded in the SPS header in a UE-encoded syntax element called log2_ctu_size_minus_2.  This encoding presently allows any CTU size from 4x4 onwards, even though sizes below 16x16 are not allowed by the encoder.

# Proposed Solution

The proposed solution is eliminate the 16x16 CTU size by changing the SPS header syntax element from log2_ctu_size_minus2 to log2_ctu_size_minus5.  A value of 0 for this syntax element gives a CTU size of 32x32 (log2_ctu_size = 5).  In addition, it is proposed to change the syntax element encoding from UE to U(2), since only 3 values are legal (0 - 32x32, 1 - 64x64, and 2 - 128x128).  A proposed bitstream conformance requirement restricts legal values for this syntax element to 0, 1, and 2.

# Experimental Results

The tests were carried out by using VTM5.0 reference software for CTU sizes 16x16, 32x32, and 64x64. The summary results are provided in Table 1, Table 2 and Table 3.

When compared to the VTM5.0 anchor which uses CTU size 128x128, the average BD-rate increase in AI/RA/LD_B is 8.32%/36.55%/33.67% for CTU size 16x16, 1.60%/10.52%/9.24% for CTU size 32x32, and 0.28%/1.83%/1.27% for CTU size 64x64, respectively. An encoder/decoder mismatch was observed in VTM5.0 when the CTU size is set to 16x16.

Although the run time may not be accurate due to the heterogeneous nature of the LSF farm, trend is that the decoder run time increases when the CTU size decreases.

## CTU Size 16x16 vs Anchor (CTU Size 128x128)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 14.73% | 73.64% | 65.77% | 32% | 219% |
| Class A2 | 8.47% | 41.52% | 30.57% | 35% | 201% |
| Class B | 6.64% | 36.01% | 45.50% | 37% | 183% |
| Class C | 3.90% | 15.18% | 16.09% | 43% | 135% |
| Class E | 10.47% | 46.04% | 55.85% | 38% | 181% |
| Overall | 8.32% | 40.24% | 41.58% | 37% | 179% |
| Class D | 2.18% | 10.90% | 10.20% | 45% | 124% |
| Class F (optional) | 6.73% | 22.42% | 24.82% | 44% | 175% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 62.93% | 112.66% | 107.23% | 22% | 205% |
| Class A2 | 48.93% | 76.31% | 62.90% | 27% | 199% |
| Class B | 31.22% | 61.42% | 70.24% | 27% | 193% |
| Class C | 14.51% | 25.25% | 25.80% | 30% | 172% |
| Class E |  |  |  |  |  |
| Overall | 36.65% | 65.00% | 64.32% | 27% | 191% |
| Class D | 8.77% | 18.22% | 17.53% | 40% | 169% |
| Class F (optional) | 21.66% | 36.59% | 39.49% | 39% | 248% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 31.58% | 61.61% | 76.87% | 31% | 183% |
| Class C | 15.25% | 27.30% | 29.12% | 32% | 150% |
| Class E | 61.71% | 108.75% | 140.36% | 40% | 250% |
| Overall | 33.67% | 61.96% | 76.82% | 34% | 185% |
| Class D | 9.19% | 19.09% | 21.69% | 45% | 150% |
| Class F (optional) | 29.17% | 43.33% | 47.27% | 50% | 257% |

Table 3. VTM5.0 (anchor CTU size 128x128) vs. VTM5.0 with CTU size 16x16

## CTU Size 32x32 vs Anchor (CTU Size 128x128)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.69% | 15.42% | 13.70% | 84% | 125% |
| Class A2 | 1.68% | 7.64% | 5.16% | 105% | 134% |
| Class B | 1.21% | 6.48% | 8.70% | 95% | 120% |
| Class C | 0.85% | 2.33% | 2.51% | 84% | 106% |
| Class E | 2.08% | 8.42% | 12.20% | 79% | 107% |
| Overall | 1.60% | 7.56% | 8.15% | 89% | 118% |
| Class D | 0.36% | 1.38% | 1.23% | 91% | 102% |
| Class F (optional) | -0.08% | 4.12% | 4.44% | 93% | 118% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 18.18% | 26.04% | 25.21% | 41% | 122% |
| Class A2 | 15.06% | 18.71% | 17.07% | 51% | 121% |
| Class B | 9.12% | 14.84% | 17.43% | 51% | 116% |
| Class C | 3.14% | 6.03% | 6.20% | 64% | 117% |
| Class E |  |  |  |  |  |
| Overall | 10.52% | 15.51% | 15.92% | 52% | 118% |
| Class D | 1.47% | 4.71% | 4.38% | 74% | 112% |
| Class F (optional) | 5.34% | 9.50% | 10.05% | 62% | 146% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 9.06% | 18.01% | 24.08% | 55% | 109% |
| Class C | 3.15% | 8.20% | 8.79% | 62% | 96% |
| Class E | 17.68% | 26.73% | 33.41% | 52% | 136% |
| Overall | 9.24% | 16.92% | 21.32% | 56% | 110% |
| Class D | 1.06% | 4.42% | 4.68% | 79% | 104% |
| Class F (optional) | 8.45% | 16.21% | 17.51% | 71% | 137% |

Table 2. VTM5.0 (anchor CTU size 128x128) vs. VTM5.0 with CTU size 32x32

## CTU Size 64x64 vs Anchor (CTU Size 128x128)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.26% | 0.94% | 1.12% | 95% | 102% |
| Class A2 | 0.35% | 0.72% | 0.77% | 98% | 97% |
| Class B | 0.23% | 0.62% | 0.72% | 94% | 102% |
| Class C | 0.23% | 0.72% | 0.94% | 85% | 85% |
| Class E | 0.35% | 0.13% | 0.54% | 98% | 98% |
| Overall | 0.28% | 0.63% | 0.81% | 93% | 97% |
| Class D | 0.10% | 0.42% | 0.31% | 91% | 96% |
| Class F (optional) | -1.25% | -0.61% | -0.41% | 98% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 3.06% | 2.95% | 3.72% | 83% | 101% |
| Class A2 | 3.00% | 3.47% | 3.73% | 85% | 103% |
| Class B | 1.63% | 2.70% | 3.42% | 88% | 101% |
| Class C | 0.28% | 1.45% | 1.47% | 97% | 103% |
| Class E |  |  |  |  |  |
| Overall | 1.83% | 2.57% | 3.02% | 88% | 102% |
| Class D | 0.03% | 0.96% | 1.05% | 99% | 100% |
| Class F (optional) | 0.12% | 0.90% | 1.41% | 85% | 107% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.37% | 3.64% | 5.00% | 101% | 104% |
| Class C | 0.22% | 1.64% | 2.05% | 90% | 95% |
| Class E | 2.50% | 3.89% | 4.37% | 81% | 100% |
| Overall | 1.27% | 3.04% | 3.86% | 92% | 100% |
| Class D | -0.23% | 0.55% | 1.17% | 96% | 95% |
| Class F (optional) | 0.90% | 3.56% | 3.14% | 86% | 99% |

Table 1. VTM5.0 (anchor CTU size 128x128) vs. VTM5.0 with CTU size 64x64

# Conclusions

This contribution recommends changing the minimum CTU size from 16x16 to 32x32 by redefining a SPS syntax element.  It is asserted that this will ease decoder implementation without any practical loss to encoder flexibility.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001-v7, the 14th JVET meeting, March 2019, Geneva, CH

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.

# Text changes

In syntax table 7.3.2.3 “Sequence parameter set RBSP syntax”, change the syntax highlighted in yellow:

| qtbtt_dual_tree_intra_flag | u(1) |
| --- | --- |
| log2_ctu_size_minus5 | u(2) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |

In section 7.4.3.3 “Sequence parameter set RBSP semantics”, change the text highlighted in yellow.

log2_ctu_size_minus5 plus 5 specifies the luma coding tree block size of each CTU.  It is a requirement of bitstream conformance that the value of log2_ctu_size_minus5 be less than or equal to 2.

log2_min_luma_coding_block_size_minus2 plus 2 specifies the minimum luma coding block size.

The variables CtbLog2SizeY, CtbSizeY, MinCbLog2SizeY, MinCbSizeY, MinTbLog2SizeY, MaxTbLog2SizeY, MinTbSizeY, MaxTbSizeY, PicWidthInCtbsY, PicHeightInCtbsY, PicSizeInCtbsY, PicWidthInMinCbsY, PicHeightInMinCbsY, PicSizeInMinCbsY, PicSizeInSamplesY, PicWidthInSamplesC and PicHeightInSamplesC are derived as follows:

CtbLog2SizeY = log2_ctu_size_minus5 + 5		(7-10)

CtbSizeY = 1  <<  CtbLog2SizeY		(7-11)