| ,Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0528 |
| --- | --- |

| Title: | Non-CE6: Simplified Transform Selection For ISP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Fabrice Le Léannec, Tangi Poirier, Philippe De Lagrange 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

In Geneva meeting, a unified transform selection for both ISP and Implicit MTS was adopted JVET-N0866. On one hand, this unification is not applicable anymore due to the adoption of LFNST and MIP (JVET-O0529). On the other hand, the ISP transform design adds new transform pairs, compared to MTS pairs, which result from the combination of DCT2 and DST7. This increases the overall design complexity and leads to additional difficulties for both future computational and perceptual optimization.

This contribution proposes a new design of ISP transform, which is unified with MTS design. This comes with 0.02% and 0.00% luma loss in AI and RA configuration.

# Introduction

In VTM5.0, the ISP transform selection is driven by this formula:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? DST7 : DCT2

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? DST7 : DCT2

where trTypeHor and trTypeVer are the horizontal and vertical transform types and nTbW and nTbH are the width and height of the transform block. This design was adopted in the last meeting, as it is a unified design with the implicit MTS (JVET-N0866). However, as can be seen in JVET-O0529, implicit MTS design leads to RD loss in VTM5.0. Therefore, implicit MTS design need to be modified and the interest behind the current design of ISP transform selection remains only due to its coding performance.

The two formulas indicate that the Implicit MTS transform selection can have the following pairs {trTypeHor,TrTypeVer}:

- {DST7,DST7}

- {DST7,DCT2}

- {DCT2,DST7}

- {DCT2,DCT2}

Compared to MTS design, ISP generates the new pairs of DST7,DCT2 and DCT2,DST7. The addition of these pairs leads to increased overall design complexity. Furthermore, it increases the complexity of the computational and perceptual optimization. For example, the optimal lambda determination can potentially depend on the transform pairs, and therefore, the more pairs the more variety of lambda. In terms of perceptual optimization, MTS transform selection distinguishes the DCT2 from DST7/DCT8 combinations, where no mix between DST7/DCT8 and DCT2 exists. This an important feature as the 2D DCT2 transform is well studied, where its coefficients can be quantized according to perceptual criteria. As DCT8 and DST7 are complementary to each other, the perceptual significance of their 2D basis functions is consistent. However, DST7 and DCT2 combination is still another pair needed to be studied. Therefore, the design can be simplified if such a combination is avoided.

# Proposition

This contribution proposes to use the same design of MTS for ISP. Namely, if ISP is used and MTS condition is met for the current subblock, MTS index is implicitly set to 1. In other words, ISP transform selection is as follows:

bool mtsAllowed = TU::isMTSAllowed( tu, COMPONENT_Y );

if (mtsAllowed)

tu.mtsIdx = 1;

else

tu.mtsIdx = 0;

Effectively, the method proposed can be re-interpreted in this way:

DstHorOk = ( nTbW >= 4  &&  nTbW <= 32 )

DstVerOk = ( nTbH >= 4  &&  nTbH <= 32 )

If (DstHorOk && DstVerOk)

trTypeHor = trTypeVer = DST7;

Else

trTypeHor = trTypeVer = DCT2;

# Experimental Results

The simulations were ran with VTM5.0 using the common testing conditions. The anchor is VTM5.0 and the test is the proposed solution. The following results were obtained:

Table 3 Proposed solution vs VTM5.0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U | V |  | EncT |  | DecT |  |
| Class A1 | 0.01% |  | 0.03% | 0.04% |  | 101% |  | 102% |  |
| Class A2 | 0.01% |  | 0.05% | -0.01% |  | 100% |  | 100% |  |
| Class B | 0.02% |  | 0.02% | 0.03% |  | 99% |  | 100% |  |
| Class C | 0.00% |  | 0.03% | 0.12% |  | 99% |  | 98% |  |
| Class E | 0.06% |  | 0.13% | 0.16% |  | 100% |  | 99% |  |
| Overall | 0.02% |  | 0.05% | 0.07% |  | 100% |  | 100% |  |
| Class D | -0.02% |  | 0.13% | -0.07% |  | 98% |  | 97% |  |
| Class F | -0.07% |  | 0.01% | 0.02% |  | 105% |  | 101% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  |  | V |  | EncT |  | DecT |
| Class A1 | 0.00% | -0.07% |  |  | 0.06% |  | 99% |  | 97% |
| Class A2 | 0.00% | 0.28% |  |  | 0.07% |  | 98% |  | 99% |
| Class B | 0.01% | 0.04% |  |  | -0.17% |  | 100% |  | 100% |
| Class C | -0.03% | -0.06% |  |  | -0.19% |  | 98% |  | 105% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.04% |  |  | -0.08% |  | 99% |  | 100% |
| Class D | 0.01% | -0.05% |  |  | -0.21% |  | 100% |  | 106% |
| Class F | -0.08% | -0.01% |  |  | 0.14% |  | 98% |  | 92% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.18% |  |  | 0.15% |  | 102% |  | 99% |
| Class C | 0.03% | 0.15% |  |  | -0.02% |  | 101% |  | 101% |
| Class E | 0.00% | 0.22% |  |  | 0.62% |  | 102% |  | 100% |
| Overall | 0.01% | 0.18% |  |  | 0.21% |  | 102% |  | 100% |
| Class D | 0.03% | -0.76% |  |  | -0.01% |  | 101% |  | 97% |
| Class F | -0.06% | 0.28% |  |  | -0.24% |  | 102% |  | 101% |

From the results in Table 3, the new ISP design results in a small loss of  0.02%, which is reasonable considering the proposed simplification.

# Spec change

–	if IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and both of nTbW and TbH are greator or equal to 4 and less or equal 32, tu_mts_idx[ xTbY ][ yTbY ] is set to 1

–	If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are both equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT or both sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA, trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(8-964)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(8-965)

- Otherwise, if cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 8-19 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-18 depending on tu_mts_idx[ xTbY ][ yTbY ]

# Conclusion

This contribution proposes a new transform selection design that is aligned with MTS, such that the overall transform design is simplified. This simplification results in negligible RD loss.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).