| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET- O0529 |
| --- | --- |

| Title: | Non-CE6 Interaction between Implicit MTS and LFNST and MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Fabrice Le Léannec, Tangi Poirier 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Implicit MTS is a recent tool in VVC that is a faster version of the explicit MTS where single pair horizontal and vertical transform is tested instead of the full pairs of MTS. The interest behind the implicit MTS mode is its rapidity compared to the explicit MTS, and its high gain when compared with the case of MTS is switched off.

In VTM5.0, this tool becomes a lossy tool. This is due to the adoption of LFNST and MIP. This is mainly because LFNST and MIP are trained tools that did not consider the special transform design of Implicit MTS, where a combination between DCT2 and DST7 occurs.

This contribution proposes a new design of Implicit MTS transform, where the transform pairs are set to DCT2 when LFNST or MIP is activated for the current CU. This proposed solution provides a gain of 0.61% on AI when Implicit MTS is used with MTS off as anchor.

# Introduction

In VTM5.0, the Implicit MTS transform selection is driven by this formula:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? DST7 : DCT2	(8-965)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? DST7 : DCT2	(8-966)

where trTypeHor and trTypeVer are the horizontal and vertical transform types and nTbW and nTbH are the width and height of the transform block. The two formula indicates that the Implicit MTS transform selection can have the following pairs {trTypeHor,TrTypeVer}:

- {DST7,DST7}

- {DST7,DCT2}

- {DCT2,DST7}

- {DCT2,DCT2}

Intuitively, the selection does not fit MIP as the latter is based on DCT2 solely. On the other hand, it does not fit LFNST as it is probably trained on residuals of explicit MTS, where no combination of DST7,DCT2 is available.

To elaborate more, the implicit MTS was tested for the cases when LFNST is enabled and disabled. Namely:

- Test1:

Anchor is VTM5.0 with “--MTS=0”

Test is VTM5.0 with “--MTS=0 --MTSImplicit=1”

- Test2:

Anchor is VTM5.0 with “--MTS=0 --LFNST=0”

Test is VTM5.0 with “--MTS=0 --LFNST=0 --MTSImplicit=1”

The test results are given below:

Table 1 Test1: Implicit MTS vs MTS off

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 MTS=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.25% | -0.71% | -0.72% | 102% | 101% |
| Class A2 | -0.10% | -1.02% | -0.90% | 101% | 102% |
| Class B | 0.05% | -1.21% | -1.38% | 102% | 102% |
| Class C | 0.42% | -0.42% | -0.49% | 100% | 107% |
| Class E | 0.30% | -0.79% | -0.92% | 101% | 100% |
| Overall | 0.18% | -0.85% | -0.92% | 101% | 103% |
| Class D | 0.40% | -0.19% | -0.21% | 100% | 101% |
| Class F | 0.25% | -0.47% | -0.46% | 100% | 100% |

Table 2 Test2: Implicit MTS vs MTS off (LFNST is disabled)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 MTS=0 LFSNT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.48% | -1.74% | -1.86% | 98% | 102% |
| Class A2 | -1.02% | -1.35% | -1.34% | 99% | 102% |
| Class B | -0.96% | -1.73% | -1.73% | 101% | 103% |
| Class C | -0.45% | -0.87% | -0.80% | 99% | 101% |
| Class E | -1.34% | -1.92% | -1.74% | 98% | 104% |
| Overall | -1.01% | -1.51% | -1.48% | 99% | 102% |
| Class D | -0.30% | -0.72% | -0.62% | 100% | 103% |
| Class F | -0.43% | -0.69% | -0.82% | 99% | 99% |

Clearly, instead of gaining 1% (Table 2),  implicit MTS cause a loss of 0.2% (Table 1) due to the activation of LFNST.

# Proposition

This contribution proposes two solutions:

- Solution 1: Set trTypeVer and trTypeHor to DCT2 when CU LFNST flag or MIP flag is set.

- Solution 2: Set trTypeVer and trTypeHor to DCT2 when CU LFNST flag is set.

The interest behind the second option is that the transform selection becomes independent from prediction mode selection, which is generally a desired feature. However, it should be noted that VTM5.0 transform selection is partially mode dependent as LFNST is mode dependent and MIP is a special mode that uses only DCT2.

# Experimental Results

The simulations were ran with VTM5.0 using the common testing conditions. The anchor is VTM5.0 with MTS disabled (--MTS=0) and the two tests are with implicit MTS (--MTS=0 --MTSImplicit=1). The following results were obtained:

Table 3 Solution 1: New Implicit transform design vs MTS off

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 MTS=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.74% | -0.74% | -0.85% | 101% | 100% |
| Class A2 | -0.75% | -0.87% | -0.84% | 102% | 102% |
| Class B | -0.64% | -0.95% | -1.01% | 102% | 103% |
| Class C | -0.30% | -0.39% | -0.35% | 100% | 102% |
| Class E | -0.69% | -0.90% | -0.87% | 100% | 103% |
| Overall | -0.61% | -0.77% | -0.79% | 101% | 102% |
| Class D | -0.25% | -0.28% | -0.16% | 101% | 102% |
| Class F | -0.19% | -0.43% | -0.48% | 100% | 102% |

Table 4 Solution 2: New Implicit transform design vs MTS off

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 MTS=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.75% | -0.82% | -0.88% | 101% | 99% |
| Class A2 | -0.72% | -0.96% | -0.87% | 101% | 100% |
| Class B | -0.58% | -1.09% | -1.21% | 103% | 101% |
| Class C | -0.20% | -0.39% | -0.38% | 101% | 101% |
| Class E | -0.62% | -0.87% | -0.87% | 101% | 98% |
| Overall | -0.55% | -0.83% | -0.86% | 101% | 100% |
| Class D | -0.13% | -0.26% | -0.35% | 101% | 97% |
| Class F | -0.15% | -0.34% | -0.53% | 101% | 99% |

From the results in Table 3 and Table 4, it can be seen that 0.6% of gain is achieved with the proposed method. Solution 2, compared to Solution 1, results in a small loss of 0.06%.

# Conclusion

This contribution proposes a modification to the transform design of the Implicit MTS. Namely, when LFNST or MIP is enabled for the current CU block, DCT2 is use for both horizontal and vertical directions. This leads to 0.6% gain for the Implicit MTS tool, instead of the current 0.18% loss.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).