| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0531 |
| --- | --- |

| Title: | Non-CE4: On LIC for Affine |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiang Li, Xiaozhong Xu, Shan Liu 2747 Park Blvd., Palo Alto, CA 94306 | Email: | guichunli@tencent.com xlxiangli@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this document, the LIC for affine as provided in CE4-3.1 and the PROF tool as provided in CE4-2.1 were studied and two related methods are proposed. The first method simplifies  LIC flag derivation for constructed affine merge by enabling LIC for all constructed affine merge candidates. Compared with VTM-5.0 with LIC (as in CE4-3.1.a), the observed performance change of this method was RA -0.01%, 99%, 99%; LB -0.05%, 100%, 100% (RA -0.47%, 118%, 101%; LB -0.41%, 119%, 101% over VTM-5). The second method is to disable PROF when LIC is enabled for an affine coded block for lower complexity and better design consistency with BDOF. Compared with VTM-5.0 with LIC (as in CE4-3.1.a) and PROF (as in CE4-2.1.a) enabled, the observed performance change of the second proposed method was RA 0.05%, 99%, 99%; LB 0.00%, 99%, 100% (RA -0.91%, 119%, 101%; LB -0.68%, 120%, 102%  over VTM-5).

# Introduction

In CE4-3.1, LIC was enabled for both regular inter prediction and affine inter prediction mode. In test CE4-3.1.a, it was observed that the LIC tool brought -0.46% BD-rate change on Random Access configuration, and -0.37% BD-rate change on Low Delay B configuration.

In CE4-2.1, prediction refinement with optical flow PROF for affine was proposed. In test CE4-2.1.a, when Affine prediction and PROF were both applied on 4x4 affine subblocks, it was observed that BD-rate change is -0.48% for Random Access, and -0.32% for Low Delay B.

# Proposed Methods

## LIC setting for constructed affine merge

In the current LIC tool as provided in CE4-3.1, LIC is applied for both regular inter coded blocks and affine coded blocks, but limited to uni-predicted blocks with size no smaller than 64 luma samples. In affine merge mode, when inherited affine merge mode is used, the LIC flag is inherited from the neighboring affine block which is used for affine model inheritance. When constructed affine mode is used, the LIC flag is set to true if at least one of those neighboring blocks that are used as source to construct control points has LIC flag on.

Figure 1 - Constructed Affien Merge mode and the position of motion information source candidates

Since the constructed affine merge mode is not inheriting the affine model from any neighboring block, but is generating a new affine model using the regular motion information from the selected neighboring blocks, setting the current block’s LIC flag depending on the source neighboring blocks may not be optimal. In the proposed method, when LIC tool is enabled, and when constructed affine merge mode is used, the LIC flag of current block is always set to true, regardless of the LIC flag values of the blocks used for constructing the control points of the current block, unless LIC can not be enabled due to constraint on block size and/or the use of bi-prediction mode. The proposed method removes a condition checking for each candidate during the process of constructed affine merge mode.

## Disabling Affine PROF when Affine LIC is on

To save computational complexity, it is proposed to not perform PROF when LIC is on. It is asserted that this is in line with not using BDOF when LIC is on.

# Test results

## Test results of LIC setting for constructed affine merge

The proposed methods were implemented on top of VTM-5.0. VTM-5.0 was used as the first anchor, and VTM-5.0 with LIC tool as provided in CE4-3.1.a was used as second anchor. Test results are reported in Table-2.

Table 2 - Simulation results of setting LIC flag for constructed affine merge

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  | Over VTM-5.0+LIC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.69% | -0.54% | -0.59% | 123% | 99% | -0.03% | 0.01% | 0.04% | 99% | 99% |
| Class A2 | -0.38% | -0.32% | -0.41% | 113% | 100% | 0.01% | 0.06% | -0.02% | 99% | 99% |
| Class B | -0.48% | -0.28% | -0.49% | 117% | 102% | -0.02% | -0.07% | 0.03% | 100% | 100% |
| Class C | -0.37% | -0.57% | -0.61% | 119% | 105% | -0.01% | -0.08% | -0.15% | 99% | 96% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.47% | -0.42% | -0.52% | 118% | 101% | -0.01% | -0.03% | -0.02% | 99% | 99% |
| Class D | -0.11% | -0.07% | 0.03% | 116% | 107% | 0.02% | 0.18% | 0.09% | 100% | 99% |
| Class F | -0.69% | -0.41% | -0.44% | 113% | 105% | -0.02% | -0.06% | 0.02% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  | Over VTM-5.0+LIC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.55% | -0.07% | -0.58% | 123% | 98% | -0.06% | -0.05% | -0.24% | 99% | 98% |
| Class C | -0.47% | -0.38% | -0.70% | 127% | 104% | -0.04% | -0.25% | -0.36% | 100% | 102% |
| Class E | -0.11% | 0.03% | -0.63% | 103% | 103% | -0.03% | -0.32% | -0.45% | 100% | 100% |
| Overall | -0.41% | -0.15% | -0.63% | 119% | 101% | -0.05% | -0.18% | -0.33% | 100% | 100% |
| Class D | -0.06% | -0.85% | 0.14% | 119% | 102% | 0.10% | -0.79% | -0.15% | 99% | 100% |
| Class F | -1.90% | -1.68% | -1.74% | 119% | 102% | -0.13% | -0.16% | -0.26% | 100% | 99% |

It was observed that comparing with VTM-5.0 with LIC tool (as in the CE test), the proposed method has BD-rate change of -0.01% on Random Access, and -0.05% on Low Delay B. The runtime behavior remains the same as the LIC tool (as in the CE test).

## Test results of disabling Affine PROF off Affine LIC is on

The proposed methods were implemented on top of VTM-5.0. VTM-5.0 was used as first anchor, and VTM-5.0 with both LIC tool as provided in CE4-3.1.a and PROF for affine tool as provided in CE4-2.1.a was used as second anchor. Test results are reported in Table-3.

Table 3 - Simulation results of disabling Affine PROF off Affine LIC is on

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  | Over VTM-5.0+LIC+PROF |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.78% | -0.67% | -0.53% | 125% | 100% | 0.05% | -0.02% | 0.04% | 100% | 100% |
| Class A2 | -1.20% | -0.06% | -0.26% | 115% | 101% | 0.08% | 0.01% | 0.05% | 100% | 100% |
| Class B | -1.02% | -0.50% | -0.58% | 117% | 101% | 0.09% | 0.06% | -0.08% | 99% | 99% |
| Class C | -0.67% | -0.52% | -0.73% | 119% | 104% | 0.00% | 0.03% | -0.09% | 99% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.91% | -0.45% | -0.54% | 119% | 101% | 0.05% | 0.03% | -0.03% | 99% | 99% |
| Class D | -0.68% | -0.32% | -0.26% | 115% | 104% | 0.03% | -0.16% | 0.01% | 98% | 97% |
| Class F | -1.51% | -0.81% | -0.88% | 112% | 103% | 0.02% | 0.06% | -0.06% | 98% | 97% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  | Over VTM-5.0+LIC+PROF |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.87% | -0.39% | -0.46% | 125% | 100% | 0.01% | 0.05% | 0.27% | 100% | 100% |
| Class C | -0.66% | -0.36% | -0.64% | 126% | 104% | 0.02% | 0.05% | -0.09% | 99% | 100% |
| Class E | -0.38% | 0.02% | -0.78% | 105% | 101% | -0.05% | -0.42% | -0.31% | 99% | 99% |
| Overall | -0.68% | -0.28% | -0.60% | 120% | 102% | 0.00% | -0.07% | 0.01% | 99% | 100% |
| Class D | -0.51% | -0.78% | -0.02% | 119% | 100% | -0.05% | -0.43% | -0.39% | 99% | 96% |
| Class F | -2.95% | -2.45% | -3.02% | 119% | 101% | 0.13% | 0.21% | -0.74% | 99% | 99% |

Comparing with VTM-5.0 with both LIC and PROF tools enabled (as in two individual CE tests), it was observed that the proposed method has 0.05% BD-rate change on Random Access, and 0.00% BD-rage on LB. This suggests that there are interactions between LIC and BDOF tools on affine coded blocks. Disabling PROF when LIC is on for affine coded blocks has very minor impact on coding performance, and the complexity is reduced by avoiding the PROF process for those blocks.

# Conclusions

It was observed that always setting LIC flag to be true for constructed affine merge candidates simplifies the merge candidate construction process while obtaining minor BD-rate improvement.

Disabling affine PROF when LIC is on saves computational complexity with minor impact on coding performance. Moreover, this is consistent with the current LIC design that no BDOF is used when LIC is on.

# Working Draft Text

## LIC setting for constructed affine merge

…

#### 8.5.5.2	Derivation process for motion vectors and reference indices in subblock merge mode

…

- When sps_affine_enabled_flag is equal to 1, the derivation process for constructed affine control point motion vector merging candidates as specified in clause 8.5.5.6 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width and height (cbWidth, cbHeight), the availability flags availableA0, availableA1, availableA2, availableB0, availableB1, availableB2, availableB3 as inputs, and the availability flags availableFlagConstK, the reference indices refIdxLXConstK, prediction list utilization flags predFlagLXConstK, motion model indices motionModelIdcConstK, local illumination compensation flags licFlagConstK, bi-prediction weight indices bcwIdxConstK and cpMvpLXConstK[ cpIdx ] with X being 0 or 1, K = 1..6, cpIdx = 0..2 as outputs.

  - For K being replaced by 0…5, when availableFlagConstK is equal to 1, the following applies:

    - When refIdxL0ConstK is equal to 1 and refIdxL1ConstK is equal to 1, the following assignment is made:

licFlagConstK = 0		(8-48x)

    - Otherwise, the following assignment is made:

licFlagConstK = 1		(8-48x)

- The initial subblock merging candidate list, subblockMergeCandList, is constructed as follows:

…

## Disabling Affine PROF off Affine LIC is on

…

### 8.5.6 Decoding process for inter blocks

#### 8.5.6.1 General

…

    - If inter_affine_flag[ xSb ][ ySb ] is equal to 1, and licFlag[ xSb ][ ySb ] is equal to 0, prediction refinement with optical flow process specified in clause 8.5.7.7 is invoked with sbWidth, sbHeight, predSamplesLXL and motion vector difference array diffMv as inputs and refined predSamplesLXL as outputs.

…

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).