| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0532-v1 |
| --- | --- |

| Title: | Non-CE5: Simplification on clipping value calculation for adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Technology Inc. |  |  |

_____________________________

# Abstract

In VTM-5.0, clipping values of adaptive loop filter are calculated with floating-point arithmetic operations. In this proposal, the calculation of clipping values is simplified by using integer operations. Compared to VTM-5.0, the proposed methods achieve similar coding efficiency and encoding/decoding time under common test conditions.

# Introduction

In VTM-5.0 and JVET-N1001-v8, clipping values of luma and chroma components are calculated as follows

filterClips[ sfIdx ][ j ] = Round( 2( BitDepthY * ( 4 − alf_luma_clip_idx[ sfIdx ][ j ] ) / 4 ) )	(7-77)

AlfClipC[ adaptation_parameter_set_id ][ j ] = Round( 2( BitDepthC − 8 ) * 2( 8 * ( 3 − alf_chroma_clip_idx[ j ] ) / 3 ) )	(7-87)

whereas the signalled variables alf_luma_clip_idx[sfIdx][j] and alf_chroma_clip_idx[j] can be 0, 1, 2 or 3.

To calculate filterClips and AlfClipC, floating-point arithmetic operations are required for divisions and exponential functions, which are not friendly to hardware. This proposal proposes two methods to simplify it.

# Proposed methods

## Method 1: integer operations

Integer version of equation (7-77) can be used to calculated clipping values for luma component as follows

shift_luma[ sfIdx ][ j ]  =  ( BitDepthY * ( 4 − alf_luma_clip_idx[ sfIdx ][ j ] ) ) / 4

filterClips[ sfIdx ][ j ] =  1 << shift_luma[ sfIdx ][ j ]			 (7-77)

Similarly, integer version of equation (7-78) can be used to calculate clipping values for chroma components as follows

shift_chroma[ j ]  =  ( 3 * BitDepthC  - 8 * alf_chroma_clip_idx[ j ] ) / 3

AlfClipC[ adaptation_parameter_set_id ][ j ] = 1 <<  shift_chroma[ j ]		(7-78)

## Method 2: unified integer operations for luma and chroma components 1

The integer version of clipping value calculation in above section still needs operation of division-by-3, which is still not friendly to hardware. In method 1, it is proposed to remove the division-by-3 to simplify the clipping value calculation further. Then equations (7-77) and (7-78) can be simplified as follows

shiftConst[4] = {0, 3, 6, 8}

filterClips[ sfIdx ][ j ] = 1 << (BitDepthY – shiftConst[ alf_luma_clip_idx[ sfIdx ][ j ] ]) 		(7-77)

AlfClipC[ adaptation_parameter_set_id ][ j ] = 1 << (BitDepthC – shiftConst[ alf_chroma_clip_idx[ j ] ]) (7-78)

## Method 3: unified integer operations for luma and chroma components 2

An alternative shiftConst in method 2 is as below. Then equations (7-77) and (7-78) can be simplified as follows

shiftConst[4] = {0, 2, 4, 8}

filterClips[ sfIdx ][ j ] = 1 << (BitDepthY – shiftConst[ alf_luma_clip_idx[ sfIdx ][ j ] ]) 		(7-77)

AlfClipC[ adaptation_parameter_set_id ][ j ] = 1 << (BitDepthC – shiftConst[ alf_chroma_clip_idx[ j ] ]) (7-78)

# Experimental results

## Result of method 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.05% | 101% | 102% |
| Class A2 | 0.00% | -0.09% | -0.01% | 100% | 102% |
| Class B | 0.01% | 0.01% | -0.01% | 100% | 98% |
| Class C | 0.01% | -0.01% | -0.09% | 99% | 97% |
| Class E | 0.03% | -0.05% | -0.04% | 100% | 101% |
| Overall | 0.01% | -0.02% | -0.02% | 100% | 100% |
| Class D | 0.01% | -0.04% | -0.08% | 99% | 96% |
| Class F | 0.06% | 0.00% | -0.05% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.13% | 101% | 100% |
| Class A2 | 0.00% | 0.09% | 0.15% | 100% | 101% |
| Class B | 0.01% | 0.07% | -0.10% | 100% | 100% |
| Class C | 0.01% | -0.02% | -0.18% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | -0.03% | 100% | 100% |
| Class D | 0.03% | 0.16% | 0.21% | 98% | 98% |
| Class F | 0.03% | 0.03% | -0.01% | 98% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.08% | -0.13% | 99% | 96% |
| Class C | 0.00% | 0.51% | 0.27% | 99% | 98% |
| Class E | 0.04% | 0.60% | -0.24% | 99% | 100% |
| Overall | 0.00% | 0.35% | -0.02% | 99% | 98% |
| Class D | 0.02% | 0.30% | 0.36% | 99% | 99% |
| Class F | 0.03% | 0.43% | -0.26% | 99% | 101% |

## Result of method 2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.05% | 100% | 101% |
| Class A2 | -0.01% | -0.09% | -0.01% | 100% | 102% |
| Class B | 0.01% | 0.01% | -0.01% | 99% | 100% |
| Class C | 0.02% | -0.02% | -0.10% | 99% | 98% |
| Class E | 0.03% | -0.05% | -0.04% | 100% | 102% |
| Overall | 0.01% | -0.02% | -0.03% | 100% | 101% |
| Class D | 0.02% | -0.05% | -0.09% | 99% | 94% |
| Class F | 0.05% | -0.01% | -0.05% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.12% | 0.02% | 101% | 101% |
| Class A2 | 0.01% | -0.04% | 0.06% | 100% | 101% |
| Class B | 0.02% | 0.04% | -0.14% | 99% | 100% |
| Class C | -0.02% | -0.02% | -0.18% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.03% | -0.08% | 100% | 100% |
| Class D | 0.03% | -0.03% | 0.05% | 98% | 98% |
| Class F | 0.05% | 0.11% | -0.03% | 99% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.08% | 0.07% | 100% | 97% |
| Class C | 0.02% | 0.27% | 0.19% | 100% | 100% |
| Class E | 0.07% | -0.20% | -0.68% | 100% | 102% |
| Overall | 0.03% | 0.01% | -0.08% | 100% | 99% |
| Class D | 0.08% | 0.11% | 1.12% | 99% | 102% |
| Class F | -0.01% | 0.99% | 0.21% | 99% | 100% |

## Result of method 3

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.22% | 0.16% | 101% | 102% |
| Class A2 | 0.00% | 0.05% | 0.04% | 100% | 103% |
| Class B | 0.00% | 0.12% | 0.15% | 99% | 100% |
| Class C | -0.01% | 0.12% | 0.11% | 99% | 97% |
| Class E | 0.01% | -0.10% | 0.03% | 100% | 101% |
| Overall | 0.00% | 0.09% | 0.10% | 100% | 100% |
| Class D | -0.01% | 0.19% | 0.08% | 99% | 95% |
| Class F | -0.01% | 0.21% | 0.38% | 100% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.38% | 0.42% | 102% | 102% |
| Class A2 | 0.06% | 1.07% | 0.52% | 101% | 100% |
| Class B | 0.03% | 0.41% | 0.20% | 99% | 99% |
| Class C | 0.02% | 0.21% | 0.13% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.48% | 0.29% | 100% | 100% |
| Class D | 0.00% | 0.19% | 0.10% | 97% | 97% |
| Class F | 0.03% | 0.30% | 0.28% | 98% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 1.21% | 1.39% | 100% | 98% |
| Class C | 0.07% | 0.84% | 0.59% | 99% | 100% |
| Class E | -0.05% | 0.34% | 0.35% | 100% | 102% |
| Overall | 0.02% | 0.87% | 0.86% | 100% | 100% |
| Class D | 0.01% | 0.75% | 0.88% | 98% | 99% |
| Class F | 0.00% | 0.40% | 0.05% | 99% | 99% |

# Patent rights declaration(s)

Qualcomm Technology Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).