| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0534 |
| --- | --- |

| Title: | Non-CE4: Redundancy removal for merge mode signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Ye, Xiang Li, Shan Liu | Tel: Email: | {jingye, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In VVC Draft v5, when signaling the merge mode, there are some redundancies. It is proposed to remove the redundancies for signaling regular merge mode, MMVD mode, subblock based merge mode, and CIIP mode. The proposal reports 0.23% coding gains on top of VTM-5.0 for LP configuration under common test condition. The proposal also reports 0.04%, 0.1% and 0.23% coding gains over VTM-5.0 for RA, LB, and LP configurations when setting the MaxNumTriangleMergeCand to be 0 for slices with a temporal level 0 and temporal level 1.

# Introduction

In VVC draft 5 [1], there are some constraints for regular merge mode, MMVD mode, subblock based merge mode, CIIP mode, and Triangle mode. Below are the conditions for these merge modes:

allowRegularMerge = MaxNumMergeCand > 0

allowMMVD = sps_mmvd_enabled_flag

allowSubMerge = MaxNumSubblockMergeCand>0 && cbWidth>=8 && cbHeight>=8

allowCIIP = sps_ciip_enabled_flag && !cu_skip_flag && cbWidth*cbHeight >= 64 && cbWidth<128 && cbHeight<128

allowTriangle = sps_triangle_enabled_flag && slice_type == B && MaxNumTriangleMergeCand >= 2 && cbWidth * cbHeight >=64

In some cases, some of the merge modes are not allowed. In those cases, there are some redundancies for mode signaling. For example, when allowTriangle is false, regular_merge_flag is false, mmvd_flag is false, and merge_subblock_flag is false, the ciip_flag must be true. However, it is still signaled in VVC. This contribution removes these redundancies.

# Proposal

It is proposed to remove the redundancies when signaling merge mode. The proposed method is illustrated in Fig. 1.

Fig.1 Proposed signaling method for merge mode

It is first observed from Fig. 1 that only when (allowRegularMerge && (allowMMVD | | allowCIIP| | allowSubMerge | | allowTriangle) is true, regular merge mode is signaled. Secondly, only when (allowMMVD && (allowSubMerge | | allowCIIP | | allowTriangle)) is true and regular_merge_flag is flase, mmvd flag is signaled. Thirdly, only when (allowSubMerge && (allowCIIP | | allowTriangle)) is true and regular merge_flag is false, mmvd flag is false, merge_subblock_flag is signaled. Last but not least, only when (allowCIIP && allowTriangle) is true and regular_merge_flag is false, mmvd_flag is false, merge_subblock_flag is false, ciip_flag is signaled.

# Results

The proposed method has been tested with CTC defined in [2]. Table 1 show the results with CTC. The encoding and decoding time is not accurate.

Table 1: Coding performance with anchor VTM-5.0 under CTC.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.20% | 100% | 100% |  |  |  |  |  |  |  |  |  |  |
| Class A2 | -0.01% | 0.07% | 0.09% | 100% | 100% |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.05% | -0.03% | 100% | 100% | 0.01% | 0.05% | -0.06% | 100% | 101% | -0.16% | -0.18% | -0.33% | 99% | 98% |
| Class C | -0.02% | -0.09% | -0.15% | 100% | 100% | 0.01% | -0.11% | 0.09% | 100% | 103% | -0.23% | -0.37% | -0.06% | 99% | 101% |
| Class E |  |  |  |  |  | -0.01% | -0.49% | -0.45% | 100% | 100% | -0.28% | -0.01% | -0.43% | 99% | 100% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 100% | 0.00% | -0.14% | -0.11% | 100% | 101% | -0.22% | -0.20% | -0.26% | 99% | 99% |
| Class D | 0.02% | 0.02% | 0.10% | 99% | 99% | 0.03% | -0.10% | 0.69% | 99% | 98% | -0.29% | -0.97% | 0.25% | 98% | 99% |
| Class F | 0.00% | 0.05% | 0.03% | 100% | 100% | 0.01% | -0.16% | -0.33% | 99% | 98% | -0.02% | -0.31% | -0.09% | 98% | 99% |

To demonstrate the benefit of the proposed method, a non-CTC case is tested in which triangle merge mode is disabled for slices with temporal level 0 and 1 by setting MaxNumTriangleMergeCand to 0 at related slice headers. Table 6 shows the results against VTM 5 under this condition. The encoding and decoding time is not accurate.

Table 6: Coding performance with anchor VTM-5.0 for slices with temporal level 0 and temporal level 1, the MaxNumTriangleMergeCand is 0.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.06% | -0.03% | 100% | 98% |  |  |  |  |  |  |  |  |  |  |
| Class A2 | -0.04% | -0.04% | 0.01% | 100% | 99% |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.05% | -0.01% | 0.03% | 100% | 100% | -0.08% | -0.13% | -0.11% | 99% | 100% | -0.18% | -0.14% | -0.55% | 102% | 98% |
| Class C | -0.05% | 0.01% | -0.01% | 100% | 101% | -0.10% | -0.27% | 0.04% | 100% | 99% | -0.22% | -0.32% | -0.05% | 100% | 99% |
| Class E |  |  |  |  |  | -0.12% | 0.16% | -0.92% | 100% | 99% | -0.32% | 0.25% | -0.89% | 100% | 100% |
| Overall | -0.04% | -0.02% | 0.00% | 100% | 100% | -0.10% | -0.11% | -0.26% | 100% | 99% | -0.23% | -0.10% | -0.47% | 101% | 99% |
| Class D | 0.01% | -0.05% | 0.06% | 100% | 102% | -0.09% | -0.20% | -0.04% | 100% | 101% | -0.30% | -0.40% | -0.49% | 99% | 94% |
| Class F | -0.03% | -0.01% | -0.03% | 100% | 101% | -0.07% | -0.19% | -0.56% | 100% | 98% | 0.00% | 0.15% | -0.31% | 100% | 97% |

# Spec

#### Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag ¦ ¦ cbWidth * cbHeight != 32 allowRegularMerge&&(allowMMVD ¦ ¦ allowSubMerge ¦ ¦ allowCIIP ¦ ¦ allowTriangle ) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag && cbWidth * cbHeight != 32 allowMMVD &&( allowSubMerge ¦ ¦ allowCIIP ¦ ¦ allowTriangle) ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 allowSubMerge && (allowCIIP ¦ ¦ allowTriangle) ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 allowCIIP && allowTriangle ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### Definition of related variables

allowRegularMerge = MaxNumMergeCand > 0

allowMMVD = sps_mmvd_enabled_flag

allowSubMerge = MaxNumSubblockMergeCand>0 && cbWidth>=8 && cbHeight>=8

allowCIIP = sps_ciip_enabled_flag && !cu_skip_flag && cbWidth*cbHeight >= 64 && cbWidth<128 && cbHeight<128

allowTriangle = sps_triangle_enabled_flag && slice_type == B && MaxNumTriangleMergeCand >= 2 && cbWidth * cbHeight >=64

Semantics

When regular_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, regular_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- allowRegularMerge is true.

- Otherwise, regular_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

When mmvd_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- allowMMVD is true.

- Otherwise, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

When merge_subblock_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, merge_subblock_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- regular_merge_flag[x0][y0] is equal to 0.

- mmvd_merge_flag[x0][y0] is equal to 0.

- allowSubMerge is true.

- Otherwise, merge_subblock_flag[ x0 ][ y0 ] is inferred to be equal to 0.

When ciip_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- allowCIIP is true.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- regular_merge_flag[x0][y0] is equal to 0.

- mmvd_merge_flag[x0][y0] is equal to 0.

- Merge_subblock_flag[x0][y0] is equal to 0.

- Otherwise, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 0.

# Conclusion

This contribution proposed redundancy removal of merge signaling. The proposed method reduced the signaling overhead during merge signaling and saved the BD-rate. The proposal reports 0.22% coding gains on top of VTM5 for LP configurations under common test condition. It is also observed that the proposed method shows benefits at slice level. such as when temporal level 0 slices and temporal level 1 slices with the setting that MaxNumTriangleCand is 0. It is recommended to include the proposed method into the VVC.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, ISO/IEC JTC1/SC29/WG11 JVET-N1001, Geneva, CH, 19–27, March 2019

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).