| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0535-v2 |
| --- | --- |

| Title: | Non-CE3: Unification of PDPC for diagonal intra prediction modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to unify the PDPC process for diagonal intra prediction modes and their adjacent intra prediction modes such that less branches in PDPC process are kept. It is reported that both proposed methods obtain 0.00% luma BD-rate change for AI configuration on top of VTM-5.0.

# Introduction

In VTM-5, PDPC is a linear combination of the prediction samples and the neighboring reconstructed samples using position-dependent weightings and the results are used to replace the original prediction samples. According to VVC Draft v5, different PDPC processes are applied to different intra prediction modes, including: planar, DC, horizontal, vertical, diagonal modes and their adjacent angular modes.

# Proposed method

In this contribution, for diagonal intra prediction modes, it is proposed to use the same PDPC process applied for the intra prediction modes adjacent to diagonal intra prediction modes such that less processing branches are kept.

Two alternatives are proposed. In the first method, two branches for diagonal modes and their adjacent modes are directly merged. In the second method, in addition to the changes in the first method, operations for deriving the positions of refT and refL are further reduced. Both methods simplify the VTM-5.0 sw and VVC spec text, as reported below in Section 4.

Additional two methods, i.e., method 3 and 4, are proposed for further unification of PDPC. In method 3, on top of method 2, the per-sample checking of reference sample availability is replaced by a per-row or per-column checking. In method 4, on top of method 3, PDPC is applied to all intra prediction modes beyond horizontal and vertical modes, i.e., intra mode index greater than 50 or less than 18.

# Test results

The proposed method has been implemented on top of VTM5.0 software [1]. Simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-N1010 [2], which are reported in below Table 1.

Table 1: Results of proposed method #1 over VTM-5.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | 0.00% | 100% | 100% | 0.01% | -0.02% | 0.06% | 100% | 100% |
| Class A2 | 0.00% | -0.02% | -0.01% | 100% | 100% | 0.04% | 0.11% | 0.06% | 100% | 100% |
| Class B | 0.01% | -0.04% | 0.01% | 100% | 100% | 0.00% | 0.04% | -0.15% | 100% | 100% |
| Class C | 0.01% | -0.06% | 0.02% | 100% | 100% | -0.03% | -0.20% | -0.07% | 100% | 101% |
| Class E | 0.02% | -0.05% | 0.06% | 100% | 100% |  |  |  |  |  |
| Overall | 0.00% | -0.03% | 0.02% | 100% | 100% | 0.00% | -0.02% | -0.04% | 100% | 100% |
| Class D | 0.02% | 0.11% | -0.11% | 99% | 101% | 0.02% | 0.38% | -0.18% | 100% | 101% |
| Class F | 0.02% | 0.08% | 0.07% | 100% | 101% | 0.02% | -0.11% | 0.21% | 100% | 102% |

Table 1: Results of proposed method #2 over VTM-5.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.03% | 100% | 99% | 0.00% | -0.13% | 0.15% | 100% | 100% |
| Class A2 | 0.00% | -0.06% | -0.05% | 100% | 99% | 0.03% | 0.08% | 0.14% | 100% | 100% |
| Class B | 0.01% | -0.05% | 0.03% | 100% | 98% | 0.01% | 0.11% | 0.11% | 100% | 98% |
| Class C | 0.01% | -0.08% | 0.08% | 101% | 100% | -0.01% | -0.09% | -0.03% | 102% | 105% |
| Class E | 0.00% | -0.05% | -0.05% | 101% | 99% |  |  |  |  |  |
| Overall | 0.00% | -0.05% | 0.00% | 100% | 99% | 0.01% | 0.00% | 0.09% | 101% | 100% |
| Class D | 0.01% | 0.19% | 0.00% | 102% | 109% | 0.02% | 0.22% | 0.40% | 102% | 106% |
| Class F | 0.01% | 0.10% | -0.06% | 101% | 100% | -0.04% | -0.05% | 0.12% | 102% | 104% |

Table 1: Results of proposed method #3 over VTM-5.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.03% | 100% | 101% | 0.01% | 0.00% | 0.09% | 100% | 100% |
| Class A2 | 0.00% | -0.03% | -0.02% | 100% | 98% | 0.02% | 0.02% | -0.01% | 100% | 100% |
| Class B | 0.01% | 0.00% | 0.06% | 100% | 97% | 0.02% | 0.06% | -0.04% | 100% | 98% |
| Class C | 0.00% | 0.01% | 0.09% | 100% | 101% | -0.02% | -0.04% | -0.17% | 101% | 104% |
| Class E | 0.02% | -0.03% | 0.04% | 101% | 100% |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.05% | 100% | 99% | 0.01% | 0.02% | -0.04% | 101% | 100% |
| Class D | 0.00% | 0.17% | 0.06% | 102% | 107% | 0.04% | -0.21% | 0.16% | 102% | 105% |
| Class F | 0.00% | 0.15% | 0.01% | 100% | 99% | 0.02% | 0.01% | 0.11% | 102% | 104% |

Table 1: Results of proposed method #4 over VTM-5.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.03% | 0.05% | 100% | 100% | -0.01% | -0.07% | 0.11% | 100% | 100% |
| Class A2 | -0.05% | 0.02% | -0.02% | 99% | 98% | 0.02% | 0.22% | 0.08% | 100% | 99% |
| Class B | -0.03% | -0.05% | 0.09% | 99% | 98% | 0.00% | 0.18% | 0.11% | 100% | 97% |
| Class C | 0.01% | 0.01% | 0.11% | 101% | 100% | -0.01% | 0.03% | -0.03% | 102% | 103% |
| Class E | -0.04% | 0.06% | 0.08% | 101% | 99% |  |  |  |  |  |
| Overall | -0.03% | 0.01% | 0.07% | 100% | 99% | 0.00% | 0.10% | 0.07% | 100% | 100% |
| Class D | 0.02% | 0.18% | -0.01% | 102% | 106% | 0.02% | 0.27% | 0.04% | 102% | 105% |
| Class F | 0.03% | 0.27% | 0.06% | 101% | 98% | -0.01% | -0.13% | -0.03% | 102% | 103% |

# Spec text changes

The following text describes the proposed method. It is based on VVC draft5 v7 [3] clause 8.4.5.2.13. Modifications related to this proposal are highlighted in yellow.

## Spec text changes of method #1

8.4.5.2.14 Position-dependent intra prediction combination process

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1 and y =0..nTbH − 1,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the modified predicted samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y =0..nTbH − 1.

Depending on the value of cIdx, the function clip1Cmp is set as follows:

- If cIdx is equal to 0, clip1Cmp is set equal to Clip1Y.

- Otherwise, clip1Cmp is set equal to Clip1C.

The variable nScale is set to ( ( Log2( nTbW ) + Log2( nTbH ) − 2 )  >>  2 ).

The reference sample arrays mainRef[ x ] and sideRef[ y ], with x = 0..refW and y = 0..refH are derived as follows:

mainRef[ x ] = p[ x ][ −1 ] 	 	 	 	 	 	 (8-211)
sideRef[ y ] = p[ −1 ][ y ]

The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ y ] and wTL[ x ][ y ] with x = 0..nTbW − 1, y =0..nTbH − 1 are derived as follows:

- If predModeIntra is equal to INTRA_PLANAR or INTRA_DC, the following applies:

refL[ x ][ y ] = p[ −1 ][ y ] 		 	 		 	(8-212)

refT[ x ][ y ] = p[ x ][ −1 ] 		 	 		 	(8-213)

wT[ y ] = 32  >>  ( ( y  <<  1 )  >>  nScale )			 (8-214)

wL[ x ] = 32  >>  ( ( x  <<  1 )  >>  nScale )			 (8-215)

wTL[ x ][ y ] = ( predModeIntra  = =  INTRA_DC )  ?  ( ( wL[ x ]  >>  4 ) + ( wT[ y ]  >>  4 ) )  :  0  														 (8-216)

- Otherwise, if predModeIntra is equal to INTRA_ANGULAR18 or INTRA_ANGULAR50, the following applies:

refL[ x ][ y ] = p[ −1 ][ y ]						(8-217)

refT[ x ][ y ] = p[ x ][ −1 ]						(8-218)

wT[ y ] = ( predModeIntra  = = INTRA_ANGULAR18 ) ? 32  >>  ( ( y  <<  1 )  >>  nScale ) : 0														(8-219)

wL[ x ] = ( predModeIntra  = = INTRA_ANGULAR50 ) ? 32  >>  ( ( x  <<  1 )  >>  nScale ) : 0							 		 		 		 	(8-220)

wTL[ x ][ y ] = ( predModeIntra  = = INTRA_ANGULAR18 ) ?  wT[ y ] : wL[ x ] 	(8-221)

- Otherwise, if predModeIntra is equal to INTRA_ANGULAR2 or INTRA_ANGULAR66, the following applies:

refL[ x ][ y ] = p[ −1 ][ x + y + 1 ] 				(8-222)

refT[ x ][ y ] = p[ x + y + 1 ][ −1 ] 				(8-223)

wT[ y ] = ( 32  >>  1 )  >>  ( ( y  <<  1 )  >>  nScale )	(8-224)

wL[ x ] = ( 32  >>  1 )  >>  ( ( x  <<  1 )  >>  nScale )	(8-225)

wTL[ x ][ y ] = 0								(8-226)

- Otherwise, if predModeIntra is less than or equal to INTRA_ANGULAR10, the following ordered steps apply:

- The variables dXPos[ y ], dXFrac[ y ], dXInt[ y ] and dX[ y ] are derived as follows using invAngle as specified in clause 8.2.4.2.7 depending on intraPredMode:

dXPos[ y ] =	( ( y + 1 ) * invAngle + 2 )  >>  2
dXFrac[ y ] =	dXPos[ y ] & 63					(8-227)
dXInt[ y ] = dXPos [ y ]   >>  6
dX[ x ][ y ] =	x + dXInt[ y ]

- The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ y ] and wTL[ x ][ y ] are derived as follows:

refL[ x ][ y ] = 0					 			 	(8-228)

refT[ x ][ y ] = ( dX[ x ][ y ] < refW − 1 )  ?  mainRef[ dX[ x ][ y ] + ( dXFrac[ y ]  >>  5 ) ]  :  0                                      														 (8-229)

wT[ y ] = ( dX[ y ] < refW − 1 )  ?  32  >>  ( ( y  <<  1 )  >>  nScale )  :  0	(8-230)

wL[ x ] = 0					 			 		(8-231)

wTL[ x ][ y ] = 0					 			 (8-232)

- Otherwise, if predModeIntra is greater than or equal to INTRA_ANGULAR58, the following ordered steps apply:

- The variables dYPos[ x ], dYFrac[ x ], dYInt[ x ] and dY[ x ] are derived as follows using invAngle as specified in clause 8.2.4.2.7 depending on intraPredMode:

dYPos[ x ] =	( ( x + 1 ) * invAngle + 2 )  >>  2
dYFrac[ x ] =	dYPos[ x ] & 63					 (8-233)
dYInt[ x ] =	dYPos[ x ]  >>  6
dY[ x ][ y ] =	x + dYInt[ x ]

- The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ y ] and wTL[ x ][ y ] are derived as follows:

refL[ x ][ y ] = ( dY[ x ][ y ] < refH − 1 )  ?  sideRef[ dY[ x ][ y ] + ( dYFrac[ x ]  >>  5 ) ]  :  0                                 				 			 			 			 	(8-234)

refT[ x ][ y ] = 0									(8-235)

wT[ y ] = 0					 			 		(8-236)

wL[ x ] = ( dY[ x ][ y ] < refH − 1 )  ?  32  >>  ( ( x  <<  1 )  >>  nScale )  :  0	(8-237)

wTL[ x ][ y ] = 0					 			 (8-238)

- Otherwise, refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ y ] and wTL[ x ][ y ] are all set equal to 0.

The values of the filtered samples filtSamples[ x ][ y ], with x = 0..nTbW − 1, y =0..nTbH − 1 are derived as follows:

filtSamples[ x ][ y ] = clip1Cmp( (  refL[ x ][ y ] * wL + refT[ x ][ y ] * wT + p[ −1 ][ −1 ] * wTL[ x ][ y ] + ( 64 − wL[ x ] − wT[ y ] + wTL[ x ][ y ] ) * predSamples[ x ][ y ] + 32  )	 >>  6 ) 	 	 	 			 			 			 		(8-239)

## Spec text changes of method #2

8.4.5.2.14 Position-dependent intra prediction combination process

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1 and y =0..nTbH − 1,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the modified predicted samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y =0..nTbH − 1.

Depending on the value of cIdx, the function clip1Cmp is set as follows:

- If cIdx is equal to 0, clip1Cmp is set equal to Clip1Y.

- Otherwise, clip1Cmp is set equal to Clip1C.

The variable nScale is set to ( ( Log2( nTbW ) + Log2( nTbH ) − 2 )  >>  2 ).

The reference sample arrays mainRef[ x ] and sideRef[ y ], with x = 0..refW and y = 0..refH are derived as follows:

mainRef[ x ] = p[ x ][ −1 ]				 		 (8-211)
sideRef[ y ] = p[ −1 ][ y ]

The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ y ] and wTL[ x ][ y ] with x = 0..nTbW − 1, y =0..nTbH − 1 are derived as follows:

- If predModeIntra is equal to INTRA_PLANAR or INTRA_DC, the following applies:

refL[ x ][ y ] = p[ −1 ][ y ] 						(8-212)

refT[ x ][ y ] = p[ x ][ −1 ] 						(8-213)

wT[ y ] = 32  >>  ( ( y  <<  1 )  >>  nScale )			(8-214)

wL[ x ] = 32  >>  ( ( x  <<  1 )  >>  nScale )			(8-215)

wTL[ x ][ y ] = ( predModeIntra  = =  INTRA_DC )  ?  ( ( wL[ x ]  >>  4 ) + ( wT[ y ]  >>  4 ) )  :  0  		 		 		 		 		 		 	(8-216)

- Otherwise, if predModeIntra is equal to INTRA_ANGULAR18 or INTRA_ANGULAR50, the following applies:

refL[ x ][ y ] = p[ −1 ][ y ]				 		 (8-217)

refT[ x ][ y ] = p[ x ][ −1 ]				 		 (8-218)

wT[ y ] = ( predModeIntra  = = INTRA_ANGULAR18 ) ? 32  >>  ( ( y  <<  1 )  >>  nScale ) : 0							 		 		 		 	(8-219)

wL[ x ] = ( predModeIntra  = = INTRA_ANGULAR50 ) ? 32  >>  ( ( x  <<  1 )  >>  nScale ) : 0							 		 		 		 	(8-220)

wTL[ x ][ y ] = ( predModeIntra  = = INTRA_ANGULAR18 ) ?  wT[ y ] : wL[ x ] 	(8-221)

- Otherwise, if predModeIntra is equal to INTRA_ANGULAR2 or INTRA_ANGULAR66, the following applies:

refL[ x ][ y ] = p[ −1 ][ x + y + 1 ] 				(8-222)

refT[ x ][ y ] = p[ x + y + 1 ][ −1 ] 				(8-223)

wT[ y ] = ( 32  >>  1 )  >>  ( ( y  <<  1 )  >>  nScale )	(8-224)

wL[ x ] = ( 32  >>  1 )  >>  ( ( x  <<  1 )  >>  nScale )	(8-225)

wTL[ x ][ y ] = 0								(8-226)

- Otherwise, if predModeIntra is less than or equal to INTRA_ANGULAR10, the following ordered steps apply:

- The variables dXPos[ y ], dXFrac[ y ], dXInt[ y ] and dX[ y ] are derived as follows using invAngle as specified in clause 8.2.4.2.7 depending on intraPredMode:

dXInt[ y ] dXPos[ y ] =	( ( y + 1 ) * invAngle + 2128 )  >>  28
dXFrac[ y ] =	dXPos[ y ] & 63				 	(8-227)
dXInt[ y ] = dXPos [ y ]   >>  6
dX[ x ][ y ] =	x + dXInt[ y ]

- The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ y ] and wTL[ x ][ y ] are derived as follows:

refL[ x ][ y ] = 0									(8-228)

refT[ x ][ y ] = ( dX[ x ][ y ] < refW − 1 )  ?  mainRef[ dX[ x ][ y ] + ( dXFrac[ y ]  >>  5 ) ]  :  0                                      														 (8-229)

wT[ y ] = ( dX[ y ] < refW − 1 )  ?  32  >>  ( ( y  <<  1 )  >>  nScale )  :  0	(8-230)

wL[ x ] = 0				 		 		 		 (8-231)

wTL[ x ][ y ] = 0								(8-232)

- Otherwise, if predModeIntra is greater than or equal to INTRA_ANGULAR58, the following ordered steps apply:

- The variables dYPos[ x ], dYFrac[ x ], dYInt[ x ] and dY[ x ] are derived as follows using invAngle as specified in clause 8.2.4.2.7 depending on intraPredMode:

dYInt[ x ] dYPos[ x ] =	( ( x + 1 ) * invAngle + 2128 )  >>  28
dYFrac[ x ] =	dYPos[ x ] & 63					(8-233)
dYInt[ x ] =	dYPos[ x ]  >>  6
dY[ x ][ y ] =	x + dYInt[ x ]

- The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ y ] and wTL[ x ][ y ] are derived as follows:

refL[ x ][ y ] = ( dY[ x ][ y ] < refH − 1 )  ?  sideRef[ dY[ x ][ y ] + ( dYFrac[ x ]  >>  5 ) ]  :  0                                 			 		 		 		 		 		 	(8-234)

refT[ x ][ y ] = 0				 		 		 	(8-235)

wT[ y ] = 0				 		 		 		 (8-236)

wL[ x ] = ( dY[ x ][ y ] < refH − 1 )  ?  32  >>  ( ( x  <<  1 )  >>  nScale )  :  0	(8-237)

wTL[ x ][ y ] = 0				 		 		 (8-238)

- Otherwise, refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ y ] and wTL[ x ][ y ] are all set equal to 0.

The values of the filtered samples filtSamples[ x ][ y ], with x = 0..nTbW − 1, y =0..nTbH − 1 are derived as follows:

filtSamples[ x ][ y ] = clip1Cmp( (  refL[ x ][ y ] * wL + refT[ x ][ y ] * wT + p[ −1 ][ −1 ] * wTL[ x ][ y ] + ( 64 − wL[ x ] − wT[ y ] + wTL[ x ][ y ] ) * predSamples[ x ][ y ] + 32  )	 >>  6 ) 	 	 												 (8-239)

## Spec text changes of method #3

8.4.5.2.14 Position-dependent intra prediction combination process

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1 and y =0..nTbH − 1,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the modified predicted samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y =0..nTbH − 1.

Depending on the value of cIdx, the function clip1Cmp is set as follows:

- If cIdx is equal to 0, clip1Cmp is set equal to Clip1Y.

- Otherwise, clip1Cmp is set equal to Clip1C.

The variable nScale is set to ( ( Log2( nTbW ) + Log2( nTbH ) − 2 )  >>  2 ).

The reference sample arrays mainRef[ x ] and sideRef[ y ], with x = 0..refW and y = 0..refH are derived as follows:

mainRef[ x ] = p[ x ][ −1 ]				 		 (8-211)
sideRef[ y ] = p[ −1 ][ y ]

The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ y ] and wTL[ x ][ y ] with x = 0..nTbW − 1, y =0..nTbH − 1 are derived as follows:

- If predModeIntra is equal to INTRA_PLANAR or INTRA_DC, the following applies:

refL[ x ][ y ] = p[ −1 ][ y ] 						(8-212)

refT[ x ][ y ] = p[ x ][ −1 ] 						(8-213)

wT[ y ] = 32  >>  ( ( y  <<  1 )  >>  nScale )			(8-214)

wL[ x ] = 32  >>  ( ( x  <<  1 )  >>  nScale )			(8-215)

wTL[ x ][ y ] = ( predModeIntra  = =  INTRA_DC )  ?  ( ( wL[ x ]  >>  4 ) + ( wT[ y ]  >>  4 ) )  :  0  		 		 		 		 		 		 	(8-216)

- Otherwise, if predModeIntra is equal to INTRA_ANGULAR18 or INTRA_ANGULAR50, the following applies:

refL[ x ][ y ] = p[ −1 ][ y ]				 		 (8-217)

refT[ x ][ y ] = p[ x ][ −1 ]				 		 (8-218)

wT[ y ] = ( predModeIntra  = = INTRA_ANGULAR18 ) ? 32  >>  ( ( y  <<  1 )  >>  nScale ) : 0							 		 		 		 	(8-219)

wL[ x ] = ( predModeIntra  = = INTRA_ANGULAR50 ) ? 32  >>  ( ( x  <<  1 )  >>  nScale ) : 0							 		 		 		 	(8-220)

wTL[ x ][ y ] = ( predModeIntra  = = INTRA_ANGULAR18 ) ?  wT[ y ] : wL[ x ] 	(8-221)

- Otherwise, if predModeIntra is equal to INTRA_ANGULAR2 or INTRA_ANGULAR66, the following applies:

refL[ x ][ y ] = p[ −1 ][ x + y + 1 ] 				(8-222)

refT[ x ][ y ] = p[ x + y + 1 ][ −1 ] 				(8-223)

wT[ y ] = ( 32  >>  1 )  >>  ( ( y  <<  1 )  >>  nScale )	(8-224)

wL[ x ] = ( 32  >>  1 )  >>  ( ( x  <<  1 )  >>  nScale )	(8-225)

wTL[ x ][ y ] = 0								(8-226)

- Otherwise, if predModeIntra is less than or equal to INTRA_ANGULAR10, the following ordered steps apply:

- The variables dXPos[ y ], dXFrac[ y ], dXInt[ y ] and dX[ y ] are derived as follows using invAngle as specified in clause 8.2.4.2.7 depending on intraPredMode:

dXInt[ y ] dXPos[ y ] =	( ( y + 1 ) * invAngle + 2128 )  >>  28
dXFrac[ y ] =	dXPos[ y ] & 63				 	(8-227)
dXInt[ y ] = dXPos [ y ]   >>  6
dX[ x ][ y ] =	x + dXInt[ y ]

- The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ y ] and wTL[ x ][ y ] are derived as follows:

refL[ x ][ y ] = 0									(8-228)

refT[ x ][ y ] = ( dX[ x ][ y ] dXInt[ y ] < refW – width − 1 )  ?  mainRef[ dX[ x ][ y ] + ( dXFrac[ y ]  >>  5 ) ]  :  0                                      														 (8-229)

wT[ y ] = ( dX[ y ] < refW − 1 )  ?  32  >>  ( ( y  <<  1 )  >>  nScale )  :  0	(8-230)

wL[ x ] = 0				 		 		 		 (8-231)

wTL[ x ][ y ] = 0								(8-232)

- Otherwise, if predModeIntra is greater than or equal to INTRA_ANGULAR58, the following ordered steps apply:

- The variables dYPos[ x ], dYFrac[ x ], dYInt[ x ] and dY[ x ] are derived as follows using invAngle as specified in clause 8.2.4.2.7 depending on intraPredMode:

dYInt[ x ] dYPos[ x ] =	( ( x + 1 ) * invAngle + 2128 )  >>  28
dYFrac[ x ] =	dYPos[ x ] & 63					(8-233)
dYInt[ x ] =	dYPos[ x ]  >>  6
dY[ x ][ y ] =	x + dYInt[ x ]

- The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ y ] and wTL[ x ][ y ] are derived as follows:

refL[ x ][ y ] = ( dY[ x ][ y ] dYInt[ x ] < refH – height − 1 )  ?  sideRef[ dY[ x ][ y ] + ( dYFrac[ x ]  >>  5 ) ]  :  0                                 			 		 		 		 		 		 	(8-234)

refT[ x ][ y ] = 0				 		 		 	(8-235)

wT[ y ] = 0				 		 		 		 (8-236)

wL[ x ] = ( dY[ x ] < refH − 1 )  ?  32  >>  ( ( x  <<  1 )  >>  nScale )  :  0	(8-237)

wTL[ x ][ y ] = 0				 		 		 (8-238)

- Otherwise, refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ y ] and wTL[ x ][ y ] are all set equal to 0.

The values of the filtered samples filtSamples[ x ][ y ], with x = 0..nTbW − 1, y =0..nTbH − 1 are derived as follows:

filtSamples[ x ][ y ] = clip1Cmp( (  refL[ x ][ y ] * wL + refT[ x ][ y ] * wT + p[ −1 ][ −1 ] * wTL[ x ][ y ] + ( 64 − wL[ x ] − wT[ y ] + wTL[ x ][ y ] ) * predSamples[ x ][ y ] + 32  )	 >>  6 ) 	 	 												 (8-239)

# Conclusion

In this contribution, it is proposed to align the PDPC process for diagonal intra prediction modes and their adjacent intra prediction modes. It is reported that both of the proposed methods obtain 0.00% luma BD-rate change for AI configuration on top of VTM-5.0. It is proposed to adopt this proposal.

# References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 March 2019.

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 2),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).