| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0536 |
| --- | --- |

| Title: | CE3-related: Redundant bit saving for CE3-2.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, it is reported that chroma intra prediction mode is redundant in CE3-2.2 when all the neighboring reference samples of the small chroma block (2x2, 2x4, or 4x2 block) in one parallel processable region (PPR) are marked as unavailable since all chroma prediction modes will result in the same prediction samples in this case. Therefore, it is proposed to skip the signaling of chroma prediction mode in such cases (5 lines of code changes). It is reported that the proposed method obtains 0.05% luma BD-rate change for the RA configuration on top of VTM-5.0 compared to 0.07% luma BD-rate change in CE3-2.2.

# Introduction

In CE3-2.2, when a chroma block of size larger than or equal to threshold chroma samples has any children blocks of size less than threshold or is not further split, the chroma block of size larger than or equal to threshold is treated as a parallel-processable region (PPR) [1]. Threshold is set equal to 16 to achieve a worst-case processing throughput of 4x that of VTM5.0. The shape of some examples of PPR is depicted in Figure 1, wherein the marked value in each block indicates the number of samples in that block, for example, Fig. 4a shows a 4x4 chroma block (16 samples) is split into four 2x2 sub-blocks, and each sub-block has 4 samples, and this 4x4 chroma block is considered as one parallel-processable region.

Figure 1. PPR examples with the minimum number of chroma samples of 16

# Proposed method

With the method proposed in CE3-2.2, for some cases (e.g., shaded blocks in Figure 2), all the neighboring reference samples are marked as unavailable. In this case, all the chroma intra prediction modes generate the same results, which means the chroma intra prediction mode signaling for these blocks is redundant.

To save the redundant bit, on top of CE3-2.2, it is proposed not to signal the chroma prediction mode when all the neighboring reference samples of the small chroma block (2x2, 2x4, or 4x2 block) in one parallel processable region (PPR) are marked as unavailable. For example, for a 4x4 block which is split using three different partitioning patterns, the bottom-right 2x2 block has only 4 samples (shaded one), its above and left neighboring samples may be marked as “unavailable” and not used for prediction of current 2x2 block.

Figure 2. Examples of small chroma blocks with unavailable neighboring samples in one PPR

# Test results

The proposed method in this contribution is implemented on top of the CE3-2.2 SW release based on VTM5.0 software [2]. The simulations are performed on Linux cluster using RA configurations under the common test condition described in JVET-N1010 [3]. The results of the proposed method and CE3-2.2 are listed below in Table 1 and Table 2.

Table 1: Results of proposed method over VTM-5.0

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.13% | 0.09% | 101% | 101% |
| Class A2 | 0.02% | 0.09% | 0.12% | 101% | 100% |
| Class B | 0.04% | 0.03% | 0.04% | 100% | 100% |
| Class C | 0.11% | 0.15% | 0.12% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.04% | 0.09% | 100% | 100% |
| Class D | 0.20% | 0.62% | 0.59% | 100% | 100% |
| Class F | 0.08% | 0.19% | 0.21% | 100% | 100% |

Table 2: Results of CE3-2.2 over VTM-5.0

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.13% | 0.11% | 100% | 101% |
| Class A2 | 0.03% | 0.08% | 0.11% | 100% | 100% |
| Class B | 0.05% | 0.02% | 0.01% | 100% | 100% |
| Class C | 0.14% | 0.10% | 0.17% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.02% | 0.09% | 100% | 101% |
| Class D | 0.21% | 0.62% | 0.63% | 100% | 101% |
| Class F | 0.10% | 0.26% | 0.20% | 100% | 102% |

# Conclusion

In this contribution, redundant bits for small chroma blocks in CE3-2.2 are saved. It is reported that the proposed method obtains 0.05% luma BD-rate change in RA configurations on top of VTM-5.0, while CE3-2.2 reported 0.07% luma BD-rate change comparing to VTM-5.0. It is proposed to adopt this proposal.

# References

- L. P. Van, G. V. der Auwera, A. K. Ramasubramonian, V. Seregin, H. Huang, and M. Karczewicz “CE3-related: Enabling parallel reconstruction of small intra-coded chroma blocks,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0396, Geneva, CH, 19–27 March 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).