| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0537 |
| --- | --- |

| Title: | Non-CE: Weighted intra and inter prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, JVET-N0395 is further investigated on top of VTM-5 with modifications. A new mode, namely Weighted Intra and Inter Prediction (WIIP) mode, is signaled together with CIIP mode. Two alternatives are proposed in this proposal. The first one is the same as JVET-N0395, wherein PDPC filtering process is applied to the inter prediction samples indicated by merge index. In the second one, the prediction samples in WIIP mode is calculated as a weighted average of intra prediction and inter prediction sample values, which is similar to CIIP mode. In addition, implicit transform scheme is applied to the prediction residues for WIIP mode in both methods. It is reported that the first method achieves -0.13%, -0.37%, and -0.45% luma BD-rates for RA, LDB, and LDP configuration with 101%, 102% and 103% encoding and 99%, 100% and 99% decoding time respectively, and the second method achieves -0.13%, -0.34%, and -0.44% luma BD-rates for RA, LDB, and LDP configuration with 101%, 102% and 103% encoding and 100%, 100%, and 100% decoding time respectively. It is also reported that when further applying CIIP and WIIP modes to 4x8 and 8x4 blocks, it achieves -0.19%, -0.39%, and -0.58% luma BD-rates for RA, LDB, and LDP configuration with 101%, 102% and 103% encoding and 100%, 100% and 100% decoding time respectively.

Introduction

In VTM-5.0 [1], combined intra-inter prediction (CIIP) mode combines intra prediction and merge indexed inter prediction. For intra prediction in CIIP mode, Planar mode is used to generate the intra prediction values for luma component and DM is used to generate the intra prediction values for chroma component. After intra prediction, PDPC process is applied to filter the intra prediction sample values for both luma and chroma component. The final prediction sample values of CIIP mode is a weighted average of intra and inter prediction sample values, wherein the weight for intra and inter predicted samples are adaptively selected based on the number of neighbouring intra-coded blocks [2]. The flowchart of CIIP mode is shown in Figure 1.

Figure 1: Flowchart of CIIP mode

Proposed methods

A new mode, namely Weighted Intra and Inter Prediction (WIIP) mode, is signaled together with CIIP mode. When CIIP flag is true, another flag, namely WIIP flag, is further signaled to indicate whether to use WIIP mode or not. When WIIP flag is true, WIIP mode is used. Otherwise (when WIIP flag is false), original CIIP mode in VTM-5.0 is used. In addition, same block size restrictions of CIIP mode are also applied to WIIP mode. Two different alternatives are proposed.

Method #1: The first alternative method is the same as that in JVET-N0395, PDPC filtering process is directly applied to inter prediction samples to generate the final prediction sample values, the flowchart of the first method is shown in Figure 2.

Figure 2: flowchart of the first alternative method for WIIP mode

Method #2: In the second alternative method, PDPC process does not depend on inter prediction data anymore. It is based on the neighboring reconstructed samples, the same as CIIP. Then the final prediction values of WIIP mode are calculated as a weighted average of PDPC prediction values and inter prediction values. To be specific, PDPC process in WIIP mode is shown below:

predPdpc(x,y) = ( WL × R-1,y + WT × Rx,-1 + 32 )  >>  6                  (1)

wherein WL and WT represent the weighting factors, which are the same as PDPC process, and R-1,y and Rx,-1 represent the left and above neighboring reconstructed samples. After that, the inter prediction values (predInter) and PDPC prediction (predPdpc) are combined to generate the final prediction values, as shown follows:

FinalPred(x,y) = ((predPdpc(x,y) << 6) + (64 – WL – WT) × predInter(x,y) + 32) )  >>  6  (2)

FinalPred(x,y) = (WL × R-1,y + WT × Rx,-1 + (64 – WL – WT) × predInter(x,y) + 32) )  >>  6

The flowchart of the second method is shown in Figure 3.

Figure 3: flowchart of the second method for WIIP mode

In addition, the same implicit transform scheme used in VTM-5.0 is applied for both proposed methods. To be specific, the implicit transform scheme used for WIIP mode is shown below:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? DST-7 : DCT-2                 (3)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? DST-7 : DCT-2                  (4)

Experimental results

The proposed methods are implemented on top of VTM-5.0 software [3]. The simulations are performed on Linux cluster using RA, LDB, and LDP configurations under the common test condition described in JVET-N1010 [4]. Three tests are conducted to evaluate the performance of the proposed methods, which are listed below:

- Test 1: Method #1

- Test 2: Method #2

- Test 3: Method #1 + further apply CIIP and WIIP to 8x4 and 4x8 blocks

Table 1: Results of Test 1 over VTM-5.0

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.01% | 0.27% | 101% | 100% |
| Class A2 | -0.12% | 0.17% | 0.22% | 101% | 100% |
| Class B | -0.12% | 0.11% | -0.02% | 100% | 98% |
| Class C | -0.18% | -0.15% | -0.16% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | 0.03% | 0.05% | 101% | 99% |
| Class D | -0.09% | 0.00% | 0.03% | 101% | 102% |
| Class F | -0.08% | -0.01% | 0.00% | 101% | 101% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.45% | -0.24% | -0.38% | 103% | 101% |
| Class C | -0.36% | -0.39% | -0.47% | 101% | 100% |
| Class E | -0.24% | 0.12% | -0.67% | 101% | 98% |
| Overall | -0.37% | -0.20% | -0.48% | 102% | 100% |
| Class D | -0.30% | -0.04% | -0.26% | 101% | 97% |
| Class F | -0.03% | -0.13% | -0.14% | 101% | 99% |

|  | Low Delay P Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.55% | -0.67% | -0.48% | 103% | 101% |
| Class C | -0.44% | -0.72% | -0.39% | 102% | 97% |
| Class E | -0.30% | -0.14% | -0.44% | 102% | 97% |
| Overall | -0.45% | -0.55% | -0.44% | 103% | 99% |
| Class D | -0.31% | -1.12% | -0.56% | 102% | 98% |
| Class F | -0.18% | 0.47% | 0.38% | 102% | 99% |

Table 2: Results of Test 2 over VTM-5.0

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | 0.01% | 0.22% | 102% | 101% |
| Class A2 | -0.12% | 0.16% | 0.22% | 102% | 101% |
| Class B | -0.13% | 0.13% | -0.01% | 101% | 100% |
| Class C | -0.16% | -0.15% | -0.26% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | 0.04% | 0.02% | 101% | 100% |
| Class D | -0.08% | -0.05% | 0.07% | 100% | 99% |
| Class F | -0.08% | 0.02% | 0.04% | 101% | 99% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.44% | 0.04% | 0.07% | 103% | 99% |
| Class C | -0.35% | -0.60% | -0.35% | 102% | 102% |
| Class E | -0.17% | 0.09% | -0.25% | 102% | 100% |
| Overall | -0.34% | -0.16% | -0.15% | 102% | 100% |
| Class D | -0.30% | -1.00% | -0.69% | 101% | 92% |
| Class F | -0.18% | 0.21% | 0.43% | 102% | 101% |

|  | Low Delay P Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.53% | -0.41% | -0.30% | 104% | 100% |
| Class C | -0.43% | -0.84% | -0.39% | 102% | 101% |
| Class E | -0.31% | 0.39% | 0.27% | 102% | 99% |
| Overall | -0.44% | -0.35% | -0.19% | 103% | 100% |
| Class D | -0.34% | -1.13% | -0.61% | 101% | 101% |
| Class F | -0.21% | 0.48% | 0.06% | 101% | 102% |

Table 3: Results of Test 3 over VTM-5.0

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | 0.01% | 0.31% | 101% | 100% |
| Class A2 | -0.15% | 0.16% | 0.37% | 101% | 100% |
| Class B | -0.18% | 0.25% | 0.16% | 101% | 100% |
| Class C | -0.26% | -0.06% | -0.16% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | 0.10% | 0.15% | 101% | 100% |
| Class D | -0.18% | 0.02% | 0.30% | 99% | 99% |
| Class F | -0.14% | 0.03% | 0.03% | 100% | 99% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.49% | -0.07% | -0.47% | 103% | 100% |
| Class C | -0.41% | -0.46% | -0.41% | 102% | 100% |
| Class E | -0.17% | -0.29% | -0.70% | 101% | 101% |
| Overall | -0.39% | -0.25% | -0.51% | 102% | 100% |
| Class D | -0.41% | -0.76% | -0.42% | 101% | 100% |
| Class F | -0.21% | -0.12% | 0.23% | 101% | 102% |

|  | Low Delay P Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.64% | -0.38% | -0.33% | 104% | 100% |
| Class C | -0.57% | -0.77% | -0.43% | 102% | 101% |
| Class E | -0.48% | 0.05% | -0.58% | 102% | 100% |
| Overall | -0.58% | -0.41% | -0.43% | 103% | 100% |
| Class D | -0.48% | -0.86% | -0.41% | 101% | 99% |
| Class F | -0.34% | 0.42% | 0.22% | 101% | 99% |

Conclusion

This contribution presents two alternative methods for WIIP mode. It is reported that the first method achieves -0.13%, -0.37%, and -0.45% luma BD-rates in RA, LDB, and LDP configurations on top of VTM-5.0 with 101%, 102 and 103% encoding time, and 99%, 100%, and 99% decoding time respectively. The second alternative method achieves -0.13%, -0.34%, and -0.44% luma BD-rates for RA, LDB, and LDP configuration with 101%, 102% and 103% encoding and 100%, 100%, and 100% decoding time respectively. It is also reported that when further applying CIIP and WIIP modes to 4x8 and 8x4 blocks, it achieves -0.19%, -0.39%, and -0.58% luma BD-rates for RA, LDB, and LDP configuration with 101%, 102% and 103% encoding and 100%, 100% and 100% decoding time respectively. It is proposed to adopt this proposal.

References

- M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L0100, Macao, CN, 3–12 Oct. 2018.

- L. P. Van, G. V. Auwera A. K. Ramasubramonian, V. Seregin, and M. Karczewicz “CE10: CIIP with position-independent weights,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0302, Geneva, CH, 19–27 March 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).