| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET- O0538 |
| --- | --- |

| Title: | Non-CE6: An SPS Control of ISP and SBT Transform Selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Fabrice Le Léannec, Michel Kerdranvat 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

ISP and SBT are TU tiling tools used for intra and inter coding (resp.), which provide significant coding gain. Their transform selection is Implicitly composed of combination of DCT2, DST7 and DCT8. Unlike the normal transform blocks, non-DCT2 transforms cannot be controlled by the MTS SPS flag (sps_mts_enabled_flag) in the case of ISP and SBT in VTM software. Therefore, in the case of a light-weight encoder that is not supported with non-DCT2 transform, ISP and SBT must be disabled, leading to large BD-rate loss.

This contribution solves this problem by enabling an ISP and SBT with DCT2 when sps_mts_enabled_flag is disabled. In other words, if non-DCT2 transform is not enabled for the normal transform blocks, it is also not enabled for ISP or SBT. This is to align VTM software with specification test.

Experimental results show significant coding gain when ISP and SBT are enabled with DCT2, compared to the anchor case when they are disabled.

# Introduction

In VTM5.0, the ISP transform selection is driven by this formula:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? DST7 : DCT2	(8-965)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? DST7 : DCT2	(8-966)

where trTypeHor and trTypeVer are the horizontal and vertical transform types and nTbW and nTbH are the width and height of the transform block. On the other hand, SBT transform selection is derived from the cu_sbt_horizontal_flag and cu_sbt_pos_flag as indicated in the table below:

Table 1 SBT transform selection

| cu_sbt_horizontal_flag | cu_sbt_pos_flag | trTypeHor | trTypeVer |
| --- | --- | --- | --- |
| 0 | 0 | DCT8 | DST7 |
| 0 | 1 | DST7 | DST7 |
| 1 | 0 | DST7 | DCT8 |
| 1 | 1 | DST7 | DST7 |

Therefore, both ISP and SBT use a combination of DCT2, DST7 and DCT8 transforms. This combination limits the use of the two tools to the availability of DST7 and DCT8. Practically, if an encoder is designed to have only DCT2 transform, it needs to switch the two tools off, which leads to significant coding loss. The interest behind such an encoder is its low memory requirement. Removing non-DCT2 transform reduces about 2.5Kbytes of data and removing LFNST reduces further 5Kbytes of data.

To elaborate on this, we consider the light-weight encoder which uses only DCT2 as a transform (no MTS and no LFNST). This encoder must switch off ISP and SBT to generate a decodable bitstream.

Taking this encoder as a test (--MTS=0 –LFNST=0 --ISP=0 --SBT=0), and the anchor is VTM5.0 with mts and LFNST disabled (--MTS=0 --LFNST=0), the following results are obtained:

Table 2 Test1: Light-weight Encoder vs VTM5.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 MTS=0 LFNST0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.44% | 0.20% | 0.08% | 93% | 101% |
| Class A2 | 0.89% | 0.31% | 0.60% | 93% | 98% |
| Class B | 0.93% | 0.26% | 0.61% | 90% | 97% |
| Class C | 0.89% | 0.51% | 0.71% | 93% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.81% | 0.33% | 0.53% | 92% | 98% |
| Class D | 0.83% | 0.86% | 0.68% | 94% | 98% |
| Class F | 0.73% | 0.44% | 0.42% | 96% | 99% |

Clearly, we are losing 0.8% just because of switching ISP and SBT off.

# Proposition

This contribution proposes enabling ISP and SBT even when only DCT2 is supported by the encoder. To do so, the SPS flag (sps_mts_enabled_flag) is utilized in this way:

- ISP transform selection:

If (sps_mts_enabled_flag)

{

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? DST7 : DCT2

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? DST7 : DCT2

}

Else

trTypeHor = trTypeHor = DCT2;

- SBT transform selection:

If sps_mts_enabled_flag use Table 1, otherwise:

trTypeHor = trTypeHor = DCT2;

# Experimental Results

The simulations were ran with VTM5.0 using the common testing conditions. The anchor is VTM5.0 with MTS and LFNST disabled (--MTS=0 –LFNST=0) and the tests is the proposed solution with MTS  and LFSNT disabled (--MTS=0 --LFNST=0). The following results were obtained:

Table 3 Experimental results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 MTS=0 LFNST0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.08% | 0.16% | 100% | 101% |
| Class A2 | 0.17% | -0.19% | 0.13% | 101% | 102% |
| Class B | 0.21% | 0.10% | 0.32% | 100% | 100% |
| Class C | 0.16% | 0.06% | 0.13% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | 0.03% | 0.20% | 100% | 101% |
| Class D | 0.16% | 0.08% | 0.07% | 100% | 100% |
| Class F | 0.11% | 0.29% | 0.24% | 100% | 101% |

The results show that the 0.81% loss is reduced to 0.17%. To show the gain of the proposed method, the initial light-weight encoder (--MTS=0 –LFNST=0 --ISP=0 --SBT=0) is taken as an anchor, and the proposed method as test (--MTS=0 –LFNST=0), the results are given in the table below:

Table 4 Experimental results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 MTS=0 LFNST0 ISP=0 SBT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.11% | 0.08% | 107% | 101% |
| Class A2 | -0.71% | -0.50% | -0.47% | 108% | 104% |
| Class B | -0.72% | -0.17% | -0.29% | 112% | 103% |
| Class C | -0.72% | -0.45% | -0.57% | 107% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.64% | -0.30% | -0.33% | 109% | 102% |
| Class D | -0.66% | -0.77% | -0.60% | 107% | 102% |
| Class F | -0.61% | -0.15% | -0.18% | 104% | 102% |

That is, the proposed method achieves 0.6% gain with 110% encoder time, which is a good trade-off between complexity and gain.

Additional results on AI is given in the table below:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 MTS=0 LFNST0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | 0.01% | -0.01% | 126% | 104% |
| Class A2 | -0.29% | -0.17% | -0.20% | 124% | 103% |
| Class B | -0.49% | -0.38% | -0.26% | 127% | 103% |
| Class C | -0.72% | -0.60% | -0.49% | 128% | 104% |
| Class E | -0.78% | -0.47% | -0.45% | 126% | 102% |
| Overall | -0.50% | -0.34% | -0.29% | 126% | 103% |
| Class D | -0.60% | -0.43% | -0.42% | 126% | 104% |
| Class F | -0.71% | -0.35% | -0.40% | 110% | 104% |

# Spec Change

No spec change as it is an alignment between the spec and VTM software.

# Conclusion

This contribution proposes enabling ISP and SBT even when only DCT2 is available in the encoder side. This is done by considering the MTS SPS flag as a tool to control the transform design of ISP and SBT. Experimental results show a significant gain when ISP and SBT are used with DCT2, in the case of light-weight encoder that supports only DCT2.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).