| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0540 |
| --- | --- |

| Title: | Non-CE6: Harmonization of LFNST, MIP and implicit MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In VVC Draft v5, when MTS is disabled, and implicit MTS is enabled, the Low Frequency Non-Separable Transform (LFNST) and Matrix-based Intra Prediction (MIP) must be coupled with a primary transform using DST-7 for block sizes with width or height being smaller than or equal to 16. However, it is noted that the implicit transform scheme in VVC Draft v5 is not functioning efficiently due to the coupling of DST-7 and LFNST (or MIP). To resolve this issue, it is proposed to apply implicit transform only when LFNST and MIP indices are zero. Experimental results reportedly show that, comparing to VTM-5.0 with configuration “--MTS=0 --MTSImplicit=1”, the proposed method achieves 0.79%, 0.35% and 0.03% luma BD-rate reduction for All Intra, Random Access and Low-Delay B configurations, respectively.

# Problem Statement

In VVC Draft v5, for intra prediction residuals, the implicit transform can be applied for the following two cases:

- ISP is applied:

IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- MTS is disabled in SPS but implicit MTS is enabled:

sps_mts_enabled_flag is 1

sps_explicit_mts_intra_enabled_flag is equal to 0

sps_explicit_mts_inter_enabled_flag is equal to 0

For the above two cases, an implicit transform scheme will be applied, and DCT-2 or DST-7 is selected as the primary transform according to the following condition:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? DST7 : DCT2

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? DST7 : DCT2

In last meeting, Low Frequency Non-Separable Transform (LFNST) and Matrix-based Intra Prediction (MIP) are adopted and integrated into VTM-5.0. According to VVC Draft v5, for the above second case, i.e., MTS is disabled in SPS but implicit MTS is enabled, LFNST and MIP must be applied on top of a primary transform scheme using DST7 for block sizes with width or height being smaller than or equal to 16. However, it is noted that DST-7 does not perform very efficiently for LFNST and MIP modes.

The following table summarizes the coding performance of implicit MTS in VTM-5.0 when MTS is disabled, i.e., --MTS=0.

Table 1: Coding performance of implicit MTS in VTM-5.0 (VTM-5.0 --MTS=0 vs VTM-5.0 –MTS=0 --MTSImplicit=1).

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.25% | -0.71% | -0.72% | 104% | 103% | 0.31% | -0.21% | -0.01% | 101% | 100% |  |  |  |  |  |
| Class A2 | -0.10% | -1.02% | -0.90% | 103% | 102% | 0.09% | -0.26% | -0.20% | 101% | 100% |  |  |  |  |  |
| Class B | 0.05% | -1.21% | -1.38% | 104% | 105% | -0.12% | -0.76% | -0.67% | 100% | 102% | -0.25% | -0.28% | -0.82% | 100% | 99% |
| Class C | 0.42% | -0.42% | -0.49% | 101% | 99% | 0.03% | -0.18% | -0.03% | 99% | 96% | -0.13% | -0.48% | -0.17% | 100% | 105% |
| Class E | 0.30% | -0.79% | -0.92% | 101% | 101% |  |  |  |  |  | 0.14% | 1.15% | -0.99% | 101% | 105% |
| Overall | 0.18% | -0.85% | -0.92% | 103% | 102% | 0.05% | -0.40% | -0.27% | 100% | 100% | -0.11% | 0.01% | -0.65% | 100% | 102% |
| Class D | 0.40% | -0.19% | -0.21% | 101% | 97% | 0.05% | -0.22% | -0.05% | 99% | 96% | -0.17% | -0.66% | -0.40% | 98% | 101% |
| Class F | 0.25% | -0.47% | -0.46% | 101% | 102% | 0.13% | -0.11% | -0.16% | 100% | 99% | -0.27% | -0.58% | -0.36% | 100% | 100% |

From the above table, it is noted that implicit MTS shows 0.18% and 0.05% loss when MTS is disabled in VTM-5.0.

# Proposal and results

For intra prediction residual block which is not coded by ISP mode, it is proposed to apply implicit MTS only when LFNST index and MIP index are both zero. That is, when MTS is disabled in SPS but implicit MTS is enabled, the primary transform applied for LFNST and MIP can be only DCT-2.

The following table summarizes the coding performance of proposed method. The anchor is VTM-5.0 and the Test is proposed method, the test condition is Common Test Condition (CTC) with MTS disabled but implicit MTS enabled, i.e., --MTS=0 --MTSImplicit=1.

Table 2: Coding performance of proposal versus VTM-5.0 with implicit MTS.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.99% | 0.00% | -0.17% | 98% | 99% | -0.56% | -0.20% | -0.21% | 99% | 99% |  |  |  |  |  |
| Class A2 | -0.65% | 0.11% | 0.05% | 99% | 100% | -0.32% | 0.01% | 0.07% | 100% | 100% |  |  |  |  |  |
| Class B | -0.70% | 0.25% | 0.30% | 98% | 97% | -0.30% | 0.19% | 0.14% | 101% | 100% | 0.00% | 0.00% | -0.10% | 100% | 99% |
| Class C | -0.70% | 0.02% | 0.11% | 99% | 104% | -0.26% | -0.16% | -0.17% | 101% | 105% | 0.01% | 0.09% | 0.02% | 100% | 96% |
| Class E | -0.99% | -0.13% | 0.09% | 100% | 103% |  |  |  |  |  | -0.12% | -0.34% | 1.25% | 101% | 97% |
| Overall | -0.79% | 0.07% | 0.10% | 99% | 100% | -0.35% | -0.02% | -0.03% | 100% | 101% | -0.03% | -0.06% | 0.28% | 100% | 97% |
| Class D | -0.62% | -0.06% | 0.08% | 100% | 103% | -0.27% | -0.19% | -0.12% | 101% | 104% | -0.07% | 0.28% | -0.05% | 102% | 103% |
| Class F | -0.41% | 0.09% | -0.07% | 100% | 101% | -0.31% | -0.19% | -0.11% | 101% | 103% | 0.06% | -0.31% | -0.21% | 101% | 100% |

Additional test is performed to report the coding performance of proposed implicit MTS comparing VTM-5.0 without implicit MTS. The anchor is VTM-5.0 with MTS disabled (--MTS=0), and the Test is MTS disabled but implicit MTS enabled (--MTS=0 --MTSImplicit=1). From the results, implicit MTS shows over 0.6% coding gain for RA.

Table 3: Coding performance of proposed implicit MTS.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.74% | -0.71% | -0.89% | 101% | 102% | -0.25% | -0.42% | -0.22% | 100% | 99% |  |  |  |  |  |
| Class A2 | -0.75% | -0.91% | -0.85% | 102% | 102% | -0.23% | -0.25% | -0.13% | 100% | 99% |  |  |  |  |  |
| Class B | -0.64% | -0.96% | -1.08% | 102% | 102% | -0.41% | -0.57% | -0.54% | 101% | 102% | -0.25% | -0.28% | -0.91% | 99% | 98% |
| Class C | -0.28% | -0.40% | -0.38% | 100% | 102% | -0.23% | -0.34% | -0.20% | 100% | 101% | -0.13% | -0.39% | -0.15% | 101% | 101% |
| Class E | -0.69% | -0.92% | -0.83% | 100% | 104% |  |  |  |  |  | 0.03% | 0.81% | 0.26% | 101% | 102% |
| Overall | -0.60% | -0.78% | -0.81% | 101% | 102% | -0.30% | -0.41% | -0.30% | 101% | 101% | -0.14% | -0.04% | -0.37% | 100% | 100% |
| Class D | -0.22% | -0.25% | -0.13% | 100% | 100% | -0.22% | -0.40% | -0.17% | 101% | 101% | -0.24% | -0.39% | -0.44% | 100% | 104% |
| Class F | -0.16% | -0.38% | -0.54% | 101% | 103% | -0.18% | -0.30% | -0.27% | 101% | 102% | -0.20% | -0.88% | -0.57% | 100% | 100% |

# Spec text changes

The following proposed text modifications are based on JVET-N1001-v8. The added texts are highlighted in green.

8.7.4.1	General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

……

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are both equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA and lfnst_idx[ x0 ][ y0 ] is equal to 0 and intra_mip_flag[ x0 ][ y0 ] is equal to 0

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT or both sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA, trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(8-965)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(8-966)

- Otherwise, if cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 8-17 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-16 depending on tu_mts_idx[ xTbY ][ yTbY ].

The variables nonZeroW and nonZeroH are derived as follows:

nonZeroW = Min( nTbW, ( trTypeHor > 0 )  ?  16  :  32 )	(8-967)

nonZeroH = Min( nTbH, ( trTypeVer > 0 )  ?  16  :  32 )	(8-968)

……

# Conclusion

This contribution proposed a modified condition for enabling implicit MTS, which applies only DCT-2 for LFNST and MIP modes. Comparing to the implicit transform design in VTM-5.0, the proposed method achieved 0.79% and 0.35% luma BD-rate reduction for All Intra and Random Access configurations, respectively.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).