| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0544 |
| --- | --- |

| Title: | Non-CE6: Modified LFNST Transform Matrices for Fast Implementation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser*, Louis Kerofsky+, Fabrice Le Léannec*, Tangi Poirier* *975 Avenue des champs blancs 35576 Cesson-Sévigné, France +9276 Scranton Road Suite 300 San Diego, CA 92121 | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Recently, LFNST was adopted to VTM5.0 as an additional transform process in non-separable manner. LFNST requires the multiplication of transform matrices (either 16x48 or 16x16) by a vector of input data (of length 16 or 48). Unlike the ordinary transforms of DCT2, DST7 and DCT8, LFNST is a learned transform whose basis functions do not have specific properties that can be used fast implementation.

This contribution proposes a slight modification to the LFNST matrices so that faster implementation can be achieved. Similar to the ordinary transform, the proposed modification allows dual implementation support, in which an LFNST transform can be either implemented by matrix multiplication or a fast algorithm to yield exactly the same results.

Experimental results show some negligible RD loss, while the number of multiplications is significantly reduced in the fast algorithms.

# Introduction

In VTM5.0 decoder side, LFSNT is applied to the top-left coefficients before the MTS inverse transform. For this, two groups of LFNST are defined one for coefficients of size 8x8 and a second for coefficients of size 4x4. For each group, four sets are available depending on the intra prediction direction. Each set comprises two LFSNT transform matrices. In summary, VTM5.0 has:

- 8 LFNST matrices of size 16x48

- 8 LFNST matrices of size 16x16

Therefore, 16 fast algorithms are needed for VTM5.0. The following conditions are considering when designing the fast algorithms:

- Dual implementation support: The transform can be implemented by both fast algorithm and matrix multiplication with exactly the same results

- No cascading of operations is required to perform the fast algorithm

- Independent computation of transform coefficients, which enable parallel computation

Each fast algorithm is obtained by slightly modifying the basis functions and find relationships that reduce the number of multiplications. For example, let us consider computing the first coefficient (C) of the inverse LFNST transform of size 16x16 of the first set and first basis,

C = LFNST_V * In

Where LFNST_V is the first row of the inverse LFNST transform matrix, and In in the input vector. The vector is given by:

LFNST_V = [108   -40    25   -32     8   -25     8     2   -16    -9   -13     8    -2     4     2     0];

And C is computed as:

C = 108*In[0]   -40*In[1]    25*In[2]   -32*In[3]     8*In[4]   -25*In[5]     8*In[6]     2*In[7]   -16*In[8]    -9*In[9]   -13*In[10]     8*In[11]    -2*In[12]     4*In[13]     2*In[14]     0*In[15]

This requires 16 multiplications. By slightly modifying LFNST_V to:

LFNST_V = [108   -40    26   -32     8   -25     8     2   -16    -9   -13     8    -2     4     2     0];

The coefficient can be computed as:

C=-13*(-In[0]*(28)+(In[1]<<3)-(In[2]<<1)+In[5]+In[9]+In[10])-(In[0]<<8)+(In[1]<<6)-(In[3]<<5)+(In[4]<<3)-In[5]*(12)+(In[6]<<3)+(In[7]<<1)-(In[8]<<4)+(In[9]<<2)+(In[11]<<3)-(In[12]<<1)+(In[13]<<2)+(In[14]<<1);

This, in turn, requires only 3 multiplications  rather than 16 multiplications needed by a matrix multiply. The same procedure is done for the all the transform matrices to yield new matrices whose fast implementation requires many fewer operations.

# Proposition

The contribution proposes fast algorithms for the modified versions of 16 transform matrices. It studies two variants with different degree of changes with respect to the original matrices. In the first variant, the maximum absolute difference (MAD) between the new and original LFNST matrices is less or equal to 2, while the second is less or equal to 4. The tables below compare the number of multiplications required for each matrix, where g_lfnst4x4[i][j] are the 4x4 LFNST transform matrices for the ith group and jth index and g_lfnst8x8[i][j] are the 8x8 LFNST transform matrices for the ith group and jth index

Table 1 Number of multiplications required for the inverse LFNST transform

| Matrix | Def: MAD=0 | MAD <= 1 | MAD <= 4 |
| --- | --- | --- | --- |
| g_lfnst4x4[0][0] | 256 | 69 | 45 |
| g_lfnst4x4[0][1] | 256 | 76 | 47 |
| g_lfnst4x4[1][0] | 256 | 72 | 45 |
| g_lfnst4x4[1][1] | 256 | 77 | 52 |
| g_lfnst4x4[2][0] | 256 | 65 | 42 |
| g_lfnst4x4[2][1] | 256 | 86 | 54 |
| g_lfnst4x4[3][0] | 256 | 83 | 58 |
| g_lfnst4x4[3][0] | 256 | 79 | 45 |

Table 2 Number of multiplications required for the forward LFNST transform

| Matrix | Def: MAD=0 | MAD <= 1 | MAD <= 4 |
| --- | --- | --- | --- |
| g_lfnst4x4[0][0] | 256 | 109 | 104 |
| g_lfnst4x4[0][1] | 256 | 111 | 99 |
| g_lfnst4x4[1][0] | 256 | 110 | 118 |
| g_lfnst4x4[1][1] | 256 | 147 | 145 |
| g_lfnst4x4[2][0] | 256 | 113 | 116 |
| g_lfnst4x4[2][1] | 256 | 113 | 107 |
| g_lfnst4x4[3][0] | 256 | 138 | 133 |
| g_lfnst4x4[3][0] | 256 | 126 | 112 |

Table 3 Number of multiplications required for the inverse LFNST transform

| Matrix | Def: MAD=0 | MAD <= 1 | MAD <= 4 |
| --- | --- | --- | --- |
| g_lfnst8x8[0][0] | 768 | 104 | 68 |
| g_lfnst8x8[0][1] | 768 | 118 | 68 |
| g_lfnst8x8[1][0] | 256 | 141 | 91 |
| g_lfnst8x8[1][1] | 768 | 135 | 90 |
| g_lfnst8x8[2][0] | 768 | 90 | 55 |
| g_lfnst8x8[2][1] | 768 | 133 | 78 |
| g_lfnst8x8[3][0] | 768 | 133 | 87 |
| g_lfnst8x8[3][0] | 768 | 110 | 79 |

Table 4 Number of multiplications required for the forward LFNST transform

| Matrix | MAD=0 | MAD <= 1 | MAD <= 4 |
| --- | --- | --- | --- |
| g_lfnst8x8[0][0] | 768 | 151 | 146 |
| g_lfnst8x8[0][1] | 768 | 154 | 153 |
| g_lfnst8x8[1][0] | 256 | 182 | 177 |
| g_lfnst8x8[1][1] | 768 | 213 | 197 |
| g_lfnst8x8[2][0] | 768 | 118 | 116 |
| g_lfnst8x8[2][1] | 768 | 190 | 176 |
| g_lfnst8x8[3][0] | 768 | 174 | 166 |
| g_lfnst8x8[3][0] | 768 | 177 | 178 |

As indicated in Table 1 and Table 3, the number of operations for computing the inverse transform is highly reduced: from 256 to 45 for g_lfnst4x4 and from 768 to 55 for g_lfnst8x8. That is, 18% and 7% of the original number of multiplications.

# Experimental Results

The simulations were run with VTM5.0 using the common testing conditions. The anchor is VTM5.0 and two test conditions are considered:

- Test1: LFNST matrices with MAD less than or equal to 2

- Test2: LFNST matrices with MAD less than or equal to 4

Table 5 Experimental results of Test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.09% | 0.12% | 100% | 100% |
| Class A2 | 0.03% | -0.03% | -0.03% | 100% | 99% |
| Class B | 0.02% | -0.03% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.01% | 0.07% | 101% | 100% |
| Class E | 0.03% | 0.15% | -0.01% | 100% | 99% |
| Overall | 0.03% | 0.03% | 0.03% | 100% | 99% |
| Class D | 0.00% | 0.25% | -0.08% | 101% | 100% |
| Class F | 0.00% | 0.05% | 0.09% | 100% | 99% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 6 Experimental results of Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.10% | 0.12% | 101% | 103% |
| Class A2 | 0.02% | -0.02% | 0.02% | 100% | 105% |
| Class B | 0.04% | -0.02% | 0.07% | 95% | 95% |
| Class C | 0.04% | -0.04% | 0.06% | 105% | 94% |
| Class E | 0.05% | 0.09% | 0.04% | 108% | 103% |
| Overall | 0.05% | 0.01% | 0.06% | 101% | 99% |
| Class D | 0.02% | 0.26% | -0.10% | 107% | 105% |
| Class F | -0.01% | 0.07% | 0.13% | 104% | 95% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

The experimental results show that the reduction in number of multiplications is associated with negligible loss (0.03% for test1 and 0.03% for test 2).

Additional tests were carried out with low QP configuration. The results are given below:

Table 7 Experimental results of Test 1 (low QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 LowQP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.02% | 0.00% | 0.03% | 98% | 99% |
| Class C | 0.02% | 0.01% | 0.02% | 104% | 103% |
| Class E | 0.03% | 0.02% | -0.01% | 95% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.03% | -0.03% | 0.02% | 109% | 104% |
| Class F | 0.04% | 0.01% | -0.01% | 103% | 103% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 8 Experimental results of Test 2 (low QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 LowQP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.04% | 0.00% | 100% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.04% | 0.04% | 0.04% | 98% | 99% |
| Class C | 0.08% | 0.07% | 0.04% | 103% | 103% |
| Class E | 0.10% | 0.00% | 0.01% | 97% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.09% | 0.00% | 0.04% | 108% | 106% |
| Class F | 0.07% | 0.05% | 0.04% | 102% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

This contribution proposed two fast algorithms of LFNST with dual support. The number of multiplications is reduced to a maximum of 7% of the original value. This comes with some negligible coding loss (0.03% and 0.05%).

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).