| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0545-v2 |
| --- | --- |

| Title: | Non-CE6: Configurable max transform size in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes to include configurable max transform size in VVC. With the proposed SW and spec text fixes, the encoder has the flexibility to choose only implementing up to 16-length, 32-length, or 64-length transform, without changing the max coding unit size.

# Proposal

In VVC Draft 5, the max transform size (log2_max_luma_transform_block_size_minus2) is signalled in the SPS but it is fixed as 64-length and not configurable. It is proposed to support configurable max transform size in SPS, such that encoder has the flexibility to choose up to 16-length, 32-length or 64-length transform size depending on the need of specific implementation.

To achieve this goal, one issue regarding the sub-block transform (SBT) needs to be addressed. In SBT, the maximum CU width and height for allowing subblock transform is indicated in VVC Draft 4 as below,

sps_sbt_max_size_64_flag equal to 0 specifies that the maximum CU width and height for allowing subblock transform is 32 luma samples. sps_sbt_max_size_64_flag equal to 1 specifies that the maximum CU width and height for allowing subblock transform is 64 luma samples.

MaxSbtSize = sps_sbt_max_size_64_flag  ?  64  :  32	(7-31)

To support maximum transform size (indicated by MaxTransformSize) smaller than 64, the derivation of MaxSbtSize is modified as below,

MaxSbtSize = min(MaxTransformSize, sps_sbt_max_size_64_flag  ?  64  :  32)	(7-31)

# Experiment results

Three experiments are performed using different configuration of max transform size after applying the proposed fixes on VTM-5.0. The experiment is performed under CTC condition except for the different settings of max transform size. The anchor for all the experiments is VTM-5.0. Detailed results for each experiment are reported in Table 1, Table 2 and Table 3, respectively.

Table 1: Coding performance of max transform size 64

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 97% | 0.00% | 0.00% | 0.00% | 100% | 98% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 101% | 103% | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 100% |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 105% | 0.00% | 0.00% | 0.00% | 101% | 104% | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 99% | 0.00% | 0.00% | 0.00% | 102% | 102% | 0.00% | 0.00% | 0.00% | 101% | 99% |

Table 2: Coding performance of max transform size 32

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.92% | 8.48% | 6.17% | 101% | 105% | 2.87% | 9.10% | 7.11% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.46% | 3.57% | 2.23% | 101% | 103% | 1.15% | 3.89% | 2.56% | 99% | 100% |  |  |  |  |  |
| Class B | 0.26% | 3.39% | 4.34% | 100% | 100% | 0.81% | 3.73% | 3.51% | 99% | 98% | 1.36% | 5.44% | 4.39% | 100% | 98% |
| Class C | 0.07% | 0.61% | 0.59% | 101% | 101% | 0.21% | 0.77% | 0.51% | 102% | 102% | 0.45% | 1.27% | 1.04% | 102% | 95% |
| Class E | 0.48% | 4.04% | 4.84% | 102% | 105% |  |  |  |  |  | 1.14% | 4.20% | 6.12% | 102% | 95% |
| Overall | 0.40% | 3.76% | 3.55% | 101% | 102% | 1.13% | 4.05% | 3.24% | 100% | 100% | 1.00% | 3.74% | 3.71% | 101% | 96% |
| Class D | 0.04% | 0.51% | 0.33% | 102% | 106% | 0.03% | 0.33% | 0.18% | 104% | 104% | 0.09% | 0.68% | 1.17% | 102% | 98% |
| Class F | 0.13% | 1.46% | 1.43% | 102% | 102% | 0.38% | 1.10% | 1.37% | 103% | 103% | 0.68% | 1.73% | 2.14% | 104% | 96% |

Table 3: Coding performance of max transform size 16

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 6.08% | 28.59% | 24.21% | 104% | 119% | 10.08% | 34.09% | 28.06% | 101% | 103% |  |  |  |  |  |
| Class A2 | 3.00% | 14.68% | 10.03% | 102% | 115% | 4.45% | 15.57% | 10.23% | 96% | 101% |  |  |  |  |  |
| Class B | 2.39% | 14.79% | 17.77% | 102% | 111% | 3.79% | 16.59% | 16.79% | 98% | 99% | 5.30% | 19.97% | 18.00% | 96% | 100% |
| Class C | 0.76% | 4.98% | 4.98% | 102% | 107% | 1.60% | 5.25% | 4.12% | 99% | 103% | 2.37% | 7.43% | 5.92% | 96% | 95% |
| Class E | 3.69% | 16.77% | 15.81% | 104% | 115% |  |  |  |  |  | 4.80% | 20.35% | 19.33% | 100% | 95% |
| Overall | 2.96% | 15.22% | 14.39% | 103% | 112% | 4.60% | 16.86% | 14.35% | 98% | 101% | 4.20% | 15.88% | 14.31% | 97% | 97% |
| Class D | 0.71% | 4.10% | 3.53% | 104% | 111% | 0.58% | 3.58% | 2.94% | 102% | 105% | 1.00% | 5.11% | 4.77% | 98% | 99% |
| Class F | 1.11% | 6.30% | 6.56% | 102% | 111% |  |  |  |  |  | 2.92% | 7.10% | 6.08% | 101% | 97% |

# Spec text changes

The following proposed text modifications are based on JVET-N1001-v8. The added texts are highlighted in green, and deleted texts are marked by red strikethrough.

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if ( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| sps_log2_max_luma_transform_size_minus4 | ue(v) |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_pcm_enabled_flag | u(1) |
| if( sps_pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| …… |  |

7.4.3.3	Sequence parameter set RBSP semantics

……

log2_ctu_size_minus2 plus 2 specifies the luma coding tree block size of each CTU.

log2_min_luma_coding_block_size_minus2 plus 2 specifies the minimum luma coding block size.

The variables CtbLog2SizeY, CtbSizeY, MinCbLog2SizeY, MinCbSizeY, MinTbLog2SizeY, MaxTbLog2SizeY, MinTbSizeY, MaxTbSizeY, PicWidthInCtbsY, PicHeightInCtbsY, PicSizeInCtbsY, PicWidthInMinCbsY, PicHeightInMinCbsY, PicSizeInMinCbsY, PicSizeInSamplesY, PicWidthInSamplesC and PicHeightInSamplesC are derived as follows:

CtbLog2SizeY = log2_ctu_size_minus2 + 2		(7-10)

CtbSizeY = 1  <<  CtbLog2SizeY		(7-11)

MinCbLog2SizeY = log2_min_luma_coding_block_size_minus2 + 2	(7-12)

MinCbSizeY = 1  <<  MinCbLog2SizeY		(7-13)

MinTbLog2SizeY = 2		(7-14)

MaxTbLog2SizeY = 6sps_log2_max_luma_transform_size_minus4 + 4	(7-15)

MinTbSizeY = 1  <<  MinTbLog2SizeY		(7-16)

MaxTbSizeY = 1  <<  MaxTbLog2SizeY		(7-17)

PicWidthInCtbsY = Ceil( pic_width_in_luma_samples ÷ CtbSizeY )	(7-18)

PicHeightInCtbsY = Ceil( pic_height_in_luma_samples ÷ CtbSizeY )	(7-19)

PicSizeInCtbsY = PicWidthInCtbsY * PicHeightInCtbsY	(7-20)

PicWidthInMinCbsY = pic_width_in_luma_samples / MinCbSizeY	(7-21)

PicHeightInMinCbsY = pic_height_in_luma_samples / MinCbSizeY	(7-22)

PicSizeInMinCbsY = PicWidthInMinCbsY * PicHeightInMinCbsY	(7-23)

PicSizeInSamplesY = pic_width_in_luma_samples * pic_height_in_luma_samples	(7-24)

PicWidthInSamplesC = pic_width_in_luma_samples / SubWidthC	(7-25)

PicHeightInSamplesC = pic_height_in_luma_samples / SubHeightC	(7-26)

[Ed. (BB): Currently the maximum transform size (64x64 luma samples and corresponding chroma sample size) and the minimum transform size (4x4 luma samples and corresponding chroma samples) is fixed, pending further specification development.]

……

sps_log2_diff_max_tt_min_qt_intra_tile_group_chroma specifies the default difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in tile groups with tile_group_type equal to 2 (I) referring to the SPS. When partition_constraints_override_flag is equal to 1, the default difference can be overridden by tile_group_log2_diff_max_tt_min_qt_chroma present in the tile group header of the tile groups referring to the SPS. The value of sps_log2_diff_max_tt_min_qt_intra_tile_group_chroma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraC, inclusive. When sps_log2_diff_max_tt_min_qt_intra_tile_group_chroma is not present, the value of sps_log2_diff_max_tt_min_qt_intra_tile_group_chroma is inferred to be equal to 0.

sps_log2_max_luma_transform_size_minus4 specifies the base 2 logarithm of the maximum transform size minus 4.

sps_sao_enabled_flag equal to 1 specifies that the sample adaptive offset process is applied to the reconstructed picture after the deblocking filter process. sps_sao_enabled_flag equal to 0 specifies that the sample adaptive offset process is not applied to the reconstructed picture after the deblocking filter process.

sps_alf_enabled_flag equal to 0 specifies that the adaptive loop filter is disabled. sps_alf_enabled_flag equal to 1 specifies that the adaptive loop filter is enabled.

…

sps_sbt_enabled_flag equal to 0 specifies that subblock transform for inter-predicted CUs is disabled. sps_sbt_enabled_flag equal to 1 specifies that subblock transform for inter-predicteds CU is enabled.

sps_sbt_max_size_64_flag equal to 0 specifies that the maximum CU width and height for allowing subblock transform is 32 luma samples. sps_sbt_max_size_64_flag equal to 1 specifies that the maximum CU width and height for allowing subblock transform is 64 luma samples.

MaxSbtSize = min(MaxTransformSize, sps_sbt_max_size_64_flag  ?  64  :  32 )	(7-31)

sps_affine_enabled_flag specifies whether affine model based motion compensation can be used for inter prediction. If sps_affine_enabled_flag is equal to 0, the syntax shall be constrained such that no affine model based motion compensation is used in the CVS, and inter_affine_flag and cu_affine_type_flag are not present in coding unit syntax of the CVS. Otherwise (sps_affine_enabled_flag is equal to 1), affine model based motion compensation can be used in the CVS.

7.3.7.9	Transform tree syntax

| transform_tree( x0, y0, tbWidth, tbHeight , treeType) { | Descriptor |
| --- | --- |
| InferTuCbfLuma = 1 |  |
| if( IntraSubPartitionsSplitType = = NO_ISP_SPLIT && !cu_sbt_flag ) { |  |
| if( tbWidth > MaxTbSizeY ¦ ¦ tbHeight > MaxTbSizeY ) { |  |
| splitVer = ( tbWidth > MaxTbSizeY && tbWidth > tbHeight ) ? 1 : 0 |  |
| trafoWidth = ( tbWidth > MaxTbSizeY ) splitVer ? (tbWidth / 2) : tbWidth |  |
| trafoHeight = ( tbHeight > MaxTbSizeY ) !splitVer ? (tbHeight / 2) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight ) |  |
| if( tbWidth > MaxTbSizeY splitVer ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight, treeType ) |  |
| elseif( tbHeight > MaxTbSizeY) |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight, treeType ) |  |
| if( tbWidth > MaxTbSizeY && tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0 + trafoHeight, trafoWidth, trafoHeight, treeType ) |  |
| } else { |  |
| transform_unit( x0, y0, tbWidth, tbHeight, treeType, 0 ) |  |
| } |  |
| } else if( cu_sbt_flag ) { |  |
| if( !cu_sbt_horizontal_flag ) { |  |
| trafoWidth = tbWidth * SbtNumFourthsTb0 / 4 |  |
| transform_unit( x0, y0, trafoWidth, tbHeight, treeType , 0 ) |  |
| transform_unit( x0 + trafoWidth, y0, tbWidth − trafoWidth , tbHeight, treeType , 1 ) |  |
| } else { |  |
| trafoHeight = tbHeight * SbtNumFourthsTb0 / 4 |  |
| transform_unit( x0, y0, tbWidth, trafoHeight, treeType , 0 ) |  |
| transform_unit( x0, y0 + trafoHeight, tbWidth, tbHeight − trafoHeight, treeType , 1 ) |  |
| } |  |
| } else if( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT ) { |  |
| trafoHeight = tbHeight / NumIntraSubPartitions |  |
| for( partIdx = 0; partIdx < NumIntraSubPartitions; partIdx++ ) |  |
| transform_unit( x0, y0 + trafoHeight * partIdx, tbWidth, trafoHeight, treeType, partIdx ) |  |
| } else if( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) { |  |
| trafoWidth = tbWidth / NumIntraSubPartitions |  |
| for( partIdx = 0; partIdx < NumIntraSubPartitions; partIdx++ ) |  |
| transform_unit( x0 + trafoWidth * partIdx, y0, trafoWidth, tbHeight, treeType, partIdx ) |  |
| } |  |
| } |  |

# Conclusion

This proposal provides several fixes to VTM software and Spec text for supporting configurable max transform size. With the proposed method, the encoder has the flexibility of choosing only up to 16-length, 32-length or 64-length transforms.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).