| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0547 |
| --- | --- |

| Title: | Non-CE: Modified MIP Matrices for Fast Implementation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser*, Louis Kerofsky+, Fabrice Le Léannec*, Tangi Poirier* *975 Avenue des champs blancs 35576 Cesson-Sévigné, France +9276 Scranton Road Suite 300 San Diego, CA 92121 | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Recently, MIP was adopted to VTM5.0 as an additional prediction mode that is based on partial multiplication of the reference samples by a prediction matrix. Unlike the ordinary intra prediction mode, MIP requires matrix-vector multiplication that can be time consuming.

This contribution proposes a slight modification to the MIP matrices so that faster implementation can be achieved. The modification allows dual implementation support, in which an MIP transform can be either implemented by matrix multiplications or fast algorithm to yield exactly the same results.

Experimental results show some negligible RD loss, while the number of multiplications is significantly reduced to 18% and 7% of the original number.

# Introduction

MIP is new tool that improves the intra prediction mechanism of VVC. In a recent update, a new version of MIP is provided with 8-bit representation of the matrices (JVET-O0084). This version is taken into consideration in this contribution.

In JVET-O0084, there exist three groups of MIP matrices: 4x4, 8x8 and 16x16. Each of the groups has multiple matrices. In summary:

- 18 MIP matrices of size 16x4 (mipMatrix4x4)

- 10 MIP matrices of size 16x8 (mipMatrix8x8)

- 6 MIP matrices of size 64x7 (mipMatrix16x16)

Therefore, 34 fast algorithms are needed to implement MIP.  The following conditions are considering when designing the fast algorithms:

- Dual implementation support: MIP can be implemented by both fast algorithm and matrix multiplication with exactly the same results

- No cascading of operations is required to perform the fast algorithm

- Independent computation of prediction samples, which enable parallel computation

Each fast algorithm is obtained by slightly modifying the prediction matrices and find relationships that reduce the number of multiplications.

# Proposition

The contribution proposes fast algorithms for the modified versions of 34 MIP matrices. It studies two variants with different degree of changes with respect to the original matrices. In the first variant, the maximum absolute difference (MAD) between the new and original LFNST matrices is less or equal to 1, while the second is less or equal to 5. The tables below compare the number of multiplications required for each matrix, where mipMatrix4x4[i] is the 4x4 MIP matrix of the ith group (similar for mipMatrix8x8 and mipMatrix16x16)

Table 1 Number of multiplications required for mipMatrix4x4 multiplication computation

| Matrix | Def: MAD=0 | MAD <= 1 | MAD <= 5 |
| --- | --- | --- | --- |
| mipMatrix4x4[0] | 64 | 45 | 39 |
| mipMatrix4x4[1] | 64 | 44 | 42 |
| mipMatrix4x4[2] | 64 | 41 | 36 |
| mipMatrix4x4[3] | 64 | 44 | 37 |
| mipMatrix4x4[4] | 64 | 38 | 31 |
| mipMatrix4x4[5] | 64 | 47 | 41 |
| mipMatrix4x4[6] | 64 | 47 | 46 |
| mipMatrix4x4[7] | 64 | 43 | 36 |
| mipMatrix4x4[8] | 64 | 47 | 44 |
| mipMatrix4x4[9] | 64 | 36 | 36 |
| mipMatrix4x4[10] | 64 | 44 | 38 |
| mipMatrix4x4[11] | 64 | 45 | 41 |
| mipMatrix4x4[12] | 64 | 46 | 41 |
| mipMatrix4x4[13] | 64 | 40 | 39 |
| mipMatrix4x4[14] | 64 | 44 | 40 |
| mipMatrix4x4[15] | 64 | 45 | 44 |
| mipMatrix4x4[16] | 64 | 43 | 36 |
| mipMatrix4x4[17] | 64 | 43 | 42 |

Table 2 Number of multiplications required for mipMatrix8x8 multiplication computation

| Matrix | Def: MAD=0 | MAD <= 1 | MAD <= 5 |
| --- | --- | --- | --- |
| mipMatrix8x8[0] | 128 | 60 | 54 |
| mipMatrix8x8[1] | 128 | 63 | 53 |
| mipMatrix8x8[2] | 128 | 59 | 50 |
| mipMatrix8x8[3] | 128 | 67 | 63 |
| mipMatrix8x8[4] | 128 | 64 | 58 |
| mipMatrix8x8[5] | 128 | 64 | 58 |
| mipMatrix8x8[6] | 128 | 65 | 59 |
| mipMatrix8x8[7] | 128 | 60 | 53 |
| mipMatrix8x8[8] | 128 | 67 | 59 |
| mipMatrix8x8[9] | 128 | 63 | 57 |

Table 3 Number of multiplications required for mipMatrix16x16 multiplication computation

| Matrix | Def: MAD=0 | MAD <= 1 | MAD <= 5 |
| --- | --- | --- | --- |
| mipMatrix16x16[0] | 448 | 186 | 167 |
| mipMatrix16x16[1] | 448 | 236 | 208 |
| mipMatrix16x16[2] | 448 | 230 | 201 |
| mipMatrix16x16[3] | 448 | 190 | 162 |
| mipMatrix16x16[4] | 448 | 225 | 205 |
| mipMatrix16x16[5] | 448 | 217 | 182 |

As indicated in Table 1 and Table 3, the number of operations for computing the inverse transform is highly reduced: from 256 to 45 for g_lfnst4x4 and from 768 to 55 for g_lfnst8x8. That is, 18% and 7% of the original number of multiplications.

# Experimental Results

The simulations were run with VTM5.0 using the common testing conditions. The anchor is VTM5.0 and two test conditions are considered:

- Test1: JVET-O0084 + MIP matrices with MAD less than or equal to 2

- Test2: JVET-O0084 + MIP matrices with MAD less than or equal to 4

Table 4 Experimental results of Test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.12% | -0.12% | 100% | 99% |
| Class A2 | -0.05% | -0.08% | -0.14% | 101% | 102% |
| Class B | -0.05% | -0.09% | -0.04% | 101% | 100% |
| Class C | -0.02% | 0.01% | -0.03% | 101% | 97% |
| Class E | -0.07% | -0.11% | -0.15% | 100% | 97% |
| Overall | -0.05% | -0.07% | -0.09% | 101% | 99% |
| Class D | -0.02% | 0.03% | -0.16% | 101% | 97% |
| Class F | -0.02% | 0.03% | -0.08% | 100% | 102% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 5 Experimental results of Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.18% | -0.15% | 102% | 103% |
| Class A2 | -0.05% | -0.06% | -0.15% | 105% | 107% |
| Class B | -0.04% | -0.14% | -0.11% | 100% | 102% |
| Class C | 0.00% | -0.17% | 0.02% | 98% | 94% |
| Class E | -0.05% | -0.06% | -0.01% | 97% | 93% |
| Overall | -0.04% | -0.12% | -0.08% | 100% | 100% |
| Class D | -0.02% | 0.14% | -0.12% | 96% | 91% |
| Class F | -0.01% | -0.02% | 0.11% | 97% | 98% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Taking JVET-O0084 as anchor, we arrive at the following results

Table 6 Test 1 over JVET-O0084

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-O0084 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.13% | #DIV/0! | #DIV/0! |
| Class A2 | 0.00% | 0.02% | -0.02% | #DIV/0! | #DIV/0! |
| Class B | 0.00% | 0.03% | 0.03% | #DIV/0! | #DIV/0! |
| Class C | 0.00% | 0.08% | -0.07% | #DIV/0! | #DIV/0! |
| Class E | -0.01% | -0.03% | -0.09% | #DIV/0! | #DIV/0! |
| Overall | 0.00% | 0.03% | 0.00% | #DIV/0! | #DIV/0! |
| Class D | 0.01% | -0.02% | -0.14% | #DIV/0! | #DIV/0! |
| Class F | -0.02% | 0.03% | -0.08% | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 7 Test2 over JVET-O0084

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.03% | 0.10% | #DIV/0! | #DIV/0! |
| Class A2 | 0.01% | 0.04% | -0.04% | #DIV/0! | #DIV/0! |
| Class B | 0.01% | -0.02% | -0.04% | #DIV/0! | #DIV/0! |
| Class C | 0.02% | -0.09% | -0.02% | #DIV/0! | #DIV/0! |
| Class E | 0.01% | 0.02% | 0.06% | #DIV/0! | #DIV/0! |
| Overall | 0.01% | -0.02% | 0.01% | #DIV/0! | #DIV/0! |
| Class D | 0.01% | 0.09% | -0.10% | #DIV/0! | #DIV/0! |
| Class F | -0.01% | -0.02% | 0.11% | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

This contribution proposes new MIP matrices that can be implemented with fast algorithm. It is based on JVET-O0084 (8 bit representation). The experimental results show no loss with the first variant of the proposed method, and 0.01% loss of the second variant.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).