| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0554-v1 |
| --- | --- |

| Title: | Removal of infrequently used context models |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexander Alshin Jill Boyce Pavel Frolov Vasily Aristarkhov | Email: | alexander.alshin@intel.com jill.boyce@intel.com pavel.frolov@intel.com vasily.aristarkhov@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

The latest VTM5.0 uses 424 context models which correspond to 65 different types of data starting with SplitFlag and ending with TsResidualSign. However the frequency of use of context models differs significantly. This contribution proposes to reduce the number of contexts by removing the 100 least frequently used context models for the RA CTC, which together account for only 0.5% of the total context usage. The removed contexts are replaced by bypass or a proposed generalized bypass.When bypass is substituted for 100 removed contexts, this contribution shows a 0.32% bitrate increase for RA. When generalized bypass is substituted for 100 removed contexts, the bitrate increase for RA is reduced to 0.19%.  When the number of removed contexts is reduced to 90 or 80 contexts and generalized bypass is used, the bitrate increase for RA is reduced to 0.16% or 0.09%, respectively.

# Introduction

The use of context models provides additional compression gain for arithmetic coding. In fact, there is a substitution of the probability for conditional probability. The data that needs to be coded is distributed according to the context models. However if number of bins inside a context model is small it does not always make sense. The number of updates is not enough to converge from initial probability to optimal value. Suppose y is current bin. If exponential smoothing technique is used for probability estimation, the method was proposed in 1957 by Holt [1]:

Pnew=αy+1-αPold,

then N=1α are usually considered as number of updates which is needed to reach optimal probability. More precisely, this number of updates is needed to reduce error of initialization e-times (e=2.718281..). Also it describes how many of previously encoded bins have significant influence to the current probability estimation. For CABAC [2] N=19.69 (it means about 20 preceding bins are taken into account in probability up-date process). Note that majority of context models has positive autocorrelation in this case the use of two estimations with different α is more effective [3]-[5]. If two estimators with α1, α2 are calculated and their mixture is used for final probability estimation the similar number of updates is  N=2(α1+α2). Thus using context bins for data where number of updates less than а few dozen for frame is not effective. The error of initialization does not allow effectively compress such data and also the impact of these context models to total bit-size is insignificant.

In order to determine which context model can be removed significant and reliable statistical analysis with regard to number of update in each context model should be carried out. Reference s/w VTM5.0 consists of 424 context models. However the frequency of use of context models varies significantly by context.  Some of them are updated thousands times per frame, while others have only a few updates. For example, the Random Access test class A cumulative statistics are shown in Figure 1.

Fig.1: Cumulative statistics for the frequency of ctx application for RA Classs A.

The 100 rarest context models altogether process only 0.5% of the bins. The ratio of number of updates for most frequent context model to rarest is 69000x.

# Proposal

Generalized Bypass (G-pass)

If the rarely used context models are removed, we can substitute context bins with bypass bins. However if the probability is much different from one half (1/2) the use of bypass bin may cause reduction of compression efficiency. The bypass bin has simplest realization. A generalized bypass mode (G-pass) is proposed for probabilities other than 1/2. There is a set of numbers among all possible probabilities that covers a range of bypass probabilities: (1/4, 3/4), (3/4, 1/4); (1/8, 7/8), (7/8, 1/8)… (1/2^p, 1-1/2^p), (1-1/2^p, 1/2^p) etc.

Fig.2 Probability mesh for G-pass.

For coding of G-pass/bypass, the probabilities are fixed and it is not necessary to update and store them. While encoding bypass bin, the range is split right down the middle and next step is renormalization and Range will be doubled. That is why in bypass case interval subdivision and renormalization finally does not change Range. The same situation is reproduced for G-pass with only difference that after division by four and multiplication by four two least significant bits become zeros. A unique property of bypass which is not inherited by G-pass is the opportunity to encode several bins simultaneously.

Comparison of bypass, G-pass, and context bin

|  | memory | update | renormalization | compression | Simul enc/dec |
| --- | --- | --- | --- | --- | --- |
| bypass | no | no | no | no | yes |
| context | 3 bytes | yes | yes | best | no |
| G-pass | no | no | simple | average | no |

Bypass/G-pass condition of the choice

Suppose p is probability of “1” but only bypass and G-pass (1/4, 3/4), (3/4, 1/4) are possible. For G-pass (1/4,3/4) average number of bits for one bin will be:

bits=-plog214-1-plog234

For G-pass (3/4,1/4) average number of bits for one bin will be:

bits=-plog234-1-plog214

And finally according famous Shannon formula [6] entropy limit is

bits=-plog2p-1-plog21-p

Fig.3. The average bits for one bin for G-pass, Bypass and Shannon limit.

Thus if probability is within the range [0.37,0.63] then conventional by-pass is better than G-pass. For intervals [0,0.37] and [0.63,1.0] G-pass is winning.

For implementation in reference s/w array of 424 bool elements was introduced. It allows to determine for each ctx_id whether context bin will be used or not. If context bin is not chosen the choice between bypass and G-pass bin depends on initial probability and should be done once for each initialization. Note that initial probability depends on QP it means that for one QP conventional bypass can be chosen but for another Qp G-pass will be used. The full list of substituted models is contained in the Appendix A.

Note that there are several context models which are considered to be removed which do not have updates at all. They are represented in Appendix B. Some of them are not used due to common test conditions but likely will be used if configuration file is modified (deltaQp etc.). Detailed study is needed

# Test results

In this contribution test results for 6 different tests are provided. Left column is 100, 90 and 80 context models were substituted for bypass mode. For right coulomb the same number of context model were substituted for bypass or G-pass modes depending on initial probability.

## VTM 5.0 100ctx substituted to bypass or G-pass

| Only bypass | G-pass/bypass |
| --- | --- |

## VTM 5.0 90ctx substituted to bypass or G-pass

| Only bypass | G-pass/bypass |
| --- | --- |

## VTM 5.0 80ctx substituted to bypass or G-pass

| Only bypass | G-pass/bypass |
| --- | --- |

# Conclusion

In this contribution 80, 90, or 100 of the least frequently used context models were replaced with bypass or G-pass mode according statistics of use. This is 20-25 % of the total number of context models. Removing these context models can reduce implementation complexity by reducing storage and operations.  A generalized bypass mode is proposed to reduce the impact on coding efficiency when the probabilities differ from one-half.

It is proposed that removal of contexts and addition of a G-pass mode be studied in a CE.

# References.

- Holt C.C. Forecasting trends and seasonal by exponentially weighted moving averages. //O.N.R. Memorandum, Carnegie Inst. Of Technology. 1957. №2.

- D. Marpe, H. Schwarz, and T. Wiegand. Context-Based Adaptive Binary Arithmetic Coding in the H.264 / AVC Video Compression Standard, IEEE Transactions on Circuits and Systems for Video Technology, Vol. 13, No. 7, pp. 620-636, July 2003

- E. Alshina, A. Alshin. JCTVC-F254: "Multi-parameter probability up-date for CABAC". Torino 2011.

- A. Alshin, E. Alshina, J.Park. High precision probability estimation for CABAC. VCIP2013.

- F. Bossen. JVET-M0453 “CE5 on arithmetic coding: experiments “ Marrakech 2019

- F. Bossen. J.Chen, S. Liu. Versatile Video Coding (Draft 5). JVET-N1001. Geneva 2019

- C. E. Shannon. A Mathematical Theory of Communication. The Bell
System Technical Journal, 27(3):2163–2177, July 1948

# Appendix A

In this table all context models are sorted in the order of initialization 65 types of data, 424 models are presented. First 80 ctx notation is usual font which according statistics are considered to be removed, next 10 ctx (90 total) are blue and italic, last 10 ctx (total 100) are bold and navy.

|  | Ctx_id | Number of ctx | Removed ctx |
| --- | --- | --- | --- |
| SplitFlag. SplitQtFlag. SplitHvFlag. Split Flag. SkipFlag. MergeFlag. RegularMergeFlag. MergeIdx. MmvdFlag. MmvdMergeIdx. MmvdStepMvpIdx. PredMode MultiRefLineIdx. IntraLumaMpmFlag. IntraLumaPlanarFlag IntraChromaPredMode MipFlag. | 0-8. 9-14. 15-19. 20-23. 24-26. 27. 28-29. 30. 31. 32. 33. 34-35. 36-38. 39. 40-41 42-44 45-48 | 9. 6. 5. 4. 3. 1. 2. 1. 1. 1. 1. 2. 3. 1. 2. 3. 4. | No 14 No No No No No No No No No No 38 No No No 47 |
| MipMode DeltaQP InterDir RefPic AffineFlag AffineType AffMergeIdx GBiIdx Mvd BDPCMMode QtRootCbf QtCbf | 49 50-52 53-57 58-59 60-62 63 64 65 66-67 68-69 70 71-82 | 1 3 5 2 3 1 1 1 2 2 1 12. | No 50-52 54 59 No 63 No No No No No 77-80 |
| SigCoeffGroup SigFlag ParFlag GtxFlag LastX LastY MVPIdx SmvdFlag SaoMergeFlag SaoTypeIdx TransquantBypassFlag LFNSTIdx | 83-90 91-180 182-212 213-276 277-305 306-334 335 336 337 338 339 340-341 | 10. 90 32 64 29 29 1 1 1 1 1 2 | 87-90 No 200,202, 209,210-12 232,234,241,242-44,274,275-76 291, 294,295-301,305 320,323,324-30,334 No No No 338 339 No |
| RdpcmFlag RdpcmDir MTSIndex ISPMode SbtFlag SbtQuadFlag SbtHorFlag SbtPosFlag CrossCompPred | 342-343 344-345 346-356 357-358 359-360 361 362-364 365 366-375 | 2 2 11 2 2 1 3 1 10 | 342-43 344-45 346,347,351,354-56 No No No 363 No 366-77 |
| ChromaQpAdjFlag ChromaQpAdjIdc ImvFlag ctbAlfFlag AlfUseLatestFilt AlfUseTemporalFilt MHIntraFlag TriangleFlag TriangleIdx | 376 377 378-381 382-390 391 392 393 394 395 | 1 1 4 9 1 1 1 1 1 | No No 380,381 382-83,385-86, 388,389 No No 394 395 |
| IBCFlag JointCbCrFlag TsSigCoeffGroup TsSigFlag TsParFlag TsGtxFlag TsResidualSign | 396-398 399 400-402 403-405 406 407-421 422-423 | 3 1 3 3 1 15 2 | 398 No 402 No No 412-21 No |

# Appendix B

Next table consists 56 of context models which don’t have any updates at all for the full CTC.

|  | Ctx_id | Number of ctx | Removed ctx |
| --- | --- | --- | --- |
| MultiRefLineIdx DeltaQP QtCbf | 36-38 50-52 71-82 | 3 3 12. | 38 50-52 77-80 |
| SigCoeffGroup LastX LastY TransquantBypassFlag | 83-90 277-305 306-334 339 | 10. 29 29 1 | 87-90 297-301,305 326-30,334 339 |
| RdpcmFlag RdpcmDir MTSIndex CrossCompPred | 342-343 344-345 346-356 366-375 | 2 2 11 10 | 342-43 344-45 346,351,355-56 366-77 |
| TriangleFlag TriangleIdx | 394 395 | 1 1 | 394 395 |
| TsGtxFlag | 407-421 | 15 | 412-21 |

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).