| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0555-v1 |
| --- | --- |

| Title: | Sub-pictures and sub-picture sets with level derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Lidong Xu | Email: | jill.boyce@intel.com lidong.xu@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

Support for sequence level sub-pictures and sub-picture sets is proposed. Sub-pictures are required to correspond to a rectangular region, and are independently decodable for the entire sequence. Tile and slices may not cross sub-picture boundaries. Sub-picture sets can be derived based on any combination of sub-pictures that form a rectangular region, and are determined by external means. Sub-picture parameters are signaled in the SPS, using a grid to indicate the sub-picture size and position. An SPS flag is proposed to indicate that the level for any sub-picture or sub-picture set can be derived based on scaling of the picture size relative to the sub-picture or sub-picture set sample size, respectively. An SEI message may alternatively be used to signal additional tier and level information for sub-pictures or sub-picture sets. A fixed length sub_pic_id syntax element is signaled early in the slice header.

# Introduction

At the Geneva meeting, many contributions related to sub-pictures were reviewed in the BoG on Coded Picture Regions (in JVET-N0754), including JVET-N0055, N0073, N0107, N0354, N0411, N0857, N0859.

A design goal of this contribution is to enable flexible sub-picture extraction and merging, while minimizing decoder burden. It aims to address the design goals of JVET-N0859, which are:

- Satisfying many of the MPEG systems requests for independently decodable and conforming “objects”

- Friendly for implementation by hardware decoders, so that sub-pictures can be supported in a Main profile of VVC

# Proposal

## Sub-picture definition

A sub-picture corresponds to a rectangular region of a coded picture. Sub-picture sizes may vary within a picture. Each particular sub-picture size and position is consistent for all pictures in the sequence.

Each sub-picture sequence shall be independent decodable. Tiles and slices (and CTBs) shall not span across sub-picture boundaries. It is the encoder’s responsibility to guarantee that each sub-picture is independently decodable. Semantic constraints on the bitstream will be needed to impose this constraint, but are not provided in this contribution.

Tile, slice, and brick arrangements within a sub-picture may vary for the pictures in a sequence.

## Sub-picture signaling

Sub-picture parameters are signaled in the SPS. A sub-picture mapping is signaled to associate sample positions in the picture with a sub-picture ID, based on a grid. The grid granularity is selected by the encoder and explicitly signaled, for the horizontal and vertical dimensions. Syntax in the SPS indicates the sub_pic_id for each grid position, similar to the signaling mechanism proposed in JVET-N0275 for signaling of sections of a picture.

The coded picture is split into a grid, with the number of rows and columns determined by the size of each grid elements and the coded picture size. For each (i, j)-th grid position, the sub_pic_id[i ][ j ] is signaled. There are no restrictions on the ordering of sub_pic_id values. However, a sub-picture must be a contiguous rectangle, so all grid samples with a particular sub_pic_id value must be contiguous and in a rectangle shape. For example, Figure 1(a) shows a valid example of the sub_pic_id values of a grid. Figure 1(b) shows an invalid example, because sub-picture 3 is not rectangle shaped. Figure 1(c) is also invalid, because sub-picture 0 is not contiguous.

| 0 | 0 | 3 | 3 |
| --- | --- | --- | --- |
| 2 | 1 | 3 | 3 |
| 4 | 4 | 3 | 3 |
| 4 | 4 | 5 | 5 |

Figure 1(a). valid example of sub_pic_id[ i ][ j ] grid values

| 0 | 0 | 0 | 3 |
| --- | --- | --- | --- |
| 2 | 1 | 3 | 3 |
| 4 | 4 | 3 | 3 |
| 4 | 4 | 5 | 5 |

Figure 1(b) invalid example of sub_pic_id[ i ][ j ] grid values

| 0 | 0 | 3 | 3 |
| --- | --- | --- | --- |
| 2 | 1 | 3 | 3 |
| 4 | 4 | 3 | 3 |
| 0 | 0 | 5 | 5 |

Figure 1(c) invalid example of sub_pic_id[ i ][ j ] grid values

The encoder is responsible for determining the grid element size. Using a finer grid granularity requires more signaling overhead, but offers more flexibility for varying the size of sub-pictures. The proposed minimum grid element size is 4x4, although other sizes could be used, including the min CTB size. The total overhead for sub-picture parameter signaling increases with the number of sub-pictures.

An advantage of the grid signaling approach is that it is very easy to change the sub-picture id assignment in a coded bitstream, e.g. for a merging or splitting process.

No change is proposed in this contribution for tile signaling, which is done in the PPS. It may be beneficial to revise the tile parameter signaling based on the sub-picture parameters present in the SPS, but this would may violate the restriction on independent parsing of parameter sets.

Several syntax element types in the SPS are converted to fixed length codes, to enable easier access to the sub-picture parameters in the SPS.

In the proposed syntax, the number of sub-pictures is limited to 256, with the highest value reserved to enable an extension if support for more sub-pictures is desired.

## Slice header signaling

A slice_sub_pic_id syntax element is proposed at the start of slice header, immediately following the slice PPS. The slice PPS is changed to be a fixed bit width field to enable easy access and/or modification to the slice_sub_pic_id syntax element.

It is suggested that the signaling and/or calculation of the slice_address or brick parameters could be modified to based on the values of sub_pic_id[ i ][ j ], SubPicTop[slice_sub_pic_id] and SubPicLeft[slice_sub_pic_id], but no solution is proposed in this contribution.

## Sub-picture sets

Sub-picture sets are made up of one or more sub-pictures. Sub-picture set parameters are not explicitly signaled within the SPS, but can be derived based on any combination of sub-pictures that form a rectangular region. External means are used to determine which sub-pictures are in the sub-picture set that a decoder is expected to decode.

Figure 2 below shows some examples. Fig 2(a) shows a picture split into several sub-pictures. Fig 2(b), 2(c), and 2(d) show examples of different sub-picture sets that could be formed from those sub-pictures.

An example usage is for 360 video coding using cube maps. For example, each of the 6 cube faces can be split into 4 regions, to make 24 sub-pictures. A decoder can select to decode and render only the sub-picture set containing the sub-pictures corresponding to the region that is being viewed.

3

2

4

0

1

Figure 2(a) Example breakdown of picture into sub-pictures

3

2

4

0

1

Figure 2(b) Example valid sub-picture set grouping

3

2

4

0

1

Figure 2(c) Example valid sub-picture set grouping

3

2

4

0

1

Figure 2(d) Example valid sub-picture set grouping

## Profile, tier, level for sub-picture sequences and sub-picture set sequences

All sub-picture sequences and sub-picture set sequences are required to conform to the same profile as the CVS.

An SPS flag is proposed to indicate that the level for all sub-picture sequences and sub-picture set sequences for the CVS can be derived based on ratio of the sub-picture or sub-picture set size, respectively, relative to the coded picture size. When this flag is set, the same tier is used for the sub-picture sequence and sub-picture set sequence as for the CVS.

More details about how the sub-picture or sub-picture set level is derived are provided in Section 4.

The encoder is responsible to ensure that all sub-pictures and all possible sub-picture set sequences conform to the derived tier and level when the flag is set, including all restrictions on bitrate, sample rate, etc.

## SEI message for explicit profile, tier, level signaling for sub-picture sequences and sub-picture set sequences

If additional level signaling is desired, an SEI message is proposed, which enables signaling of tier and level for any desired sub-pictures and sub picture sets. The proposed syntax differs from that used in the HEVC temporal motion-constrained tile sets SEI message. It is asserted that the proposed syntax is more easily modifiable when merged bitstreams are formed.

# Proposed syntax

The proposed syntax shows change marks with respect to JVET-N1001-v8.

## SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gra_enabled_flag | u(1) |
| sps_seq_parameter_set_id | u(4) |
| chroma_format_idc | u(3) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| sub_pics_present_flag | u(1) |
| if( sub_pics_present_flag ) { |  |
| sub_pic_derive_tier_level_flag | u(1) |
| max_sub_pics_minus1 | u(8) |
| sub_pic_grid_element_width_minus1 | u(v) |
| sub_pic_grid_element_height_minus1 | u(v) |
| SubPicGridCols = pic_width_in_luma_samples + sub_pic_grid_element_width_minus1* 4 + 3) / (sub_pic_grid_element_width_minus1* 4 + 4) |  |
| SubPicGridRows = pic_height_in_luma_samples + sub_pic_grid_element_height_minus1 * 4 + 3) / (sub_pic_grid_element_height_minus1*4 + 4 ) |  |
| for( i = 0; i < SubPicGridRows; i++ ) { |  |
| for( j = 0; j < SubPicGridCols; j++ ) { |  |
| sub_pic_id[ i ][ j ] | u(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |

sub_pics_present_flag equal to 1 indicates that sub-picture parameters are present in the syntax structure. sub_pics_present_flag equal to0 indicates that sub-picture parameters are not present in the syntax structure.

sub_pic_derive_tier_level_flag equal to 1 indicates that the tier and level indicators of all sub-pictures and sub-picture sets in the CVS can be derived according to sub-clause A.X.

max_sub_pics_minus1 plus 1 specifies the maximum number of sub-pictures that may be present in the CVS. max_sub_pics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

sub_pic_grid_element_width_minus1 specifies the width of each element of the sub-picture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_width_in_luma_samples / 4) ) bits.

sub_pic_grid_element_height_minus1 specifies the width of each element of the sub-picture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_height_in_luma_samples / 4) ) bits.

sub_pic_id[ i ][ j ] specifies the sub-picture identifier of the (i, j)-th grid position. The length of the syntax element is Ceil( Log2( max_sub_pics_minus1 + 1 ) ) bits.

It is a requirement of bitstream conformance that

if (i > 0 && j > 0)

if (sub_pic_id[ i ][ j ] = = sub_pic_id[ i – 1][ j ] && (sub_pic_id[ i ][ j ] = = sub_pic_id[ i ][ j – 1])

sub_pic_id[ i ][ j ] shall equal sub_pic_id[ i – 1][ j – 1]

The variables SubPicTop[sub_pic_id [ i ][ j ] ], SubPicLeft[sub_pic_id [ i ][ j ] ], are derived as follows.

if ( i == 0)

SubPicTop[sub_pic_id[ i ][ j ]] = 0

else if (sub_pic_id[ i ][ j ] != sub_pic_id[ i - 1][ j ] )

SubPicTop[sub_pic_id[ i ][ j ]] = i

if ( j == 0)

SubPicLeft[sub_pic_id[ i ][ j ]] = 0

else if (sub_pic_id[ i ][ j ] != sub_pic_id[ i ][ j - 1 ] )

SubPicLeft[sub_pic_id[ i ][ j ]] = j

## Slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | u(5) |
| if( sub_pics_present_flag ) |  |
| slice_sub_pic_id | u(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| … |  |

slice_sub_pic_id specifies the sub-picture identifier of the slice. The length of the syntax element is Ceil( Log2( max_sub_pics_minus1 + 1 ) ) bits.

## SEI

| sub_picture_ptl( ) { | Descriptor |
| --- | --- |
| sp_id_len | u(3) |
| sp_tier_present_flag | u(1) |
| num_sub_pic_params_minus1 | u(8) |
| for (i = 0; i <= num_sub_pics_params_minus1; i++) { |  |
| sp_sub_pic_id | u(v) |
| if ( sp_tier_present_flag) |  |
| sp_tier_idc[sp_sub_pic_id] | u(1) |
| sp_level_idc[sp_sub_pic_id] | u(8) |
| } |  |
| num_sub_pic_sets_params | u(8) |
| for (i = 0; i < num_sub_pic_sets_params; i++) { |  |
| sp_sub_pic_set_id | u(8) |
| for (j = 0; i < sp_max_sub_pics_minus1; i++) |  |
| sp_sub_pic_id_in_set[sp_sub_pic_set_id][ j ] | u(8) |
| sp_set_level_idc[sp_sub_pic_set_id] | u(v) |
| if ( sp_tier_present_flag) | u(8) |
| sp_set_tier_idc[sp_sub_pic_set_id] | u(v) |
| } |  |
| } |  |

sp_id_len specifies the number of bits used to signal the sp_sub_pic_id syntax element.

sp_tier_present_flag equal to 1 specifies that the 	sp_tier_idc[sp_sub_pic_id] and sp_set_tier_idc[sp_sub_pic_set_id] syntax elements are present in the syntax structure. sp_tier_present_flag equal to 0 specifies that the 	sp_tier_idc[sp_sub_pic_id] and sp_set_tier_idc[sp_sub_pic_set_id] syntax elements are not present in the syntax structure.

num_sub_pic_params_minus1 + 1 specifies the number of sub-pictures for which parameters are present in the syntax structure.

sp_sub_pic_id specifies the identifier of the sub-picture for the signaled parameters.

sp_tier_idc[sp_sub_pic_id] specifies a level  context for the interpretation of level context for the interpretation of sp_level_idc[ sp_sub_pic_id]  as specified in Annex A. When not present, the value of sp_tier_idc[ sp_sub_pic_id] is inferred to be equal to tier_idc for the active SPS.

sp_level_idc[ sp_sub_pic_id]  indicates a level to which the sp_sub_pic_id-th sub-bitstream sequence conforms as specified in Annex A. When not present, the value of sp_level_idc[ sp_sub_pic_id] is inferred to be equal to level_idc for the active SPS.

num_sub_pic_sets_params specifies the number of sub-picture sets for which parameters are present in the syntax structure.

sp_sub_pic_set_id specifies the identifier of the sub-picture set for the signaled parameters.

sp_sub_pic_id_in_set[sp_sub_pic_set_id][ j ] equal to 1 specifies that the j-th sub-picture is included in the sp_sub_pic_set_id-th sub-picture set. sp_sub_pic_id_in_set[sp_sub_pic_set_id][ j ] equal to 0 specifies that the j-th sub-picture is not included in the sp-sub_pic_set_id-th sub-picture set.

sp_set_tier_idc[sp_sub_pic_set_id] specifies a level  context for the interpretation of level context for the interpretation of sp_level_idc[ sp_sub_pic_id]  as specified in Annex A. When not present, the value of sp_tier_idc[ sp_sub_pic_set_id] is inferred to be equal to tier_idc for the active SPS.

sp_set_level_idc[ sp_sub_pic_set_id]  indicates a level to which the sp_sub_pic_id-th sub-bitstream sequence conforms as specified in Annex A. When not present, the value of sp_set_level_idc[ sp_sub_pic_set_id] is inferred to be equal to level_idc for the active SPS.

# Sub-picture extraction process

The existing sub-bitstream extraction process can be updated to include a sub-picture set extraction process. A constraint would need to be imposed that the extracted sub-picture set sequence is a valid decodable bitstream.

External means are expected to provide a list of the sub-picture ID values included in the sub-picture set, applicable for the entire CVS, to be used in the sub-picture extraction process. The sub-picture set must form a rectangle.

# Level derivation

The VVC draft specification in JVET-N1001 lacks a description of how level restrictions are imposed.

This section derives a level value for sub-picture sets based on the level limits of HEVC Annex A. In HEVC Annex A, the general_level_idc syntax element value is 30 * the Level value indicated in table A.6.

The level for an individual sub-picture is derived by deriving the level for a sub-picture set containing only the individual sub-picture.

The derivation is based on scaling the max luma sample rate for the CVS’s level_idc by the ratio of the sub-picture set size to the picture size. It is based on the assumption that the frame rate for the sub-pictures in the sequence is the same as for the pictures in the sequence.

The text section of Annex A which imposes level limits on CVSes, including bitrate limits, which is not yet available, will need to apply those restrictions to extracted sub-picture set sequences. It is the encoder’s responsibility to ensure that those constraints are obeyed by all sub-pictures and all possible sub-picture sets.

## Derivation of sub-picture set sequence level indicator

The set of sub-pictures included in the sub-picture set is provided via external means.

The size of the n-th sub-picture in units of grid samples, SubPicNumGridSamples[ n ], for n in 0 .. max_sub_pics_minus1, is derived as follows.

SubPicNumGridSamples[ n ] = 0

for (i = 0; i <  SubPicGridRows; i ++)

for (j = 0; i <  SubPicGridCols; j ++)

if (n = sub_pic_id[ i ][ j ])

SubPicNumGridSamples[ n ]++

The size of the sub-picture set in units of grid samples, SubPicSetNumGridSamples, is derived as follows.

SubPicSetNumGridSamples = 0

for (n = 0; n <= max_sub_pics_minus1; n ++)

if (sub-picture n is in the sub-picture set)

SubPicSetNumGridSamples += SubPicNumGridSamples[ n ]

The size of the sub-picture set in units of luma samples, SubPicSetSizeY, is set equal to SubPicSetNumGridSamples * (sub_pic_grid_element_width_minus1 + 1) *  (sub_pic_grid_element_height_minus1 + 1)  * 16

The values of  MaxLumaPs[LevelIdc ] and MaxLumaSr[ LevelIdc ] are taken from Table A.X.For values of LevelIdc not in Table A.X, MaxLumaPs[LevelIdc]   = 0 and MaxLumaSr LevelIdc]  = 0.

Table A.X Level limits

| Level | LevelIdc | Max luma picture size MaxLumaPs[LevelIdc] (samples) | Max luma sample rate MaxLumaSr LevelIdc] (samples/sec) |
| --- | --- | --- | --- |
| 1 | 30 | 36 864 | 552 960 |
| 2 | 60 | 122 880 | 3 686 400 |
| 2.1 | 63 | 245 760 | 7 372 800 |
| 3 | 90 | 552 960 | 16 588 800 |
| 3.1 | 93 | 983 040 | 33 177 600 |
| 4 | 120 | 2 228 224 | 66 846 720 |
| 4.1 | 123 | 2 228 224 | 133 693 440 |
| 5 | 150 | 8 912 896 | 267 386 880 |
| 5.1 | 153 | 8 912 896 | 534 773 760 |
| 5.2 | 156 | 8 912 896 | 1 069 547 520 |
| 6 | 180 | 35 651 584 | 1 069 547 520 |
| 6.1 | 183 | 35 651 584 | 2 139 095 040 |
| 6.2 | 186 | 35 651 584 | 4 278 190 080 |

The value of the sub-picture set sequence level indicator,  SubPicSetLevelIdc, is derived as follows.

SubPicSetLevelIdc = level_idc

for (k = level_idc; k > 0; k --)

if (  SubPicSetSizeY <= MaxLumaPs[ k ]  &&

MaxLumaSr[ level_idc ]*SubPicSetSizeY / PicSizeInSamplesY <= MaxLumaSr[ k ] )

SubPicSetLevelIdc = k

# Bit count calculations

For 6 equally sized sub-pictures, such as for cube faces, in a 4Kx2K picture, there would be 47 additional bits in the SPS, and 3 additional bits per slice for the sub-picture id plus up to 4 additional bits for fixed-length coding the slice PPS.

For 24 equally sized sub-pictures in a 4Kx2K picture, there would be 149 additional bits in the SPS and 5 additional bits per slice for the sub-picture id plus up to 4 additional bits for fixed-length coding the slice PPS.  .

For 96 equally sized sub-pictures in a 4Kx2K picture, there would be 701 additional bits in the SPS and 7 additional bits per slice for the sub-picture id plus up to 4 additional bits for fixed-length coding the slice PPS.

Table 1. SPS bit counts for proposed sub-picture parameters

| Syntax elements | 6 sub-pictures | 24 sub-pictures | 96 sub-pictures |
| --- | --- | --- | --- |
| sub_pics_present_flag | 1 | 1 | 1 |
| sub_pic_derive_tier_level_flag | 1 | 1 | 1 |
| max_sub_pics_minus1 | 8 | 8 | 8 |
| sub_pic_grid_element_width_minus1 | 10 | 10 | 10 |
| sub_pic_grid_element_height_minus1 | 9 | 9 | 9 |
| sub_pic_id | 6*3=18 | 24*5=120 | 96*7=672 |
| Total | 47 | 149 | 701 |

# Discussion

Using explicit signaling of sub-pic ID in a grid in the SPS allows flexibility when extracting or combining bitstreams, as it places no restrictions on the order of sub-picture ID values within a picture. Use of u(v) coding allows for easy rewriting, if desired. Use of a grid pattern with signaled granularity allows flexibility of sub-picture sizes within a picture.

For small number of sub-pictures, the proposed grid signaling using u(v) coding is bitrate efficient compared to other possibly sub-picture signaling approaches, but for large numbers of sub-pictures may be less efficient. The proposed signaling is also efficient for variable sized sub-pictures, especially when all sub-pictures sizes are multiples of the smallest sub-picture size.

Derivation of levels for sub-picture and sub-picture sets is very bitrate efficient, and enables arbitrary rectangular sub-picture sets to be formed. A burden is placed on the encoder to guarantee that the bitrate constraints on arbitrary sub-picture sets are obeyed, but this burden can be met by imposing bitrate constraints on individual sub-pictures. If encoders wish to enable more variation in bitrate across sub-pictures, the proposed SEI message can be used for signaling of tier and level for sub-pictures and identified sub-picture sets.

It may be advantageous to modify the slice, tile, and brick parameter signaling to be local within a sub-picture, but is not proposed within this contribution. Tile parameter signaling is in the PPS, while sub-picture signaling is proposed for the SPS, which would preclude the ability to use the sub-picture parameters in the tile signaling, unless the restriction against cross-parameter set parsing is eliminated, as proposed in JVET-O0113 and JVET-M0133.

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).