| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0557 |
| --- | --- |

| Title: | Non-CE7: Context modeling unification in significant flag of coefficient group |  |
| --- | --- | --- |
| Status: | Input document to JVET |  |
| Purpose: | Proposal |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Muhammed Coban , Marta Karczewicz (Qualcomm) | Tel: Email: yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |

_____________________________

##### Derivation process of ctxInc for the syntax element coded_sub_block_flag

Inputs to this process are the colour component index cIdx, the current sub-block scan location ( xS, yS ), the previously decoded bins of the syntax element coded_sub_block_flag and the binary logarithm of the transform block width log2TbWidth and the transform block height log2TbHeight.

Output of this process is the variable ctxInc.

The variable csbfCtx is derived using the current location ( xS, yS ), two previously decoded bins of the syntax element coded_sub_block_flag in scan order, log2TbWidth and log2TbHeight, as follows:

- The variables log2SbWidth and log2SbHeight are dervied as follows:

log2SbWidth = ( Min( log2TbWidth, log2TbHeight ) < 2  ?  1  :  2 )	(9-36)

log2SbHeight = log2SbWidth		(9-37)

- The variables log2SbWidth and log2SbHeight are modified as follows:

- If log2TbWidth is less than 2 and cIdx is equal to 0, the following applies

log2SbWidth = log2TbWidth		(9-38)

log2SbHeight = 4 − log2SbWidth		(9-39)

- Otherwise, if log2TbHeight is less than 2 and cIdx is equal to 0, the following applies

log2SbHeight = log2TbHeight		(9-40)

log2SbWidth = 4 − log2SbHeight		(9-41)

- The variable csbfCtx is initialized with 0 and modified as follows:

- If transform_skip_flag[ xS ][ yS ] is equal to 1, the following applies:

- When xS is greater than 0, csbfCtx is modified as follows:

csbfCtx  +=  coded_sub_block_flag[ xS − 1 ][ yS ]	(9-42)

- When yS is greater than 0, csbfCtx is modified as follows:

csbfCtx  +=  coded_sub_block_flag[ xS ][ yS − 1 ]	(9-43)

- Otherwise (transform_skip_flag[ xS ][ yS ] is equal to 0), the following applies:

- When xS is less than ( 1  <<  ( log2TbWidth − log2SbWidth ) ) − 1, csbfCtx is modified as follows:

csbfCtx  +=  coded_sub_block_flag[ xS + 1 ][ yS ]	(9-44)

- When yS is less than ( 1  <<  ( log2TbHeight − log2SbHeight ) ) − 1, csbfCtx is modified as follows:

csbfCtx  +=  coded_sub_block_flag[ xS ][ yS + 1 ]	(9-45)

The context index increment ctxInc is derived using the colour component index cIdx and csbfCtx as follows:

- If cIdx is equal to 0, ctxInc is derived as follows:

- If transform_skip_flag[ xS ][ yS ] is equal to 1, the following applies:

ctxInc = 4 + Min( csbfCtx, 1 )		(9-46)

- Otherwise (transform_skip_flag[ xS ][ yS ] is equal to 0), the following applies:

ctxInc = Min( csbfCtx, 1 )		(9-47)

- Otherwise (cIdx is greater than 0), ctxInc is derived as follows:

#### ctxInc = 2 + Min( csbfCtx, 1 )

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.