| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0557 |
| --- | --- |

| Title: | Non-CE7: Context modeling unification in significant flag of coefficient group |  |
| --- | --- | --- |
| Status: | Input document to JVET |  |
| Purpose: | Proposal |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Muhammed Coban , Marta Karczewicz (Qualcomm) | Tel: Email: yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |

_____________________________

# Abstract

In this proposal, it is proposed to unify the context modeling of significant coefficient group (CG) flag, syntax element coded_sub_block_flag, in regular coefficient coding and transform skip residual coding. The unification shows minor difference versus VTM5.0: Class F: 0.01% AI, -0.04% RA, -0.01% LB BD-Rate, and Class SCC: 0.03% AI, -0.08% RA, -0.10% LB BD-Rate.

# Proposal

In the context modeling of coded_sub_block_flag in regular and transform skip residual coding in VTM5.0, context index is derived based on coded_sub_block_flag in the neighboring CGs, i.e., CGs on the right and below for regular residual coding and CGs on the top and left for transform skip residual coding. Specifically, define the current sub-block scan location as ( xS, yS ), subWidth and subHeight as the number of subblocks horizontally and vertically inside a transform unit, the context index ctxInc for regular residual coding is derived as follows for each color component:

ctxInc =  max (coded_sub_block_flag[ xS + 1 ][ yS ] + coded_sub_block_flag[ xS ][ yS + 1 ], 1)

For transform skip residual coding, however, the context modeling is slightly different from regular residual coding and the context index is derived as follows:

ctxInc =  coded_sub_block_flag[ xS - 1 ][ yS ] + coded_sub_block_flag[ xS ][ yS - 1 ]

In this proposal, it is proposed to unify the context modeling of coded_sub_block_flag in transform skip residual coding with the one in regular residual coding. The context index for TS coded_sub_block_flag  after the proposed unification is derived as:

ctxInc =  max ( coded_sub_block_flag[ xS - 1 ][ yS ] + coded_sub_block_flag[ xS ][ yS - 1 ], 1)

# Simulation results

The proposed method is implemented on top of VTM5.0 and is evaluated according to the CE7 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. IBC, HashME, and RDPCM are turned on only for class F and class TGM. Simulation results for standard QPs (22, 27, 32, 37) are shown in Table 1.

Table 1: Results of proposed unification in coded_sub_block_flag

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.02% | 100% | 101% |
| Class A2 | 0.00% | 0.01% | -0.02% | 100% | 102% |
| Class B | 0.00% | -0.03% | 0.01% | 100% | 100% |
| Class C | -0.01% | 0.02% | 0.03% | 100% | 98% |
| Class E | 0.00% | 0.02% | 0.00% | 99% | 101% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.07% | -0.01% | 97% | 93% |
| Class F | 0.01% | -0.01% | -0.05% | 100% | 100% |
| Class TGM | 0.03% | 0.01% | -0.02% | 101% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | -0.01% | 101% | 102% |
| Class A2 | 0.02% | 0.11% | 0.06% | 100% | 101% |
| Class B | -0.01% | 0.07% | -0.06% | 101% | 102% |
| Class C | -0.03% | -0.10% | -0.06% | 103% | 111% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | -0.03% | 101% | 104% |
| Class D | -0.02% | 0.00% | 0.17% | 97% | 93% |
| Class F | -0.04% | 0.09% | 0.03% | 101% | 101% |
| Class TGM | -0.08% | -0.10% | -0.05% | 101% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.28% | -0.29% | 103% | 106% |
| Class C | 0.00% | 0.31% | -0.12% | 102% | 110% |
| Class E | 0.05% | -0.13% | 0.00% | 101% | 108% |
| Overall | 0.01% | 0.19% | -0.16% | 102% | 108% |
| Class D | 0.00% | -0.09% | 0.27% | 97% | 99% |
| Class F | -0.01% | 0.01% | -0.35% | 101% | 105% |
| Class TGM | -0.10% | -0.13% | -0.08% | 101% | 105% |

# Reference

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding”, JVET-N1027, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.