| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0558 |
| --- | --- |

| Title: | CE7-related: Reduction of Gtx passes in transform skip residual coding |  |
| --- | --- | --- |
| Status: | Input document to JVET |  |
| Purpose: | Proposal |  |
| Author(s) or Contact(s): | Marta Karczewicz, Yung-Hsuan Chao, Hongtao Wang, Muhammed Coban (Qualcomm) | Tel: Email: martak@qti.qualcomm.com yunghsua@qti.qualcomm.com hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |

_____________________________

# Abstract

In this proposal, it is proposed to simplify the transform skip residual coding in JVET-O0122 by reducing the number of passes in coding the absolute coefficient levels, i.e. syntax element abs_level_gtx_flag. By reducing the number of passes for abs_level_gtx_flag from 5 to 2 in CE7-3.10 (sign context modeling and level mapping), the results show Class F: -0.53% AI, -0.46% RA, -0.30% LB BD-Rate, and Class SCC: -0.60% AI, -0.52% RA, -0.41% LB BD-Rate, versus the VTM-5.0 anchor under CTC. Compared to CE7-3.10, the proposed scheme show: Class F: 0.23% AI, 0.13% RA, 0.08% LB BD-Rate, and Class SCC: 0.66% AI, 0.27% RA, 0.26% LB BD-Rate.

# Proposal

In document JVET-O0122 CE7-3.10 [1], a sign context modeling and level mapping scheme utilizing the information from quntized neiboring (left and top) residual sample is proposed on top of the transform scheme residual coding in [2]. In the transform skip residual coding scheme proposed in [2], absolute coefficient level absCoeffLevel are coded using sig_coeff_flag, abs_level_gtX_flags, where X = 1,2,..5,  par_level_flag, abs_remainder. On the decoder side, reconstructed transform coefficient for sig_coeff_flag equal to 1, are derived by:

absCoeffLevel = 1 + abs_level_gt1_flag + par_level_flag + 2 * (abs_level_gt2_flag + abs_level_gt3_flag+…+abs_level_gt5_flag) + 2*abs_remainder

syntax elements sig_coeff_flag, abs_level_gt1_flag, and par_level_flag are coded interleaved in the first pass. Starting from second pass, elements abs_level_gtX_flags, where X = 2, 3 … 5, are coded bitplane by bitplane. In this proposal, it is proposed to reduce the abs_level_gtX_flags (X = 2, 3, ..5 ) passes from 4 to 1 on top of JVET-O0122.

# Simulation results

The proposed method is implemented on top of CE7-3.10 (with sign context modeling and level mapping) in JVET-O0122 and is evaluated according to the CE7 description [3] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. All tests, including anchors and test, include ticket #330 bugfix. IBC, HashME, and RDPCM are turned on only for class F and class TGM. Simulation results for standard QPs (22, 27, 32, 37) are shown in Table 1.

Table 1: Results of proposed method on top of CE7-3.10 (#330 bugfix)

|  |  |  | All Intra |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0 (ticket#330 bugfix) |  |  |  |  |  |  |  |  |  |  |  |  |  | Over CE7-3.10 (ticket#330 bugfix) |  |  |  |  |  |  |  |
|  | Y |  |  | U |  | V |  | EncT |  | DecT |  | Y |  | U |  | V |  |  | EncT |  | DecT |  |
| Class A1 | -0.01% |  |  | -0.02% |  | -0.01% |  | 103% |  | 102% |  | 0.00% |  | -0.01% |  | 0.00% |  |  | 99% |  | 101% |  |
| Class A2 | -0.01% |  |  | -0.01% |  | -0.05% |  | 101% |  | 101% |  | 0.01% |  | -0.02% |  | -0.04% |  |  | 100% |  | 99% |  |
| Class B | -0.02% |  |  | -0.04% |  | 0.01% |  | 102% |  | 97% |  | 0.00% |  | -0.03% |  | 0.03% |  |  | 102% |  | 93% |  |
| Class C | -0.03% |  |  | -0.11% |  | -0.15% |  | 102% |  | 112% |  | 0.01% |  | -0.02% |  | -0.13% |  |  | 101% |  | 109% |  |
| Class E | -0.05% |  |  | -0.06% |  | 0.05% |  | 102% |  | 111% |  | 0.01% |  | 0.05% |  | 0.05% |  |  | 101% |  | 107% |  |
| Overall | -0.02% |  |  | -0.05% |  | -0.03% |  | 102% |  | 104% |  | 0.01% |  | -0.01% |  | -0.02% |  |  | 101% |  | 101% |  |
| Class D | -0.08% |  |  | 0.18% |  | 0.09% |  | 104% |  | 92% |  | 0.00% |  | 0.07% |  | 0.09% |  |  | 102% |  | 91% |  |
| Class F | -0.53% |  |  | -0.53% |  | -0.63% |  | 98% |  | 92% |  | 0.23% |  | 0.24% |  | 0.10% |  |  | 97% |  | 91% |  |
| TGM | -0.60% |  |  | -0.60% |  | -0.56% |  | 103% |  | 89% |  | 0.66% |  | 0.58% |  | 0.60% |  |  | 99% |  | 89% |  |
|  |  |  | Random Access |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM5.0 (ticket#330 bugfix) |  |  |  |  |  |  |  |  |  |  |  |  |  | Over CE7-3.10 (ticket#330 bugfix) |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | c | V | EncT | DecT |
| Class A1 | -0.01% |  |  | 0.00% |  | 0.01% |  | 104% |  | 101% |  | -0.01% |  | -0.02% |  | 0.00% |  |  | 104% |  | 103% |  |
| Class A2 | 0.01% |  |  | -0.06% |  | 0.08% |  | 100% |  | 100% |  | -0.03% |  | -0.07% |  | -0.01% |  |  | 98% |  | 98% |  |
| Class B | -0.01% |  |  | -0.07% |  | 0.15% |  | 101% |  | 93% |  | 0.01% |  | -0.05% |  | -0.03% |  |  | 101% |  | 95% |  |
| Class C | -0.04% |  |  | -0.05% |  | -0.12% |  | 102% |  | 102% |  | 0.02% |  | 0.12% |  | -0.04% |  |  | 102% |  | 104% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% |  |  | -0.05% |  | 0.03% |  | 102% |  | 98% |  | 0.00% |  | 0.00% |  | -0.02% |  |  | 101% |  | 100% |  |
| Class D | 0.01% |  |  | -0.16% |  | 0.07% |  | 108% |  | 93% |  | 0.02% |  | -0.29% |  | -0.20% |  |  | 107% |  | 93% |  |
| Class F | -0.46% |  |  | -0.31% |  | -0.27% |  | 102% |  | 94% |  | 0.13% |  | 0.26% |  | 0.16% |  |  | 100% |  | 93% |  |
| TGM | -0.52% |  |  | -0.42% |  | -0.46% |  | 101% |  | 90% |  | 0.27% |  | 0.26% |  | 0.31% |  |  | 98% |  | 93% |  |
|  |  |  | Low delay B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM5.0 (ticket#330 bugfix) |  |  |  |  |  |  |  |  |  |  |  |  |  | Over CE7-3.10 (ticket#330 bugfix) |  |  |  |  |  |  |  |
|  | Y |  |  | U |  | V |  | EncT |  | DecT |  | Y |  | U |  | V |  |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.02% |  |  | 0.06% |  | 0.21% |  | 102% |  | 99% |  | 0.03% |  | 0.16% |  | 0.26% |  |  | 102% |  | 102% |  |
| Class C | 0.05% |  |  | -0.30% |  | -0.12% |  | 101% |  | 100% |  | 0.02% |  | -0.22% |  | 0.12% |  |  | 101% |  | 106% |  |
| Class E | -0.22% |  |  | 0.33% |  | 0.47% |  | 103% |  | 100% |  | -0.07% |  | -0.14% |  | 0.30% |  |  | 102% |  | 107% |  |
| Overall | -0.05% |  |  | 0.00% |  | 0.17% |  | 102% |  | 100% |  | 0.00% |  | -0.04% |  | 0.23% |  |  | 102% |  | 104% |  |
| Class D | 0.03% | 0.29% | 0.33% | 104% | 88% | 0.04% | -0.19% | 0.21% | 104% | 88% |
| Class F | -0.30% |  |  | -0.23% |  | 0.08% |  | 99% |  | 91% |  | 0.08% |  | -0.02% |  | 0.68% |  |  | 98% |  | 98% |  |
| TGM | -0.41% |  |  | -0.33% |  | -0.39% |  | 100% |  | 98% |  | 0.26% |  | 0.41% |  | 0.39% |  |  | 101% |  | 103% |  |

# Additional test results

In Table 2, the results of the proposed method on top of VTM5.0 are provided. The test is evaluated according to the CE7 description [3] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. All tests, including anchors and test, include ticket #330 bugfix. IBC, HashME, and RDPCM are turned on only for class F and class TGM.

Table 2: Results of proposed method on top of VTM5.0 (with ticket #330 bugfix)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 (ticket#330 bugfix) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.00% | 99% | 97% |
| Class A2 | 0.00% | 0.03% | -0.01% | 101% | 100% |
| Class B | 0.00% | 0.01% | -0.04% | 99% | 106% |
| Class C | 0.00% | -0.03% | -0.03% | 99% | 108% |
| Class E | 0.01% | 0.04% | 0.13% | 100% | 106% |
| Overall | 0.00% | 0.01% | 0.00% | 99% | 104% |
| Class D | 0.01% | 0.06% | -0.09% | 98% | 108% |
| Class F | 0.30% | 0.14% | 0.11% | 100% | 107% |
| TGM | 0.89% | 0.74% | 0.77% | 99% | 105% |
|  | Random Access |  |  |  |  |
|  | Over VTM5.0 (ticket#330 bugfix) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.00% | 99% | 101% |
| Class A2 | 0.01% | 0.00% | -0.11% | 100% | 102% |
| Class B | -0.01% | -0.07% | 0.14% | 100% | 109% |
| Class C | -0.06% | -0.05% | -0.03% | 100% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.04% | 0.02% | 100% | 105% |
| Class D | 0.05% | 0.17% | 0.04% | 100% | 106% |
| Class F | 0.15% | 0.42% | 0.11% | 100% | 105% |
| TGM | 0.52% | 0.56% | 0.50% | 100% | 106% |
|  | Low delay B |  |  |  |  |
|  | Over VTM5.0 (ticket#330 bugfix) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.08% | -0.10% | 103% | 97% |
| Class C | 0.03% | -0.33% | -0.33% | 100% | 108% |
| Class E | -0.14% | 0.43% | -0.19% | 101% | 110% |
| Overall | -0.03% | 0.03% | -0.20% | 101% | 104% |
| Class D | 0.04% | 0.44% | -0.20% | 100% | 105% |
| Class F | 0.21% | 0.52% | 0.95% | 101% | 109% |
| TGM | 0.32% | 0.48% | 0.47% | 102% | 97% |

# Reference

- M. Karczewicz, H. Wang, M. Coban, Y.-H. Chao, Y. Han, “CE7: Sign context, level mapping, and bitplane coding for TS residual coding (CE7-3.7, CE7-3.8, CE7-3.9, CE7-3.10, and CE7-3.11)”, JVET document JVET-O0122, Gothenburg, SE, July 2019.

- B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip”, JVET document JVET-M0464, Marrackech, MA, Jan 2019.

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding”, JVET-N1027, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).