| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0560 |
| --- | --- |

| Title: | Merge Mode Syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Limin Wang and Krit Panusopone | Tel: Email: | +1 858-335-9196 limin.2.wang@nokia.com krit.panusopone@nokia.com |
| Source: | Nokia, USA |  |  |

_____________________________

# Abstract

This contribution proposes possible changes for merge mode syntax. Option 1 removes the last condition check in merge_date(), option 2 moves regular merge mode to the end of merge_data(), and option 3 adds regular merge mode as the last merge mode.

# Introduction

Merge mode syntax in the current JVET / VVC specification is broken. Below are the partial syntax table of coding_unit() and the full syntax table of merge_data() from the latest VVC specificaton JVET N1001-v8 [1].

#### Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| ……. |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| general_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( general_merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| } else if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| ……. |  |
| } |  |

#### Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag ¦ ¦ cbWidth * cbHeight != 32 ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag && cbWidth * cbHeight != 32 ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

As seen, if the flag general_merge_flag[x0][y0] for a curent CU at (x0,y0) in coding_unit() is on (or 1), the function of merge_date() will be called for the current CU and a set of checks will be performed, as follows,

- Check if the current CU is in IBC mode,

  - If yes, decode the current CU in IBC mode,

  - If not, check if the current CU is in regular merge mode,

    - If yes, decode the current CU in regular merge mode,

    - If not, check if the current CU is in mmvd mode,

      - If yes, decode the current CU in mmvd mode,

      - If not, check if the current CU is in subblock merge mode,

        - If yes, decode the current CU in subblock merge mode,

        - If not, check if the current CU is in ciip mode,

          - If yes, decode the current CU in ciip mode

          - If not, check if the current CU is in triangle mode,

            - If yes, decoder the current CU in triangle mode,

            - If not, no more check, meaning no code mode for the current CU.

The value of MergeTriangleFlag[ x0 ][ y0 ] in the last conditon check depends upon the following nine conditions:

- sps_triangle_enabled_flag = 1,

- slice_type = B,

- general_merge_flag[ x0 ][ y0 ] = 1,

- MaxNumTriangleMergeCand > 2,

- cbWidth * cbHeight is greater than or equal to 64,

- regular_merge_flag[ x0 ][ y0 ] = 0,

- mmvd_merge_flag[ x0 ][ y0 ] = 0,

- merge_subblock_flag[ x0 ][ y0 ] = 0, and

- ciip_flag[ x0 ][ y0 ] = 0.

If any of the above conditions is set differently, the value of MergeTriangleFlag[ x0 ][ y0 ] will be 0. If the outcome of the last condition check is false (or 0), there will be no more check for the current CU, implying no code mode is available for the current CU and the code will be broken.

# Proposal

To fix the specification, a few possible options are proposed as follows.

Option 1: remove the last condition check of MergeTriangleFlag[ x0 ][ y0 ]. That is, if all the other merge mode flags are not on (or set to 0), the current CU has to be coded in triangle mode.

The following shows the syntax table of merge_date() with the possible changes in red.

#### Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag ¦ ¦ cbWidth * cbHeight != 32 ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag && cbWidth * cbHeight != 32 ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) { |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

The semantics needs to indicate that if the flag general_merge_flag[x0][y0] for a curent CU at (x0,y0) is on (or 1), one of merge modes has to be implemented for the current CU. Possible changes are as follows.

=====================================

general_merge_flag[ x0 ][ y0 ] specifies whether the inter prediction parameters for the current coding unit are inferred from a neighbouring inter-predicted partition. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When general_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If cu_skip_flag[ x0 ][ y0 ] is equal to 1, general_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1.

- Otherwise, general_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

If general_merge_flag[ x0 ][ y0 ] is equal to 1, one of the following coding modes has to be true for the current coding unit,

- IBC

- Regular merge

- mmvd

- subblock merge

- ciip

- triangle

=======================================

Option 2: move the regular merge mode to the end of merge mode checks, implying that the regular merge mode is the last, or default, merge mode option. If all the special merge mode flags are not on (or set to 0), the current CU will be decoded in regualr merge mode. For this option, merge_triangle_flag[ x0 ][ y0 ] needs to be added.

The following shows the possible syntax table of merge_data(). Note that the ciip mode check of ciip_flag[ x0 ][ y0 ] = = 0 for triangle mode has been removed.

#### Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| ciip_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| ciip_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( sps_triangle_enabled_flag && slice_type = = B && MaxNumTriangleMergeCand > 2 && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Semantics for merge_triangle_flag[ x0 ][ y0 ] is as follows.

=========================================

merge_triangle_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, when decoding a B slice, triangular shape based motion compensation is used to generate the prediction samples of the current coding unit. merge_triangle_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not predicted by triangular shape based motion compensation. When merge_triangle_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

==========================================

Option 3. add regular merge mode to the end of merge mode checks as a default merge mode, which means if all the merge mode flags are not on (or set to 0), the current CU has to be decoded in regular merge mode. For this option, merge_triangle_flag[ x0 ][ y0 ] needs to be added.

The following shows the syntax table of merge_data() with the possible changes in red.

#### Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag ¦ ¦ cbWidth * cbHeight != 32 ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag && cbWidth * cbHeight != 32 ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) { |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| if( sps_triangle_enabled_flag && slice_type = = B && MaxNumTriangleMergeCand > 2 && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Semantics for merge_triangle_flag[ x0 ][ y0 ] is as follows.

=========================================

merge_triangle_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, when decoding a B slice, triangular shape based motion compensation is used to generate the prediction samples of the current coding unit. merge_triangle_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not predicted by triangular shape based motion compensation. When merge_triangle_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

==========================================

# Conclusions

This contribution proposes three possible options to fix merge mode syntax. Option 1 removes the last condition check of MergeTriangleFlag[ x0 ][ y0 ], option 2 moves regular merge mode to the end of merge mode checks, and option 3 adds regular merge mode as the last possible merge mode.

# References

[1] B. Bross, J. Chen and S. Liu, “Versatile Video Coding”, JVET- JVET-N1001-v8, March 2019.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).