| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0563-v1 |
| --- | --- |

| Title: | Non-CE8: disabling TPM blending |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Li Zhang, Jizheng Xu, Kai Zhang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

It is proposed to disable the blending process in TPM based on a flag signaled at the slice level in this document. When the blending process is disabled, prediction samples for the two partitions will be directly copied from their own motion compensated blocks. The samples located on the diagonal line or anti-diagonal line will be determined by comparing the motion vectors of two partitions. Motion storage for TPM is modified so that only one uni-directional MV is stored for each 4x4 block. Simulation results report that the proposed scheme can achieve -0.83% and -0.84% BD bitrate changes compared with VTM-5.0 on class TGM. On 4:4:4 sequences, the proposed scheme -0.85% and -1.11% BD bitrate changes compared with VTM-5.0 on TGM 1080p.

# Introduction

Triangle prediction mode (TPM) was adopted into VVC [1] to handle a block with complex texture. There is a blending process in the TPM. Weighted bi-prediction is applied to generate prediction samples of the 7 lines in the luma block and 3 lines in the chroma block, as depicted in Figure 1.

The following weights are used in the blending process:

- {7/8, 6/8, 5/8, 4/8, 3/8, 2/8, 1/8} for luma blocks and {6/8, 4/8, 2/8} for chroma blocks.

Figure 1. Weights used in the blending process

However, blending two prediction signals maybe inefficient in coding efficiency with unnecessary complexity for screen contents. To address this issue, this contribution proposes to replace the blending process with directly copying in TPM mode. In addition, we also propose to simplify the TPM motion storage.

# Proposed Methods

The proposed method replaces the blending process with directly copying and simplifies the motion storage in TPM, when the current slice is determined as screen contents. And a slice level flag is signalled to indicate whether the proposed method is enabled. The determination of the value of such a slice level flag is same as the hash-based fractional MMVD decision for non-4K sequences.

## Disabling blending in TPM

When the proposed method is enabled, the two partitions as shown in the Figure 2 will copy its own prediction samples. The prediction values of samples located on the diagonal or anti-diagonal line are determined by comparing the motion vectors (MVs) of two partitions. Their values will copy from the partition with a smaller MV. Here, the magnitude of a MV is defined as its L-1 norm, i.e. if |MV1|1>|MV0|1, MV0 is smaller. When they have the same L-1 norm, we simply select MV0.

Figure 2. Diagonal and anti-diagonal partitions in TPM

## Modified motion storage for TPM

We also propose to modify the motion storage for the TPM mode when the current tile group is considered as screen contents. In this method, the stored motion information will be from the partition with a smaller motion vector, as defined in 2.1. In addition, as shown in Figure 3, this method is only applied to 4x4 units that contain diagonal or anti-diagonal samples.

Figure 3. Motion storage of the proposed scheme

# Simulation Results

The proposed method is implemented on top of CE8-1.5a and evaluated with VTM-5.0. The simulation follows the CE8 test condition [2]. Summarized results can be found in the following tables. The anchor is VTM-5.0 and the test is the proposed method.

2 sets of simulations have been launched, as follows:

1) Test #1: Test conditions of 4:2:0 sequences in CE8.

2) Test #2: Test conditions of 4:4:4 sequences in CE8.

Table 1. Test #1 RA Results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.03% | 0.04% | 0.11% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class B | 0.02% | 0.04% | 0.04% | 100% | 101% |
| Class C | 0.01% | 0.01% | 0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.02% | 0.04% | 101% | 101% |
| Class D | 0.04% | 0.03% | 0.03% | 100% | 119% |
| Class F | -0.02% | 0.09% | 0.05% | 100% | 106% |
| TGM | -0.83% | -0.61% | -0.58% | 100% | 100% |

Table 2. Test #1 LDB Results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.02% | 0.02% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.02% | 0.02% | 0.02% | 100% | 104% |
| Overall | 0.01% | 0.01% | 0.01% | 100% | 101% |
| Class D | 0.01% | 0.01% | 0.01% | 100% | 103% |
| Class F | -0.05% | 0.34% | 0.16% | 102% | 95% |
| TGM | -0.84% | -0.70% | -0.68% | 99% | 100% |

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.02% | 0.02% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.02% | 0.02% | 0.02% | 100% | 104% |
| Overall | 0.01% | 0.01% | 0.01% | 100% | 101% |
| Class D | 0.01% | 0.01% | 0.01% | 100% | 103% |
| Class F | -0.05% | 0.34% | 0.16% | 102% | 95% |
| TGM | -0.84% | -0.70% | -0.68% | 99% | 100% |

Table 3. Test #2 RA Results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.85% | -0.66% | -0.70% | 100% | 101% |
| TGM 720p | -0.09% | -0.08% | -0.07% | 99% | 104% |
| Animation | 0.01% | 0.01% | 0.01% | 100% | 103% |
| Mixed content | -0.21% | -0.20% | -0.21% | 98% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.29% | -0.24% | -0.25% | 99% | 102% |

Table 4. Test #2 LDB Results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -1.11% | -1.00% | -1.02% | 98% | 99% |
| TGM 720p | -0.02% | -0.37% | -0.30% | 99% | 101% |
| Animation | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Mixed content | -0.10% | -0.13% | -0.03% | 97% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | -0.32% | -0.39% | -0.36% | 99% | 100% |

# Conclusions

This document proposes to change the TPM blending process based on a signaled flag at the slice level. Simulation results report that the proposed scheme can -0.85% and -1.11% BD-rate changes compared with VTM-5.0 on TGM 1080p. We recommend adopting the proposed method to the next version of VVC draft.

# Text Changes

The changes on top of the draft provided by JVET- N1001-v7 are highlighted in grey. Deleted texts are marked with strikethrough.

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| sps_mmvd_enabled_flag | u(1) |
| … |  |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| sps_tpm_blending_off_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| … |  |
| } |  |

7.3.5 Slice header syntax

7.3.5.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| … |  |
| if ( slice_type != I ) { |  |
| … |  |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| … |  |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if(sps_tpm_blending_off _flag) |  |
| slice_tpm_blending_off _flag | u(1) |
| slice_qp_delta | se(v) |
| … |  |
| } |  |

sps_tpm_blending_off _flag equal to 1 specifies that bledning process is replaced by directly copying. When sps_tpm_blending_off _flag is not present, it is inferred to be equal to 0.

slice_tpm_blending_off _flag equal to 1 specifies that bledning process is replaced by directly copying. When slice_scc_flag is not present, it is inferred to be equal to 0.

8.5.7 Decoding process for triangle inter blocks

8.5.7.1 General

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB,

- the chroma motion vectors mvCA and mvCB,

- the reference indices refIdxA and refIdxB,

- the prediction list flags predListFlagA and predListFlagB.

Outputs of this process are:

- an (cbWidth)x(cbHeight) array predSamplesL of luma prediction samples,

- an (cbWidth / 2)x(cbHeight / 2) array predSamplesCb of chroma prediction samples for the component Cb,

- an (cbWidth / 2)x(cbHeight / 2) array predSamplesCr of chroma prediction samples for the component Cr.

Let predSamplesLAL and predSamplesLBL be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesLACb, predSamplesLBCb, predSamplesLACr and predSamplesLBCr be (cbWidth / 2)x(cbHeight / 2) arrays of predicted chroma sample values.

The predSamplesL, predSamplesCb and predSamplesCr are derived by the following ordered steps:

- For N being each of A and B, the following applies:

- The reference picture consisting of an ordered two-dimensional array refPicLNL of luma samples and two ordered two-dimensional arrays refPicLNCb and refPicLNCr of chroma samples is derived by invoking the process specified in clause 8.5.6.2 with X set equal to predListFlagN and refIdxX set equal to refIdxN as input.

- The array predSamplesLNL is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the luma coding block width sbWidth set equal to cbWidth, the luma coding block height sbHeight set equal to cbHeight, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvN and the reference array refPicLXL set equal to refPicLNL, the variable bdofFlag set euqal to FALSE, and the variable cIdx is set equal to 0 as inputs.

- The array predSamplesLNCb is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding block width sbWidth set equal to cbWidth / 2, the coding block height sbHeight set equal to cbHeight / 2, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvCN, and the reference array refPicLXCb set equal to refPicLNCb, the variable bdofFlag set euqal to FALSE, and the variable cIdx is set equal to 1 as inputs.

- The array predSamplesLNCr is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding block width sbWidth set equal to cbWidth / 2, the coding block height sbHeight set equal to cbHeight / 2, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvCN, and the reference array refPicLXCr set equal to refPicLNCr, the variable bdofFlag set euqal to FALSE, and the variable cIdx is set equal to 2 as inputs.

- The partition direction of merge triangle mode variable triangleDir is set equal to merge_triangle_split_dir[ xCb ][ yCb ].

- The prediction samples inside the current luma coding block, predSamplesL[ xL ][ yL ] with xL = 0..cbWidth − 1 and yL = 0..cbHeight − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2 with the coding block width nCbW set equal to cbWidth, the coding block height nCbH set equal to cbHeight, the sample arrays predSamplesLAL and predSamplesLBL, the MVa and MVb set equal to mvA and mvB and the variables triangleDir, and cIdx equal to 0 as inputs.

- The prediction samples inside the current chroma component Cb coding block, predSamplesCb[ xC ][ yC ] with xC = 0..cbWidth / 2 − 1 and yC = 0..cbHeight / 2 − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2 with the coding block width nCbW set equal to cbWidth / 2, the coding block height nCbH set equal to cbHeight / 2, the sample arrays predSamplesLACb and predSamplesLBCb, and the MVa and MVb set equal to mvCA and mvCB, and the variables triangleDir, and cIdx equal to 1 as inputs.

- The prediction samples inside the current chroma component Cr coding block, predSamplesCr[ xC ][ yC ] with xC = 0..cbWidth / 2 − 1 and yC = 0..cbHeight / 2 − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2 with the coding block width nCbW set equal to cbWidth / 2, the coding block height nCbH set equal to cbHeight / 2, the sample arrays predSamplesLACr and predSamplesLBCr, the MVa and MVb set equal to mvCA and mvCB, and the variables triangleDir, and cIdx equal to 2 as inputs.

- The motion vector storing process for merge triangle mode specified in clause 8.5.7.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight, the partition direction triangleDir, the luma motion vectors mvA and mvB, the reference indices refIdxA and refIdxB, and the prediction list flags predListFlagA and predListFlagB as inputs.

8.5.7.2 Weighted sample prediction process for triangle merge mode

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesLA and predSamplesLB,

- a variable triangleDir specifying the partition direction,

- a variable cIdx specifying colour component index.

- a variable MVa specifying the motion vector of the prediction block A

- a variable MVb specifying the motion vector of the prediction block B

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

The variable nCbR is derived as follows:

nCbR = ( nCbW > nCbH )  ?  ( nCbW / nCbH )  :  ( nCbH / nCbW )	(8-820)

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

Variables shift1 and offset1 are derived as follows:

–	The variable shift1 is set equal to Max( 5, 17 − bitDepth).

–	The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

Depending on the values of triangleDir, wS and cIdx, the prediction samples pbSamples[ x ][ y ] with x = 0..nCbW − 1 and y = 0..nCbH − 1 are derived as follows:

–	The variable wIdx is derived as follows:

- If cIdx is equal to 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( x / nCbR − y ) + 4 ) )	(8-821)
	: ( Clip3( 0, 8, ( x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is equal to 0 and triangleDir is equal to 1, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( nCbH − 1 − x / nCbR − y ) + 4 ) ) 	(8-822)
		( Clip3( 0, 8, ( nCbW − 1 − x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is greater than 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( x / nCbR − y ) + 2 ) )	(8-823)
	: ( Clip3( 0, 4, ( x − y / nCbR ) + 2 ) )

- Otherwise (if cIdx is greater than 0 and triangleDir is equal to 1), the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( nCbH − 1 − x / nCbR − y ) + 2 ) ) 	(8-824)
		( Clip3( 0, 4, ( nCbW − 1 − x − y / nCbR ) + 2 ) )

–	The variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows:

if slice_tpm_blending_off _flag is equal to 1, the followings apply:

The variable wOffset is derived as

wOffset = (Max(Abs(MVa[0]),Abs(MVa[1])) <= Max(Abs(MVb[0]),Abs(MVb[1]))) ? 0.5 : -0.5

wValue = ( cIdx == 0 ) ? Clip3(0, 8, (wIdx+wOffset-4)*16) : Clip3(0, 8, (wIdx + wOffset-2)*16)

Otherwise,

wValue = ( cIdx = = 0 )  ?  Clip3( 0, 8, wIdx )  :  Clip3( 0, 8, wIdx * 2 )	(8-825)

–	The prediction sample values are derived as follows

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesLA[ x ][ y ] * wValue + 
		predSamplesLB[ x ][ y ] * ( 8 − wValue ) + offset1 ) >> shift1 ) 	(8-826)

#### 8.5.7.3 Motion vector storing process for triangle merge mode

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable triangleDir specifying the partition direction,

- the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB,

- the reference indices refIdxA and refIdxB,

- the prediction list flags predListFlagA and predListFlagB.

The variables numSbX and numSbY specifying the number of 4x4 blocks in the current coding block in horizontal and vertical direction are set equal to numSbX = cbWidth >> 2 and numSbY = cbHeight >> 2.

The variable minSb is set equal to min( numSbX, numSbY ).

The variable cbRatio is derived as follows:

cbRatio = ( cbWidth > cbHeight ) ? ( cbWidth / cbHeight ) : ( cbHeight / cbWidth )	(8-812)

The variable refIdxTempA is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with X set equal to predListFlagA and refIdxN set equal to refIdxA as inputs.

The variable refIdxTempB is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with the X set equal to predListFlagB and refIdxN set equal to refIdxB as inputs.

For each 4x4 subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the following applies:

- The variables xIdx and yIdx are derived as follows:

xIdx = ( cbWidth > cbHeight )  ?  ( xSbIdx / cbRatio ) : xSbIdx	(8-813)

yIdx = ( cbWidth > cbHeight )  ?  ySbIdx  :  ( ySbIdx / cbRatio )	(8-814)

- The variable sType is derived as follows:

- If triangleDir is equal to 0, the following applies:

sType = ( xIdx = = yIdx )  ?  2  :  ( ( xIdx > yIdx )  ?  0  :  1 )	(8-815)

- Otherwise (triangleDir is equal to 1), the following applies:

sType = ( xIdx + yIdx = = minSb )  ?  2  :  ( ( xIdx + yIdx < minSb )  ?  0  :  1 )	(8-816)

- The varibale storeFromA is derived as follows

If slice_tpm_blending_off _flag is equal to 1,

storeFromA = (Max(Abs(MVa[0]),Abs(MVa[1])) <= Max(Abs(MVb[0]),Abs(MVb[1])) ? 1 : 0

otherwise

storeFromA = -1

- Depending on the value of sType, the following assignments are made:

- If sType is equal to 0, or storeFromA is euqal to 1, the following applies:

predFlagL0 = ( predListFlagA = = 0 )  ?  1  :  0		(8-817)

predFlagL1 = ( predListFlagA = = 0 )  ?  0  :  1		(8-818)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA : −1	(8-819)

refIdxL1 = ( predListFlagA = = 0 )  ?  −1  :  refIdxA	(8-820)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  0	(8-821)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  0	(8-822)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 0 ]	(8-823)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 1 ]	(8-824)

- Otherwise, if sType is equal to 1, or storeFromA is euqal to 0, the following applies:

predFlagL0 = ( predListFlagB = = 0 )  ?  1  :  0		(8-825)

predFlagL1 = ( predListFlagB = = 0 )  ?  0  :  1		(8-826)

refIdxL0 = ( predListFlagB = = 0 )  ?  refIdxB : −1	(8-827)

refIdxL1 = ( predListFlagB = = 0 )  ?  −1  :  refIdxB	(8-828)

mvL0[ 0 ] = ( predListFlagB = = 0 )  ?  mvB[ 0 ]  :  0	(8-829)

mvL0[ 1 ] = ( predListFlagB = = 0 )  ?  mvB[ 1 ]  :  0	(8-830)

mvL1[ 0 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 0 ]	(8-831)

mvL1[ 1 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 1 ]	(8-832)

- Otherwise (sType is equal to 2), the following applies:

…

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Mar. 2019

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-N1028, Mar. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).