| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0564-v1 |
| --- | --- |

| Title: | CE8-related: Signalling of chroma IBC prediction modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Li Zhang, Jizheng Xu, Kai Zhang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

It is proposed to code the IBC flag for chroma CU only when the collocated luma block is coded with IBC mode. Accordingly, the signalling cost of IBC prediction modes on chroma block can be saved if the center block of the collocated luma block is not coded with IBC mode. Simulation results reports that the proposed method can achieve -0.36%/-0.16%/0.01% BD-rate changes on TGM sequences under AI/RA/LDB configurations. In addition, the proposed method can achieve -0.84%/-0.38%/0.00% BD-rate changes under AI/RA/LDB configurations on class TGM 1080p 4:4:4 sequences.

# Introduction

In the current VVC, the motion compensation in the chroma IBC mode is performed at sub-block level. The chroma block will be partitioned into 2x2 sub-blocks. Each sub-block determines whether the corresponding luma block has a block vector and the validity if it is present. There is encoder constrain in the current VTM, where the chroma IBC mode will be tested if all sub-blocks in the current chroma CU have corresponding valid luma block vectors. For example, on a YUV 4:2:0 video, the chroma block is NxM and then the collocated luma region is 2Nx2M. The sub block size of a chroma block is 2x2. There are several steps to perform the chroma mv derivation then the block copy process.

First, the chroma block will be first partitioned into (N >> 1)*(M >> 1) sub blocks. Seond, Each sub block with a top left sample coordinated at (x, y) fetches the corresponding luma block covering the same top-left sample which is coordinated at (2x, 2y). Then the encoder checks the block vector (bv) of the fetched luma block. If one of the following three conditions is satisfied, the bv is considered as invalid. The first condition is the bv of the corresponding luma block is not existing.The second condition is the prediction block identified by a bv is not reconstructed yet. The third condition is the prediction block identified by a bv is partially or fully overlapped with the current block. At last, The chroma motion vector of a sub-block is set to the motion vector of the corresponding luma block.

The IBC mode is allowed at the encoder when all sub-blocks find a valid bv.

CE8-1.5a derives the motion information of Chroma IBC mode by fetching motion information of the collocated luma block. Sample copying could be applied only when the corresponding luma bv is also valid for the chroma block. When there is no corresponding luma bv, currently a chroma IBC flag is still signalled. It is thus redundant.

# The proposed method

In this document, we propose to modify the signalling of prediction modes of intra block copy (IBC) on chroma blocks when dual tree is enabled. In the proposed method, when coding a chroma CU, the chroma IBC is enabled only when the collocated luma CU is coded with IBC mode. Accordingly, the signalling cost of IBC prediction modes on chroma block can be saved if the collocated luma block is not coded with IBC mode. The collocated luma block of a chroma block is shown in Figure 1.

Figure 1. The collocated luma block of a chroma block

# Simulation Results

The proposed scheme is implemented on top of VTM-5.0 and CE8-1.5a [2] respectively then evaluated with VTM-5.0. The simulation follows the CE 8 test condition [2].

Four sets of simulations have been tested, as follows:

1) Test #1: Test conditions of 4:2:0 sequences in CE8.

Anchor: VTM-5.0

Test: The proposed method

2) Test #2: Test conditions of 4:4:4 sequences in CE8.

Anchor: VTM-5.0

Test: The proposed method

3) Test #3: Test conditions of 4:2:0 sequences in CE8.

Anchor: VTM-5.0

Test: The proposed method based on CE8-1.5a

4) Test #4: Test conditions of 4:4:4 sequences in CE8.

Anchor: VTM-5.0

Test: The proposed method based on CE8-1.5a

Results are summarized in the following tables.

Table 1. Test #1 AI results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.05% | 0.03% | 0.00% | 141% | 102% |
| Class A2 | -0.81% | -1.04% | -0.98% | 147% | 100% |
| Class B | -0.80% | -0.76% | -0.84% | 155% | 102% |
| Class C | -0.29% | -0.38% | -0.24% | 156% | 108% |
| Class E | -1.26% | -1.19% | -1.26% | 159% | 107% |
| Overall | -0.64% | -0.66% | -0.66% | 152% | 104% |
| Class D | -0.57% | -0.56% | -0.72% | 149% | 111% |
| Class F | -0.05% | -0.06% | 0.05% | 102% | 106% |
| TGM | -0.18% | -0.18% | -0.21% | 101% | 101% |

Table 2. Test #1 RA results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.11% | -0.22% | 0.17% | 108% | 102% |
| Class A2 | -0.10% | -0.13% | -0.20% | 108% | 102% |
| Class B | -0.13% | -0.42% | -0.58% | 110% | 104% |
| Class C | 0.14% | -0.26% | -0.11% | 114% | 109% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.28% | -0.23% | 110% | 104% |
| Class D | 0.10% | -0.52% | 0.27% | 113% | 116% |
| Class F | -0.02% | -0.02% | 0.17% | 102% | 108% |
| TGM | -0.06% | -0.03% | -0.09% | 103% | 105% |

Table 3. Test #1 LDB results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.28% | -0.40% | 120% | 99% |
| Class C | 0.11% | 0.28% | -0.21% | 126% | 105% |
| Class E | 0.08% | -0.59% | 0.15% | 116% | 106% |
| Overall | 0.07% | 0.06% | -0.20% | 121% | 103% |
| Class D | 0.08% | -0.39% | 0.76% | 127% | 109% |
| Class F | -0.01% | 0.32% | -0.06% | 103% | 103% |
| TGM | -0.06% | -0.15% | -0.15% | 102% | 102% |

Table 4. Test #2 AI results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.71% | -0.75% | -0.73% | 101% | 103% |
| TGM 720p | -0.41% | -0.44% | -0.43% | 101% | 104% |
| Animation | -0.05% | -0.07% | 0.00% | 102% | 103% |
| Mixed content | -0.25% | -0.29% | -0.32% | 102% | 101% |
| Camera-Captured content | 0.02% | -0.03% | -0.04% | 102% | 104% |
| Overall | -0.35% | -0.39% | -0.38% | 101% | 103% |

Table 5. Test #2 RA results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.38% | -0.33% | -0.38% | 103% | 104% |
| TGM 720p | -0.21% | -0.28% | -0.23% | 102% | 108% |
| Animation | -0.06% | 0.04% | 0.06% | 102% | 104% |
| Mixed content | -0.19% | -0.23% | -0.22% | 102% | 103% |
| Camera-Captured content | -0.02% | 0.01% | -0.01% | 103% | 102% |
| Overall | -0.21% | -0.20% | -0.20% | 102% | 105% |

Table 6. Test #2 LDB results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.06% | 0.07% | 0.06% | 102% | 100% |
| TGM 720p | 0.22% | 0.02% | 0.17% | 104% | 100% |
| Animation | -0.01% | 0.00% | 0.14% | 102% | 100% |
| Mixed content | -0.08% | 0.06% | 0.03% | 103% | 101% |
| Camera-Captured content | 0.01% | -0.05% | -0.21% | 103% | 104% |
| Overall | 0.06% | 0.03% | 0.06% | 103% | 101% |

Table 7. Test#3 AI results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.05% | -0.01% | 0.00% | 141% | 101% |
| Class A2 | -0.85% | -1.12% | -1.05% | 146% | 100% |
| Class B | -0.81% | -0.89% | -1.06% | 155% | 100% |
| Class C | -0.32% | -0.45% | -0.28% | 164% | 115% |
| Class E | -1.25% | -1.32% | -1.39% | 165% | 112% |
| Overall | -0.66% | -0.75% | -0.76% | 154% | 105% |
| Class D | -0.57% | -0.60% | -0.71% | 149% | 105% |
| Class F | -0.09% | -0.12% | -0.07% | 105% | 103% |
| TGM | -0.36% | -0.42% | -0.45% | 102% | 98% |

Table 8. Test#3 RA results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.11% | -0.23% | 0.12% | 106% | 101% |
| Class A2 | -0.09% | -0.21% | -0.16% | 106% | 100% |
| Class B | -0.13% | -0.53% | -0.76% | 108% | 101% |
| Class C | 0.17% | -0.27% | -0.19% | 118% | 112% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.34% | -0.31% | 110% | 104% |
| Class D | 0.09% | -0.43% | 0.14% | 113% | 109% |
| Class F | -0.04% | -0.04% | -0.02% | 105% | 106% |
| TGM | -0.16% | -0.17% | -0.21% | 104% | 102% |

Table 9. Test#3 LDB results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.21% | 0.21% | 120% | 98% |
| Class C | 0.13% | 0.08% | -0.07% | 128% | 98% |
| Class E | 0.11% | -0.59% | 0.04% | 118% | 110% |
| Overall | 0.08% | -0.04% | 0.07% | 122% | 101% |
| Class D | 0.10% | -0.25% | 0.66% | 127% | 103% |
| Class F | -0.02% | 0.03% | 0.01% | 103% | 98% |
| TGM | 0.01% | -0.08% | 0.00% | 100% | 100% |

Table 10. Test#4 AI results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.84% | -0.98% | -1.01% | 102% | 100% |
| TGM 720p | -0.41% | -0.64% | -0.59% | 101% | 100% |
| Animation | -0.08% | -0.06% | -0.07% | 103% | 98% |
| Mixed content | -0.36% | -0.48% | -0.48% | 101% | 99% |
| Camera-Captured content | 0.01% | -0.02% | -0.03% | 102% | 100% |
| Overall | -0.42% | -0.54% | -0.54% | 102% | 99% |

Table 11. Test#4 RA results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.38% | -0.33% | -0.35% | 101% | 100% |
| TGM 720p | -0.27% | -0.45% | -0.40% | 101% | 101% |
| Animation | -0.06% | 0.01% | 0.01% | 100% | 100% |
| Mixed content | -0.20% | -0.35% | -0.26% | 99% | 100% |
| Camera-Captured content | -0.01% | 0.00% | -0.09% | 100% | 99% |
| Overall | -0.22% | -0.28% | -0.26% | 100% | 100% |

Table 12. Test#4 LDB results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.00% | -0.04% | -0.03% | 101% | 97% |
| TGM 720p | 0.33% | -0.34% | -0.25% | 101% | 100% |
| Animation | 0.01% | -0.01% | 0.03% | 101% | 97% |
| Mixed content | -0.14% | -0.25% | -0.06% | 100% | 99% |
| Camera-Captured content | 0.02% | -0.07% | -0.20% | 102% | 101% |
| Overall | 0.06% | -0.16% | -0.11% | 101% | 99% |

# Conclusion

This document proposed a redundancy removal on chroma IBC mode signalling, where the chroma IBC is only enabled when the collocated CU is coded with IBC as well. We recommend adopting the proposed method into next version of VVC draft.

# Text changes

The text changes, marked in grey, are on top of JVET-N1001-v8 [2].

#### 7.3.7.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( cbWidth = = 4 && cbHeight = = 4 && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ¦ ¦ (isLumaIbc[(x0+(cbWidth>>1))*SubWidthC][(y0+(cbHeight>>1))*SubHeightC] && treeType = = DUAL_TREE_CHROMA) ) && sps_ibc_enabled_flag && ( cbWidth != 128 ¦ ¦ cbHeight != 128 ) ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| … |  |
| } |  |

pred_mode_ibc_flag equal to 1 specifies that the current coding unit is coded in IBC prediction mode. pred_mode_ibc_flag equal to 0 specifies that the current coding unit is not coded in IBC prediction mode.

When pred_mode_ibc_flag is not present, it is inferred as follows:

- If cu_skip_flag[ x0 ][ y0 ] is equal to 1, and cbWidth is equal to 4, and cbHeight is equal to 4, pred_mode_ibc_flag is inferred to be equal 1.

- Otherwise, if both cbWidth and cbHeight are equal to 128 or treeType is equal to DUAL_TREE_CHROMA, pred_mode_ibc_flag is inferred to be equal to 0.

- Otherwise, pred_mode_ibc_flag is infered to be equal to the value of sps_ibc_enabled_flag when decoding an I slice, and 0 when decoding a P or B slice, respectively.

7.4.8.5 Coding unit semantics

..

pred_mode_ibc_flag equal to 1 specifies that the current coding unit is coded in IBC prediction mode. pred_mode_ibc_flag equal to 0 specifies that the current coding unit is not coded in IBC prediction mode.

When pred_mode_ibc_flag is not present, it is inferred as follows:

- If cu_skip_flag[ x0 ][ y0 ] is equal to 1, and cbWidth is equal to 4, and cbHeight is equal to 4, pred_mode_ibc_flag is inferred to be equal 1.

- Otherwise, if both cbWidth and cbHeight are equal to 128, pred_mode_ibc_flag is inferred to be equal to 0.

- Otherwise, pred_mode_ibc_flag is infered to be equal to the value of sps_ibc_enabled_flag when decoding an I slice, and 0 when decoding a P or B slice, respectively.

When pred_mode_ibc_flag is equal to 1, the variable CuPredMode[ x ][ y ] is set to be equal to MODE_IBC for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

When treeType is equal to DUAL_TREE_LUMA, the variable isLumaIbc[ x ][ y] is set to be equal to pred_mode_ibc_flag for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

# References

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-N1028, Mar. 2019

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Mar. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).