| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0567 |
| --- | --- |

| Title: | Non-CE4: Cleanups on MVD ranges |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Hongbin Liu, Li Zhang, Kai Zhang, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to fix the following problems in VVC draft 5,

- The MVD derived by affine AMVR with 1/16-pel resolution may exceed the current MVD range (e.g., 16-bit in VVC draft).

- The MvdL1 of SMVD-coded blocks may exceed the current valid MVD range at corner cases.

- In the VTM-5.0 reference software, a reduced range of MVD constraint is applied, which does not align with the current VVC working draft.

In this contribution, two methods are proposed addressing the problems. In method #1, it is proposed to constrain the signaled MVD according to the MVD resolution. In method #2, it is proposed to constrain the internal MVD instead of the signaled MVD. With method #1 or #2, the MVD out-of-range problem in SMVD is fixed and the reference software is aligned with the working draft. It is reported that the proposed method provides almost the same performance as the anchor in terms of both BD-rate and running time.

# MVD ranges in VVC draft 5

There are three problems in VVC working draft 5 [1].

1) In the VVC spec, there is a bitstream constraint that the value of a signaled motion vector difference (MVD) shall be in the range of −215 to 215 − 1, inclusive. However, due to the adaptive motion vector resolution (AMVR) for affine motion prediction, the signaled MVD could be in the 1/16-pel precision. And hence the dynamic ranges of signaled MVDs could be [−217, 217 − 1].

2) In the VVC spec, if the decoded MvdL0 is equal to −215, then the MvdL1 of a SMVD-coded block would be set to 215, which exceeds the current valid MVD range.

3) In the VVC spec, the signaled MVD (e.g., could be in resolutions of 1/16-pel, 1/4-pel, 1-pel, or 4-pel) would be shifted to the internal 1/16-pel precision (e.g., 18 bits) after parsing the AMVR flag. However, the VTM-5.0 software limits the internal 1/16-pel precision MVD to the 16-bit range, which erroneously forbids potential large MVD values within the valid range as specified in the current VVC working draft.

# Proposed solution

To solve the above problems, this contribution proposes two methods to constrain the MVD values. Both methods allow a 1/16-pel precision MVD value being represented in the18-bit range, align the software and working draft, and avoid the case of out of range MVD caused by SMVD mirroring.

## Method #1

The constraint on the signaled MVD values is modified to be dependent on the MVD resolution.

## Method #2

Instead of constraining the signaled MVD, it is proposed to add a constraint on the internal MVD values to 18 bits with all MVDs in the same precision (e.g., the signaled MVD values are shifted to internal 18-bit MVD values according to the AMVR flags).

# Proposed spec changes

The proposed changes on top of VVC draft 5 [1] are described as below. Newly added parts are highlighted in yellow, and the deleted parts from [1] are highlighted in green strikethrough.

## Method #1

#### Motion vector difference semantics

The motion vector difference lMvd[ compIdx ] for compIdx = 0..1 is derived as follows:

lMvd[ compIdx ] = abs_mvd_greater0_flag[ compIdx ] *
	( abs_mvd_minus2[ compIdx ] + 2 ) * ( 1 − 2 * mvd_sign_flag[ compIdx ] )	(7-114)

The value of lMvd[ compIdx ] shall be in the range of −215 to 215 − 1, inclusive.

The value of lMvd[ compIdx ] shall be in the range of –(1<<log2MvdRange) + sym_mvd_flag[ x0 ][ y0 ] to (1<<log2MvdRange) − 1, inclusive, where log2MvdRange is derived as below:

| MVD resolution | log2MvdRange |
| --- | --- |
| 1/16-pel | 17 |
| 1/4-pel | 15 |
| 1-pel | 13 |
| 4-pel | 11 |

## Method #2

#### Motion vector difference semantics

The motion vector difference lMvd[ compIdx ] for compIdx = 0..1 is derived as follows:

lMvd[ compIdx ] = abs_mvd_greater0_flag[ compIdx ] *
	( abs_mvd_minus2[ compIdx ] + 2 ) * ( 1 − 2 * mvd_sign_flag[ compIdx ] )	(7-114)

The value of lMvd[ compIdx ] shall be in the range of −215 to 215 − 1, inclusive.

#### Coding unit semantics

…

The motion vector differences are modified as follows:

- If inter_affine_flag[ x0 ][ y0 ] is equal to 0, the variable MvShift is derived and the variables MvdL0[ x0 ][ y0 ][ 0 ], MvdL0[ x0 ][ y0 ][ 1 ], MvdL1[ x0 ][ y0 ][ 0 ], MvdL1[ x0 ][ y0 ][ 1 ] are modified as follows:

…

- Otherwise (inter_affine_flag[ x0 ][ y0 ] is equal to 1), the variable MvShift is derived and the variables MvdCpL0[ x0 ][ y0 ][ 0 ][ 0 ], MvdCpL0[ x0 ][ y0 ][ 0 ][ 1 ], MvdCpL0[ x0 ][ y0 ][ 1 ][ 0 ], MvdCpL0[ x0 ][ y0 ][ 1 ][ 1 ], MvdCpL0[ x0 ][ y0 ][ 2 ][ 0 ] and MvdCpL0[ x0 ][ y0 ][ 2 ][ 1 ] are modified as follows:

…

The value of MvdLX[ x0 ][ y0 ][ idx ], MvdCpLX[ x0 ][ y0 ][ctrlP][idx], with X being 0 or 1, idx being 0 or 1, ctrlP being 0, or 1, or 2, shall be in the range of −217 to 217 − 1, inclusive.

# Simulation results

The proposed method was implemented based on the VTM-5.0 reference software [2]. Simulation results of Method #1 and Method #2 based on the common test conditions [3] are provided in Table 1 and Table 2, respectively. It can be found that these two methods provide identical BD-Rate results.

Table 1: Simulation results of the proposed method #1

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | -0.02% | -0.02% | 100% | 101% |

Table 2: Simulation results of the proposed method #2

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Low Delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | -0.02% | -0.02% | 100% | 97% |

# Conclusions

In this contribution, two methods were proposed to fix the MVD range constraint from 16-bit to 18-bit, which are motion vector resolution dependent. This allows potential large MVD values and fix the syntax bug of SMVD in VVC draft. Simulation results show that the proposed fixes provide similar performance as VTM-5.0. It is recommended to adopt method #1 or #2 into VVC.

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, March 2019, Geneva, CH.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, CH.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).