| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0568-v1 |
| --- | --- |

| Title: | Non-CE8: An Alternative VPDU Memory Reuse for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This proposal presents an alternative way to reuse on-chip VPDU memory for IBC. When CTU size is 128x128, two most recently coded VPDUs in the even-number VPDU row and one most recently coded VPDU in the odd-number VPDU row are kept as reference for IBC. Compared with VTM-5.0, BD-rate changes for class F and 4:2:0 TGM are: AI: -0.78%/-1.82%; RA: -0.55%/-1.09%; LDB: -0.32%/-0.71% with almost the same running time.

# Introduction

In the current VVC design, VPDU memory reuse is applied to define the IBC reference area to reduce on-chip memory. Depending on the location of current VPDU, the reference area has different shapes, shown in Figure 1.

Figure 1. Reference area definition in the current design.

In addition, for each IBC coded block, whether the BV is valid or not needs to be checked. More specifically, suppose that CTU size is 128x128 and a WxH block at (X, Y) related to the upper-left of the picture has a block vector (xBv, yBv). Then the reference block’s four corners can be calculated as

- refLeftX = X + xBv

- refRightX = refLeftX + W - 1

- refTopY = Y + yBv

- refBottomY = refTopY + H – 1

In the current VTM implementation, there are five groups of condition checks, i.e.

- If a reference block is outside of picture boundary.

  - refLeftX >=0

  - refRightX < picWidth

  - refTopY >=0

  - refBottomY < picHeight

- If a reference block is outside of current CTU row.

  - (refToY >> 7) >= (Y>>7)

  - (refBottom >> 7) <= (Y>>7)

- If a reference block is outside of VPDU memory reused area

  - isDecomp((refLeftX + (1<<7))/64*64, refTopY/64*64) == false

  - ((refLeftX + (1<<7))/64*64, refTopY/64*64) != (X, Y)

- If the upper left sample of the reference block has been reconstructed and accessible.

  - isDecomp((refLeftX, refTopY) == true

- If the bottom right sample of the reference block has been reconstructed and accessible.

  - isDecomp((refRightX, refBottomY) == true

# Proposed method

## Modified reference area

It is proposed to keep the most recently decoded two VPDUs in an even-number VPDU row and one VPDU in an odd-number VPDU as the reference area for an IBC block. Depending on the location of current VPDU, the reference is shown in figure 2. In general, such an arrangement makes the reference area closer for an IBC block.

Figure 2. Reference area definition in the proposed design.

## Changes to the block vector checking conditions

In the proposed method, the checking processes of 1), 2), 4), 5) in section 1 remain the same and 3) in section 1 is replaced by

- isDecomp((refLeftX + 128)/64*64 - (refY&0x40) + (x%64), refTopY/64*64+ (refY^y)>>6)?0:(y%64)) == false
where refY = (Y & 0x40) ? refTopY : refBottomY

# Experimental results

Experiments are conducted following the CE8 test conditions [1]. The results are summarized in the following table.

Table 1. Comparison between the proposed method with the VTM-5.0 anchor.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over vtm5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.02% | 0.07% | 142% | 100% |
| Class A2 | -0.78% | -1.00% | -0.97% | 147% | 99% |
| Class B | -0.78% | -0.77% | -0.78% | 158% | 97% |
| Class C | -0.30% | -0.34% | -0.21% | 158% | 97% |
| Class E | -1.29% | -1.22% | -1.43% | 162% | 94% |
| Overall | -0.64% | -0.66% | -0.65% | 154% | 97% |
| Class D | -0.55% | -0.56% | -0.61% | 152% | 96% |
| Class F | -0.78% | -0.71% | -0.75% | 102% | 98% |
| 4:2:0 TGM | -1.82% | -1.85% | -1.87% | 101% | 97% |
|  | Random Access |  |  |  |  |
|  | Over vtm5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.23% | 0.14% | 105% | 100% |
| Class A2 | -0.06% | -0.38% | -0.07% | 107% | 101% |
| Class B | -0.13% | -0.48% | -0.63% | 109% | 99% |
| Class C | 0.15% | -0.33% | -0.12% | 114% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.37% | -0.23% | 109% | 100% |
| Class D | 0.10% | -0.23% | -0.06% | 113% | 119% |
| Class F | -0.55% | -0.68% | -0.58% | 101% | 103% |
| 4:2:0 TGM | -1.09% | -1.14% | -1.15% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over vtm5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.35% | -0.11% | 119% | 81% |
| Class C | 0.08% | 0.25% | -0.33% | 126% | 99% |
| Class E | 0.03% | -0.27% | 0.62% | 116% | 98% |
| Overall | 0.05% | 0.16% | 0.00% | 120% | 91% |
| Class D | 0.08% | 0.23% | 1.17% | 128% | 104% |
| Class F | -0.32% | -0.25% | -0.68% | 101% | 98% |
| 4:2:0 TGM | -0.71% | -0.86% | -0.85% | 100% | 98% |

# Changes to the working draft

Changes, marked in yellow, to the working draft based on JVET-N1001-v8 are shown below.

8.6.2 Derivation process for motion vector components for IBC blocks

8.6.2.1 General

……

- When the motion vector mvL points to a reference block where the top-left sample is located inside the neighbouring coding tree block to the left, i.e. ( xRefTL ) >> CtbLog2SizeY is equal to ( xCb >> CtbLog2SizeY ) − 1, and CtbSizeY is equal to 128, the motion vector mvL shall not point to a reference block where the top-left sample is located left to or inside the same 64x64 area as the current block but inside the neighbouring coding tree block to the left, i.e. the following conditions shall be true:

  - The derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( ( xRefTL + 128 ) / 64 * 64, yRefTL / 64 * 64 ) ( ( xRefTL + 128 ) / 64 * 64 – (refY & 0x40) + (xCb % 64), yRefTL / 64 * 64 + (refY ^ yCb) ? 0 : (yCb % 64) ) as inputs, and the output shall be equal to FALSE.

  - The luma location ( ( xRefTL + 128 ) / 64 * 64, yRefTL / 64 * 64 ) shall not be equal to ( xCb, yCb ).

# References

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-N1028, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).