| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0569-v1 |
| --- | --- |

| Title: | Non-CE6: LFNST Signaling at the TU Level |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Hilmi E. Egilmez, Amir Said, Muhammed Coban, Vadim Seregin, Wei-Jung Chien and Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | analci@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a new signaling approach for Low-frequency Non-Separable Transform (LFNST). In VTM-5.0, signaling of the LFNST index is done at the coding unit (CU) level after primary transform and before quantization at encoder side. Likewise, at decoder LFNST index is read after dequantization and inverse primary transform.

This contribution proposes to: (1) move LFNST index signaling from CU level to transform unit (TU) level, and (2) signal LFNST index only for the first TU in a given CU to reduce possible signaling overhead. The proposed approach is an inherent simplification considering that LFNST is a complementary secondary transform to the primary transform (e.g. primary transform is signaled at the TU level). Note that we perform LFNST on all TUs as in VTM-5.0 regardless of the proposed signaling.

To warrant the proposed simplification, 4 separate tests were conducted for both (1) and (2), and under common test conditions (CTC) and under non-CTC conditions in VTM-5.0.

Under CTC conditions signaling of LNST index can be moved to TU level without any losses with average BD-rates of  YUV = (0%, 0%, 0%) for all intra (AI) and (0%, 0%, 0%) for random access (RA). This is partly because in CTC conditions a TU is usually equivalent to a CU as 128x128 CU splitting case does not appear often.

Under non-CTC conditions (i.e. where DualTree and LCTUFAST are disabled) LNST index signaling can be moved to TU level with average BD rates of  (0.00%, 0.02%, 0.02%) for AI and (0.00%, 0.05%, 0.04%) for RA.

# Introduction

In VTM-5.0, reduced secondary transform (RST) also known as LFNST is applied between the primary transform and quantization stage at encoder and between dequantization and inverse primary transform at decoder. This is illustrated in Figure 1, where a 4x4 or an 8x8 LFNST is applied to a subset (top-left) of primary transform coefficients. The respective LFNST index is signaled at the CU level after signaling transform coefficients. LFNST is used for both intra and inter slices and signaled separately for luma and chroma when dual tree is enabled. However, if dual tree is disabled only one LFNST index is signaled for both luma and chroma.

In VTM-5.0, a single CU can consist of multiple TUs. For instance, when CU size is 128x128, then CU is split into 4 TUs of size 64x64 as illustrated in Figure 2. In the current design, LFNST does not have direct access to TU level parameters. Moreover, CU based signaling of LFNST index is waits until all TUs are processed.

Figure 1. Low Frequency Non-Separable Transform (LFNST) Process in VTM-5.0

# Proposed method

In the proposed approach, LFNST signaling is moved to TU level at the end of transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ). One advantage of this approach is that, LFNST signaling can now be done for each TU separately, or for a subset of TUs residing in a host CU. This results in cleaner design since (a) LFNST is a complementary transform to the primary transform, which is signaled at the TU level, and (b) LFNST signaling can be immediately signaled at the TU level without waiting for all TUs to be processed.

Figure 2. Proposed Change in LFNST Index Signaling at the TU level.  Signaling of the LFNST index for the First TU (top-left TU) is mandatory, however as shown with ** for other TUs, LFNST index can be inferred from the first TU and are not required to be signaled.

To further reduce signaling overhead, we also propose to signal LFNST index only for the first TU in a given CU. Therefore, LFNST index is signaled only once for the first TU in a present CU. In this case, other TUs residing in the same CU can derive/infer LFNST index from the first TU.

Proposed signaling for LFNST index is briefly illustrated in Figure 2, where LFNST signaling is moved to TU instead of CU, and signaling LFNST index for the First TU (e.g. top-left block on the right hand side) is mandatory. However, signaling  LFNST index for remaining TUs is not necessary (as shown with ** for these TUs).

# Results

The proposed method is tested over VTM-5.0 on top of both CTC and non-CTC configurations (e.g. DualTree and LCTUFAST disabled). These results are presented for AI and RA cases below. Note that LFNST is disabled for

## Under CTC Conditions

### Move LFNST Signaling to TU Level and Signal only for the First TU

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 106% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 114% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 113% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 107% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 104% |

Table 1. AI results under CTC conditions when signaling LFNST index for the first TU

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 103% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 104% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 99% |

Table 2. RA results under CTC conditions when signaling LFNST index for the first TU

### Move LFNST Signaling to TU Level and Signal for Each TU

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 107% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 113% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 113% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 107% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 93% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 99% |

Table 3. AI results under CTC conditions when signaling LFNST index for each TU

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 103% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 100% |

Table 4. RA results under CTC conditions when signaling LFNST index for each TU

## Under Non-CTC Conditions (DualTree and LCTUFAST disabled)

### Move LFNST Signaling to TU Level and Signal only for the First TU

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM-5.0 (DualTree=0, LCTUFAST=0) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.10% | 0.04% | 100% | 100% |
| Class B | 0.00% | 0.00% | -0.02% | 100% | 102% |
| Class C | 0.00% | 0.00% | -0.01% | 100% | 97% |
| Class E | 0.00% | 0.03% | 0.14% | 100% | 98% |
| Overall | 0.00% | 0.02% | 0.02% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | -0.05% | 0.02% | 100% | 102% |

Table 5. AI results on top of CTC with DualTree and LCTUFAST disabled over VTM5.0 when signaling LFNST index for the first TU

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM-5.0 (DualTree=0, LCTUFAST=0) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.09% | -0.03% | 100% | 101% |
| Class A2 | 0.00% | 0.11% | 0.19% | 101% | 100% |
| Class B | -0.01% | 0.13% | 0.04% | 99% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.05% | 0.04% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F | -0.05% | -0.03% | 0.07% | 99% | 98% |

Table 6. RA results on top of CTC with DualTree and LCTUFAST disabled over VTM5.0 when signaling LFNST index for the first TU

### Move LFNST Signaling to TU Level and Signal for Each TU

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM-5.0 (DualTree=0, LCTUFAST=0) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.01% | 0.01% | 100% | 100% |
| Class A2 | 0.01% | 0.02% | 0.01% | 101% | 100% |
| Class B | 0.01% | 0.02% | 0.03% | 100% | 102% |
| Class C | 0.00% | -0.01% | -0.01% | 100% | 98% |
| Class E | 0.04% | -0.10% | 0.14% | 100% | 98% |
| Overall | 0.02% | -0.01% | 0.03% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.04% | 0.02% | 0.04% | 100% | 100% |

Table 7. AI results on top of CTC with DualTree and LCTUFAST disabled when signaling LFNST index for each TU separately

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM-5.0 (DualTree=0, LCTUFAST=0) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.02% | 0.04% | 100% | 101% |
| Class A2 | 0.01% | -0.14% | 0.07% | 100% | 100% |
| Class B | 0.01% | 0.10% | -0.02% | 98% | 99% |
| Class C | 0.01% | -0.02% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.02% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class F | 0.01% | 0.10% | 0.17% | 100% | 100% |

Table 8. RA results on top of CTC with DualTree and LCTUFAST disabled when signaling LFNST index for each TU separately

# Conclusions

In this contribution, it is proposed to: (1) move the LFNST index signaling to TU level, and (2) signal LFNST index only for the First TU (i.e. still apply LFNST for every TU). The proposed change provides cleaner design with no losses under CTC conditions, and negligible losses/gains for the Non-CTC conditions discussed above.

It is recommended to adopt the proposed method in VVC.

# References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5),” JVET-N1001, 14th Meeting, Geneva, CH, Mar. 2019.

- J. Chen, T. Ye and S. H. Kim, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5),” JVET-N1002, 14th Meeting, Geneva, CH, Mar. 2019.

- M. Salehifar et al., “CE 6.2.6: Reduced Secondary Transform (RST)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0099, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- M. Koo et al., “CE 6-2.1: Reduced Secondary Transform (RST)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0133, 12th meeting, Macao, CN, 3-12 Oct. 2018.

- M. Koo et al., “CE6: Reduced Secondary Transform (RST)(CE6-3.1)”, JVET-N0193, 14th meeting, Geneva, CH, 19-27 March 2019.

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).