| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0570 |
| --- | --- |

| Title: | Non-CE9: Unified gradient calculations in BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang, Zhipin Deng, Na Zhang, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In Bi-Direction Optical Flow, the spatial gradient and the temporal gradient are derived in different ways, which makes the logic complex. This contribution proposes to unify the derivation methods of spatial and temporal gradients. Simulation results reportedly show that the proposed method has negligible impact on the coding performance.

# Introduction

In Bi-Direction Optical Flow (BDOF) [1], both spatial gradient and temporal gradient need to be derived. Suppose neigS0 and neigS1 are two samples used for deriving the spatial gradient, and neigT0 and neighT1 are two samples used for deriving the temporal gradient, the spatial gradient and the temporal gradient are right shifted by shiftS and shiftT respectively. The spatial gradient and the temporal gradient are derived differently as follows:

- Spatial gradient: (neigS0 – neigS1) >> shiftS.

- Temporal gradient: (neigT0 >> shiftT) – (neigT1 >> shiftT).

# Proposed method

To tackle the abovementioned problem, this contribution tests two unified derivation methods of the spatial gradient and the temporal gradient.

- Test #1: the right shift is performed before the subtraction.

  - Spatial gradient: (neigS0 >> shiftS) – (neigS1 >> shiftS).

  - Temporal gradient: (neigT0 >> shiftT) – (neigT1 >> shiftT).

- Test #2: the right shift is performed after the subtraction.

  - Spatial gradient: (neigS0 – neigS1) >> shiftS.

  - Temporal gradient: (neigT0 – neigT1) >> shiftT.

# Experimental results

The proposed method was implemented on top of the reference software VTM-5.0 [2], and simulations were performed following the common test conditions [3]. The results are shown in Table 1 and Table 2. It can be seen that both methods have negligible impact on the coding performance.

Table 1: Coding performance of test #1.

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.20% | -0.04% | 99% | 99% |
| Class A2 | -0.01% | -0.05% | -0.02% | 99% | 99% |
| Class B | 0.00% | 0.02% | 0.02% | 100% | 98% |
| Class C | -0.02% | -0.02% | -0.07% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.05% | -0.02% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.15% | 101% | 102% |
| Class F | 0.00% | 0.00% | -0.01% | 100% | 100% |

Table 2: Coding performance of test #2.

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.20% | -0.05% | 100% | 99% |
| Class A2 | 0.01% | -0.01% | 0.03% | 100% | 99% |
| Class B | 0.02% | 0.00% | 0.00% | 100% | 98% |
| Class C | -0.03% | 0.03% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.01% | 100% | 99% |
| Class D | 0.01% | 0.00% | 0.05% | 101% | 103% |
| Class F | -0.02% | 0.03% | 0.00% | 101% | 101% |

# Conclusions

This contribution proposes to unify the derivation method of the spatial gradients and the temporal gradients in BDOF. It has negligible impact on the coding performance. It is recommended to adopt the proposed method into VVC.

# References

- “Versatile Video Coding (Draft 5)”, B. Bross, J. Chen, S. Liu, Doc. JVET-N1001, Mar. 2019.

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, Doc. JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).