| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0571 |
| --- | --- |

| Title: | Non-CE3: Up-sampling with a fixed order in MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang, Zhipin Deng, Jizheng Xu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

When the reduced prediction signal in MIP needs to be up-sampled in both directions, the up-sampling is always firstly performed along the shorter side of a CU. Afterwards, the up-sampling along the longer side is performed. Therefore, the up-sampling order in MIP varies with different CUs. This contribution proposes to perform the up-sampling in a fixed up-sampling order for the prediction signal up-sampling process in MIP. Simulation results reportedly show that the proposed method has negligible impact on the coding performance.

# Introduction

MIP has been adopted into VVC [1]. In MIP, the prediction signal of a CU is generated by the following steps:

- First, a sample vector (with a size equal to 4 for a 4×4 block or 8 for other blocks) is generated from the left neighboring samples and the above neighboring samples.

- Second, a reduced prediction signal is generated by multiplying the sample vector with a matrix and adding a basis vector.

- Finally, the reduced prediction signal is up-sampled (if necessary) in the horizontal direction or the vertical direction or both directions.

When the reduced prediction signal needs to be up-sampled in both directions, the up-sampling is performed along the shorter side of the CU first. Therefore, different CUs may require different up-sampling orders.

# Proposed method

To tackle the abovementioned problem, this contribution proposes to perform the up-sampling in a fixed order for MIP when the up-sampling is required in both directions. Two fixed up-sampling orders are tested.

- Test #1: the up-sampling is performed firstly in the vertical direction and secondly in the horizontal direction.

- Test #2: the up-sampling is performed firstly in the horizontal direction and secondly in the vertical direction.

# Experimental results

The proposed method is implemented on top of the reference software VTM-5.0 [2], and simulations were performed following the common test conditions [3]. The results are shown in Table 1 and Table 2 for test #1 and test #2, respectively. It can be seen that both tests have negligible impact on the coding performance.

Table 1: Coding performance of test #1.

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.09% | 0.07% | 101% | 100% | 0.01% | -0.10% | 0.13% | 100% | 100% |
| Class A2 | -0.01% | -0.02% | -0.04% | 100% | 99% | 0.01% | -0.13% | 0.02% | 100% | 99% |
| Class B | 0.01% | -0.06% | 0.00% | 98% | 99% | 0.01% | -0.02% | 0.05% | 100% | 99% |
| Class C | 0.00% | -0.01% | 0.02% | 99% | 93% | -0.05% | -0.10% | -0.05% | 99% | 97% |
| Class E | 0.00% | 0.02% | 0.00% | 99% | 97% |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.01% | 99% | 97% | -0.01% | -0.08% | 0.03% | 100% | 98% |
| Class D | 0.00% | 0.06% | 0.02% | 101% | 102% | 0.00% | 0.02% | 0.13% | 101% | 103% |
| Class F | -0.01% | -0.06% | -0.01% | 101% | 100% | 0.00% | 0.01% | 0.10% | 99% | 97% |

Table 2: Coding performance of test #2.

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | -0.02% | 100% | 100% | 0.02% | -0.02% | 0.16% | 99% | 99% |
| Class A2 | -0.01% | -0.05% | -0.03% | 99% | 99% | 0.04% | 0.13% | -0.05% | 99% | 100% |
| Class B | 0.00% | -0.06% | -0.03% | 98% | 99% | 0.02% | 0.01% | -0.05% | 99% | 98% |
| Class C | 0.00% | -0.04% | 0.05% | 99% | 92% | -0.01% | -0.02% | 0.08% | 99% | 97% |
| Class E | -0.02% | -0.08% | 0.08% | 99% | 95% |  |  |  |  |  |
| Overall | 0.00% | -0.04% | 0.01% | 99% | 97% | 0.01% | 0.02% | 0.03% | 99% | 98% |
| Class D | -0.03% | 0.24% | 0.01% | 100% | 98% | 0.00% | 0.00% | 0.07% | 100% | 100% |
| Class F | 0.00% | 0.06% | -0.04% | 100% | 100% | -0.02% | -0.18% | 0.16% | 100% | 98% |

# Conclusions

This contribution proposes a fixed up-sampling order for MIP. It has negligible impact on the coding performance. It is recommended to adopt the proposed method into VVC.

# References

- “Versatile Video Coding (Draft 5)”, B. Bross, J. Chen, S. Liu, Doc. JVET-N1001, Mar. 2019.

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, Doc. JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).