| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0573 |
| --- | --- |

| Title: | Non-CE9: Restriction on motion vector refinement in BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang, Hsiao Chiang Chuang, Jizheng Xu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In Bi-Direction Optical Flow (BDOF), two multiplications per sample are required to derive a sample adjustment. This contribution proposes to restrict the motion vector refinement to be in a form of +2N or –2N (N is an integer). In this way, the two multiplications can be replaced by two shifts when calculating the sample adjustment. Simulation results reportedly show -0.01%, -0.05% and -0.02% BD-rate changing under RA configurations for Y, Cb and Cr component, respectively.

# Introduction

In BDOF [1], the sample adjustment is derived based on the motion vector refinement and the spatial gradient of the prediction samples. Suppose the motion vector refinement for a sample is (vx, vy), the corresponding horizontal gradient and vertical gradient in prediction direction X (X = 0 or 1) are denoted as (gradHorX, gradVerX), the sample adjustment is calculated as:

vx * (gradHor1 – gradHor0) + vy * (gradVer1 – gradVer0).

Two multiplications per sample are required to calculate the sample adjustment.

# Proposed method

To tackle the abovementioned problem, this contribution proposes to restrict the motion vector refinement to be in a form of +2N or –2N. In this way, the two multiplications can be replaced by two shifts.

# Experimental results

The proposed method was implemented on top of the reference software VTM-5.0 [2], and simulations were performed following the common test conditions [3]. The results are shown in Table 1. It can be seen that the proposed method has negligible impact on the coding performance.

Table 1: Coding performance of test #1.

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.20% | 0.00% | 101% | 101% |
| Class A2 | -0.01% | -0.02% | -0.03% | 101% | 101% |
| Class B | -0.02% | -0.03% | 0.00% | 100% | 100% |
| Class C | -0.02% | 0.00% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.05% | -0.02% | 101% | 100% |
| Class D | 0.05% | -0.10% | 0.10% | 101% | 102% |
| Class F | -0.01% | -0.01% | 0.01% | 100% | 101% |

# Conclusions

This contribution proposes to restrict the motion vector refinement in BDOF to be in a form of +2N or –2N. Consequently, the two multiplications required in calculating the sample adjustment can be removed. It has negligible impact on the coding performance. It is recommended to adopt the proposed method into VVC.

# References

- “Versatile Video Coding (Draft 5)”, B. Bross, J. Chen, S. Liu, Doc. JVET-N1001, Mar. 2019.

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, Doc. JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).