| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0576 |
| --- | --- |

| Title: | Non-CE6: Reduced contexts and context coded bins for MTS index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Li Zhang, Hongbin Liu, Kai Zhang, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC draft, four context coded bins with eight contexts are assigned for the syntax element tu_mts_idx which may impact the parsing throughput. In this contribution, it proposes to reduce the number of context coded bins for MTS index coding from four to two. In addition, the contexts used for coding the MTS index is reduced from eight to two. Simulation results reported show that there is almost no impact on the coding performance.

# CtxInc assignment of tu_mts_idx in VVC draft 5

In the VVC draft 5 [1], there are eight contexts used for the four context coded bins of the syntax element tu_mts_idx. As illustrated in Table 1, the context modeling of the bin index 0 depends on QT depth, which requires four contexts. And three more contexts are used for the bin index 1, 2, and 3, respectively. The four context coded bins with eight context models used for tu_mts_idx may impact the parsing throughput of VVC.

Table 1: Assignment of ctxInc to syntax elements tu_mts_idx with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| tu_mts_idx[ ][ ] | cqtDepth | 6 | 7 | 8 | na | na |

# Proposed solution

To improve the parsing throughput of VVC, this contribution proposes to reduce the context coded bins of the syntax element tu_mts_idx from four to two. That is, only the first two bins are context coded and the remaining bins are bypass coded. Moreover, instead of using eight context models, only two context models are used in the proposed method, in which one context is used for the first bin, and the other context is used for the second bin, as depicted in Table 2.

Table 2: Assignment of ctxInc to syntax elements tu_mts_idx with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| tu_mts_idx[ ][ ] | 0 | 1 | bypass | bypass | na | na |

# Simulation results

The proposed method was implemented based on the VTM-5.0 reference software [2] and followed the common test conditions [3]. Simulation results of the proposed method is provided in Table 3.

Table 3: Simulation results of the proposed method

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | -0.08% | -0.05% | 99% | 100% |
| Class B | 0.00% | -0.01% | 0.06% | 100% | 100% |
| Class C | -0.01% | -0.07% | 0.09% | 99% | 98% |
| Class E | -0.01% | 0.05% | -0.01% | 99% | 98% |
| Overall | 0.00% | -0.02% | 0.03% | 100% | 99% |
| Class D | 0.01% | 0.04% | -0.10% | 103% | 100% |
| Class F | -0.03% | 0.00% | -0.10% | 101% | 98% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.13% | 0.08% | 99% | 101% |
| Class A2 | 0.02% | -0.10% | 0.03% | 100% | 101% |
| Class B | 0.01% | -0.10% | -0.13% | 100% | 99% |
| Class C | -0.03% | -0.07% | -0.23% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.10% | -0.08% | 99% | 100% |
| Class D | 0.01% | 0.12% | -0.18% | 100% | 98% |
| Class F | -0.01% | -0.15% | -0.01% | 100% | 99% |

# Conclusions

In this contribution, the number of context coded bins and the number of context models used for tu_mts_idx are both reduced, which is asserted to improve the parsing throughput of VVC. It is recommended adopting this method to the next version of VVC.

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, March 2019, Geneva, CH.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, CH.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).