| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0577 |
| --- | --- |

| Title: | Non-CE9: Bi-directional optical flow (BDOF) for chroma components |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen Yuwen He 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4855 Wei.Chen@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Proposed specification text changes

#### 8.5.6.1 General

This process is invoked when decoding a coding unit coded in inter prediction mode.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- variables numSbX and numSbY specifying the number of luma coding subblocks in horizontal and vertical direction,

- the motion vectors mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the refined motion vectors refMvL0[ xSbIdx ][ ySbIdx ] and refMvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the bi-prediction weight index bcwIdx,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an array predSamples of prediction samples.

Let predSamplesL0L, predSamplesL1L and predSamplesIntraL be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr and predSamplesL1Cr, predSamplesIntraCb, and predSamplesIntraCr be (cbWidth / 2)x(cbHeight / 2) arrays of predicted chroma sample values.

  - The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - sps_bdof_enabled_flag is equal to 1.

      - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

      - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

      - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

      - sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

      - BcwIdx[ xCb ][ yCb ] is equal to 0.

      - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

      - cbHeight is greater than or equal to 8

      - cIdx is equal to 0 or cbWidth is greater than or equal to 8.

    - Otherwise, bdofFlag is set equal to FALSE.

- If numSbY is equal to 1 and numSbX is equal to 1 the following applies:

  - When bdofFlag is equal to TRUE, the variables numSbY, numSbX are modified as follows:

numSbX = ( cbWidth > 16 )  ?  ( cbWidth >> 4 )  :  1	(8-700)

numSbY = ( cbHeight > 16 )  ?  ( cbHeight >> 4 )  :  1	(8-701)

  - For X = 0..1, xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

    - predFlagLX[ xSbIdx ][ ySbIdx ] is set equal to predFlagLX[ 0 ][ 0 ].

    - refMvLX[ xSbIdx ][ ySbIdx ] is set equal to refMvLX[ 0 ][ 0 ].

    - mvLX[ xSbIdx ][ ySbIdx ] is set equal to mvLX[ 0 ][ 0 ].

The width and the height of the current coding sublock sbWidth, sbHeight in luma samples are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-702)

sbHeight  =  cbHeight / numSbY		(8-703)

For each coding subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1, the following applies:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

( xSb, ySb )  =  ( xCb + xSbIdx * sbWidth, yCb + ySbIdx * sbHeight )	(8-704)

- For X being each of 0 and 1, when predFlagLX[ xSbIdx ][ ySbIdx ] is equal to 1, the following applies:

  - The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in clause 8.5.6.2 with X and refIdxLX as inputs.

  - The motion vector offset mvOffset is set equal to refMvLX[ xSbIdx ][ xSbIdx ] − mvLX[ xSbIdx ][ ySbIdx ].

  - When one or more of the following conditions is true, mvOffset[ 0 ] is set equal to 0:

    - xSb is not equal to xCb and mvOffset[ 0 ] is less than 0

    - ( xSb + sbWidth ) is not equal to ( xCb + cbWidth) and mvOffset[ 0 ] is greater than 0

  - When one or more of the following conditions is true, mvOffset[ 1 ] is set equal to 0:

    - ySb is not equal to yCb and mvOffset[ 1 ] is less than 0

    - ( ySb + sbHeight ) is not equal to ( yCb + cbHeight ) and mvOffset[ 1 ] is greater than 0

  - If cIdx is equal to 0, the following applies:

    - The array predSamplesLXL  is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xSb, ySb ), the coding subblock width sbWidth, the coding subblock height sbHeight in luma samples, the luma motion vector offset mvOffset, the refined luma motion vector refMvLX[ xSbIdx ][ xSbIdx ], the reference array refPicLXL, bdofFlag, and cIdx as inputs.

  - Otherwise if cIdx is equal to 1, the following applies:

    - The array predSamplesLXCb is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xSb, ySb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the chroma motion vector offset mvOffset, the refined chroma motion vector refMvLX[ xSbIdx ][ xSbIdx ], the reference array refPicLXCb, bdofFlag, and cIdx as inputs.

  - Otherwise (cIdx is equal to 2), the following applies:

    - The array predSamplesLXCr is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xSb, ySb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the chroma motion vector offset mvOffset, the refined chroma motion vector refMvLX[ xSbIdx ][ xSbIdx ], the reference array refPicLXCr, bdofFlag, and cIdx as inputs.

- If bdofFlag is equal to TRUE, the following applies:

- The variable shift is set equal to Max( 2, 14 − BitDepthY ).

- The variables sbDiffThres, bdofBlkDiffThres, and sbSumDiff are derived as follows:

sbDiffThres  =  ( 1 << ( BitDepthY − 8 + shift ) ) * sbWidth*sbHeight 	(8-705)

bdofBlkDiffThres  =  1 << ( BitDepthY − 3 + shift )	(8-706)

sbSumDiff =  0		(8-707)

- For xIdx = 0..(sbWidth >> 2 ) − 1 and yIdx = 0..( sbHeight >> 2 ) − 1, the variables bdofBlkSumDiff and the bidirectional optical flow utilization flag bdofUtilizationFlag[ xIdx ][ yIdx ] are derived as follows:

bdofBlkSumDiff = i=03j=03Abs( predSamplesL0L[ ( xIdx << 2 ) + 1 + i ][ ( yIdx << 2 ) + 1 + j ] − 	(8-708)
		predSamplesL1L[ ( xIdx << 2 ) + 1 + i ][ ( yIdx << 2 ) + 1 + j] )

bdofUtilizationFlag[ xIdx ][ yIdx ] = 	bdofBlkSumDiff >= bdofBlkDiffThres	 (8-709)

sbSumDiff += bdofBlkSumDiff		(8-710)

- The variable sbBdofFlag is derived as follows:

- If sbSumDiff is less than sbDiffThres, sbBdofFlag is set equal to FALSE.

- Otherwise, sbBdofFlag set equal to TRUE.

  - The (sbWidth / 4)x(sbHeight / 4)x2 motion offset array bdofMo is set equal to 0.

- The array predSamples of prediction samples is derived as follows:

- If cIdx is equal to 0, the prediction samples inside the current luma coding subblock, predSamples[ xL + xSb ][ yL + ySb ] with xL = 0..sbWidth − 1 and yL = 0..sbHeight − 1, are derived as follows:

- If sbBdofFlag is equal to TRUE, the bidirectional optical flow sample prediction process as specified in clause 8.5.6.4 is invoked with nCbW set equal to the luma coding subblock width sbWidth, nCbH set equal to the luma coding subblock height sbHeight and the sample arrays predSamplesL0L and predSamplesL1L, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1 and , bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( sbWidth >> 2 ) − 1, yIdx = 0..( sbHeight >> 2 ) – 1, cIdx and bdofMo as inputs, and predSamples[ xL + xSb ][ yL + ySb ] as outputs.

- Otherwise (sbBdofFlag is equal to FALSE), the weighted sample prediction process as specified in clause 8.5.6.5 is invoked with the luma coding subblock width sbWidth, the luma coding subblock height sbHeight and the sample arrays predSamplesL0L and predSamplesL1L, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bcwIdx, and cIdx as inputs, and predSamples[ xL + xSb ][ yL + ySb ] as outputs.

- Otherwise, if cIdx is equal to 1, the prediction samples inside the current chroma component Cb coding subblock, predSamples[ xC + xSb / 2 ][ yC + ySb / 2 ] with xC = 0..sbWidth / 2 − 1 and yC = 0..sbHeight / 2 − 1, are derived as follows:

- If sbBdofFlag is equal to TRUE, the bidirectional optical flow sample prediction process as specified in clause 8.5.6.4 is invoked with nCbW set equal to sbWidth / 2, nCbH set equal to sbHeight / 2, the sample arrays predSamplesL0Cb and predSamplesL1Cb, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( sbWidth >> 2 ) − 1, yIdx = 0..( sbHeight >> 2 ) – 1, cIdx and bdofMo as inputs, and predSamples[ xC + xSb / 2 ][ yC + ySb / 2 ] as outputs.

- Otherwise (sbBdofFlag is equal to FALSE), the weighted sample prediction process as specified in clause 8.5.6.5 is invoked with nCbW set equal to sbWidth / 2, nCbH set equal to sbHeight / 2, the sample arrays predSamplesL0Cb and predSamplesL1Cb, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bcwIdx, and cIdx as inputs, and predSamples[ xC + xSb / 2 ][ yC + ySb / 2 ] as outputs.

- Otherwise (cIdx is equal to 2), the prediction samples inside the current chroma component Cr coding subblock, predSamples[ xC + xSb / 2 ][ yC + ySb / 2 ] with xC = 0..sbWidth / 2 − 1 and yC = 0..sbHeight / 2 − 1, are derived as follows:

- If sbBdofFlag is equal to TRUE, the bidirectional optical flow sample prediction process as specified in clause 8.5.6.4 is invoked with nCbW set equal to sbWidth / 2, nCbH set equal to sbHeight / 2, the sample arrays predSamplesL0Cr and predSamplesL1Cr, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( sbWidth >> 2 ) − 1, yIdx = 0..( sbHeight >> 2 ) – 1, cIdx and bdofMo as inputs, and predSamples[ xC + xSb / 2 ][ yC + ySb / 2 ] as outputs.

- Otherwise (sbBdofFlag is equal to FALSE), the weighted sample prediction process as specified in clause 8.5.6.5 is invoked with nCbW set equal to sbWidth / 2, nCbH set equal to sbHeight / 2, the sample arrays predSamplesL0Cr and predSamplesL1Cr, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bcwIdx, and cIdx as inputs, and predSamples[ xC + xSb / 2 ][ yC + ySb / 2 ] as outputs.

- When cIdx is equal to 0, the following assignments are made for x = 0..sbWidth − 1 and y = 0..sbHeight − 1:

MvL0[ xSb + x ][ ySb + y ] = mvL0[ xSbIdx ][ ySbIdx ]	(8-711)

MvL1[ xSb + x ][ ySb + y ] = mvL1[ xSbIdx ][ ySbIdx ]	(8-712)

MvDmvrL0[ xSb + x ][ ySb + y ] = refMvL0[ xSbIdx ][ ySbIdx ]	(8-713)

MvDmvrL1[ xSb + x ][ ySb + y ] = refMvL1[ xSbIdx ][ ySbIdx ]	(8-714)

RefIdxL0[ xSb + x ][ ySb + y ] = refIdxL0		(8-715)

RefIdxL1[ xSb + x ][ ySb + y ] = refIdxL1		(8-716)

PredFlagL0[ xSb + x ][ ySb + y ] = predFlagL0[ xSbIdx ][ ySbIdx ]	(8-717)

PredFlagL1[ xSb + x ][ ySb + y ] = predFlagL1[ xSbIdx ][ ySbIdx ]	(8-718)

BcwIdx[ xSb + x ][ ySb + y ] = bcwIdx		(8-719)

When ciip_flag[ xCb ][ yCb ] is equal to 1, the array predSamples of prediction samples is modified as follows:

- If cIdx is equal to 0, the following applies:

- The general intra sample prediction process as specified in clause 8.4.5.2.5 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xCb, yCb ), the intra prediction mode predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the transform block width nTbW and height nTbH set equal to cbWidth and cbHeight, the coding block width nCbW and height nCbH set equal to cbWidth and cbHeight, and the variable cIdx as inputs, and the output is assigned to the (cbWidth)x(cbHeight) array predSamplesIntraL.

- The weighted sample prediction process for combined merge and intra prediction as specified in clause 8.5.6.6 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xCb, yCb ), the coding block width cbWidth, the coding block height cbHeight, the sample arrays predSamplesInter and predSamplesIntra set equal to predSamples and predSamplesIntraL, respectively, the intra prediction mode predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], and the colour component index cIdx as inputs, and the output is assigend to the (cbWidth)x(cbHeight) array predSamples.

- Otherwise, if cIdx is equal to 1, the following applies:

- The general intra sample prediction process as specified in clause 8.4.5.2.5 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xCb / 2, yCb / 2 ), the intra prediction mode predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the transform block width nTbW and height nTbH set equal to cbWidth / 2 and cbHeight / 2, the coding block width nCbW and height nCbH set equal to cbWidth / 2 and cbHeight / 2, and the variable cIdx as inputs, and the output is assigned to the (cbWidth / 2)x(cbHeight / 2) array predSamplesIntraCb.

- The weighted sample prediction process for combined merge and intra prediction as specified in clause 8.5.6.6 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xCb, yCb ), the coding block width cbWidth / 2, the coding block height cbHeight / 2, the sample arrays predSamplesInter and predSamplesIntra set equal to predSamplesCb and predSamplesIntraCb, respectively, the intra prediction mode predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], and the colour component index cIdx as inputs, and the output is assigend to the (cbWidth / 2)x(cbHeight / 2) array predSamples.

- Otherwise (cIdx is equal to 2), the following applies:

- The general intra sample prediction process as specified in clause 8.4.5.2.5 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xCb / 2, yCb / 2 ), the intra prediction mode predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the transform block width nTbW and height nTbH set equal to cbWidth / 2 and cbHeight / 2, the coding block width nCbW and height nCbH set equal to cbWidth / 2 and cbHeight / 2, and the variable cIdx as inputs, and the output is assigned to the (cbWidth / 2)x(cbHeight / 2) array predSamplesIntraCr.

- The weighted sample prediction process for combined merge and intra prediction as specified in clause 8.5.6.6 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xCb, yCb ), the coding block width cbWidth / 2, the coding block height cbHeight / 2, the sample arrays predSamplesInter and predSamplesIntra set equal to predSamplesCr and predSamplesIntraCr, respectively, the intra prediction mode predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], and the colour component index cIdx as inputs, and the output is assigend to the (cbWidth / 2)x(cbHeight / 2) array predSamples.

#### 8.5.6.3	Fractional sample interpolation process

##### 8.5.6.3.1	General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture,

- a variable sbWidth specifying the width of the current coding subblock,

- a variable sbHeight specifying the height of the current coding subblock,

- a motion vector offset mvOffset,

- a refined motion vector refMvLX,

- the selected reference picture sample array refPicLX,

- the bidirectional optical flow flag bdofFlag,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an (sbWidth +bdofOffset)x(sbHeight +bdofOffset) array predSamplesLX of prediction sample values.

The bidirectional optical flow boundary offset bdofOffset is derived as follows:

bdofOffset = bdofFlag  ?  2  :  0		(8-720)

- If cIdx is equal to 0, the following applies:

  - Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in 1/16-sample units. These variables are used only in this clause for specifying fractional-sample locations inside the reference sample arrays refPicLX.

  - For each luma sample location ( xL = 0..sbWidth − 1 +bdofOffset, yL = 0..sbHeight − 1 +bdofOffset ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL and yFracL are derived as follows:

xIntL = xSb + ( refMvLX[ 0 ]  >>  4 ) + xL		(8-721)

yIntL = ySb + ( refMvLX[ 1 ]  >>  4 ) + yL		(8-722)

xFracL = refMvLX[ 0 ] & 15		(8-723)

yFracL = refMvLX[ 1 ] & 15		(8-724)

  - If bdofFlag is equal to TRUE and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntL, yIntL ), ( xFracL, yFracL ), cIdx and refPicLX as inputs:

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

  - Otherwise, the following applies:

    - The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

    - The list padVal[ dir ] is derived as follows for dir = 0..1:

      - The variable disp is derived as follows:

disp = ( refMvLX[ dir ]  >>  4) − ( mvLX[ dir ]  >>  4) + ( dir = = 0 ? xL : yL)	(8-725)

      - If disp is less than 0, padVal[ dir ] is set equal to disp.

      - Otherwise, if disp is greater than ( dir  = = 0  ?  sbWidth : sbHeight ) − 1, padVal[ dir ] is set equal to disp − ( ( dir  = =  0  ?  sbWidth : sbHeight ) − 1).

      - Otherwise, padVal[ dir ] is set equal to 0.

    - The prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 8-tap interpolation filtering process as specified in clause 8.5.6.3.2 with ( xIntL, yIntL ), ( xFracL, yFracL ), refPicLX, sbWidth, sbHeight, ( xSb, ySb ) and padVal as inputs.

- Otherwise (cIdx is not equal to 0), the following applies:

  - Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in 1/32 sample units. These variables are used only in this clause for specifying general fractional-sample locations inside the reference sample arrays refPicLX.

  - For each chroma sample location ( xC = 0..sbWidth – 1 +bdofOffset, yC = 0.. sbHeight − 1 + bdofOffset) inside the prediction chroma sample arrays predSamplesLX, the corresponding prediction chroma sample value predSamplesLX[ xC ][ yC ] is derived as follows:

- The variables xIntC, yIntC, xFracC and yFracC are derived as follows: [Ed. (SL): Shall we make it consistent: /2 or /SubWidthC and /SubHeightC?]

xIntC = ( xSb / SubWidthC ) + ( mvLX[ 0 ]  >>  5 ) + xC	(8-726)

yIntC = ( ySb / SubHeightC ) + ( mvLX[ 1 ]  >>  5 ) + yC	(8-727)

xFracC = mvLX[ 0 ] & 31		(8-728)

yFracC = mvLX[ 1 ] & 31		(8-729)

  - If bdofFlag is equal to TRUE and one or more of the following conditions are true, the prediction chroma sample value predSamplesLX[ xC ][ yC ] is derived by invoking the chroma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntC, yIntC ), ( xFracC, yFracC ) , cIdx and refPicLX as inputs.

    - xC is equal to 0.

    - xC is equal to sbWidth + 1.

    - yC is equal to 0.

    - yC is equal to sbHeight + 1.

  - Otherwise, the following applies:

  - The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

  - The list padVal[ dir ] is derived as follows for dir = 0..1:

    - The variable disp is derived as follows:

disp = ( refMvLX[ dir ]  >>  4) − ( mvLX[ dir ]  >>  4) + ( dir = = 0 ? xC : yC)	(8-730)

    - If disp is less than 0, padVal[ dir ] is set equal to disp.

    - Otherwise, if disp is greater than ( dir  = = 0  ?  sbWidth / SubWidthC : sbHeight / SubWidthC ) − 1, padVal[ dir ] is set equal to disp − ( ( dir  = =  0  ?  sbWidth / SubWidthC : sbHeight / SubWidthC ) − 1).

    - Otherwise, padVal[ dir ] is set equal to 0.

  - The prediction sample value predSamplesLX[ xC ][ yC ] is derived by invoking the process specified in clause 8.5.6.3.4 with ( xIntC, yIntC ), ( xFracC, yFracC ), refPicLX, and padVal as inputs.

##### 8.5.6.3.3 Luma iInteger sample fetching process

Inputs to this process are:

- a luma sample location in full-sample units ( xInt, yInt ),

- a variable cIdx specifying the colour component index of the current block,

- the luma reference sample array refPicLX,

Output of this process is a predicted luma sample value predSampleLX

  - The variable shift is set equal to Max( 2, 14 – (cIdx==0?BitDepthY : SubWidthC)).

  - The variable cRatio is set equal to (cIdx==0?1: SubWidthC).

  - The variable picW is set equal to (pic_width_in_luma_samples/cRatio) and the variable picH is set equal to pic_height_in_luma_samples.

The luma sample locations in full-sample units ( xInt, yInt ) are derived as follows:

xInt = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?	(8-752)
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY / cRatio, picW, xInt ) : xInt )

yInt = Clip3( 0, picH − 1, yInt )		(8-753)

The predicted luma sample value predSampleLX is derived as follows:

predSampleLX = refPicLX[ xInt ][ yInt ] << shift3

#### 8.5.6.4	Bidirectional optical flow prediction process

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW + 2)x(nCbH + 2) luma prediction sample arrays predSamplesL0 and predSamplesL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the reference indices refIdxL0 and refIdxL1,

- the bidirectional optical flow utilization flags bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( nCbW >> 2 ) − 1, yIdx = 0..( nCbH >> 2 ) − 1,

- a variable cIdx specifying the colour component index of the current block,

- the motion offset array bdofMo.

Output of this process is the (nCbW)x(nCbH) array pbSamples of luma prediction sample values.

Variables bitDepth, shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows:

- The variable bitDepth is set equal to (cIdx==0?BitDepthY : BitDepthC)).

- The variable shift1 is set to equal to Max( 6, bitDepth − 6 ).

- The variable shift2 is set to equal to Max( 4, bitDepth − 8 ).

- The variable shift3 is set to equal to Max( 1, bitDepth − 11 ).

- The variable shift4 is set equal to Max( 3, 15 − bitDepth ) and the variable offset4 is set equal to 1  <<  ( shift4 − 1 ).

- The variable mvRefineThres is set equal to 1 << Max( 5, bitDepth − 7 ).

For xIdx = 0..( nCbW >> 2 ) − 1 and yIdx = 0..( nCbH >> 2 ) − 1, the following applies:

- The variable xSb is set equal to ( xIdx << 2) + 1 and ySb is set equal to ( yIdx << 2 ) + 1.

- If cIdx is equal to 0 and bdofUtilizationFlag[ xIdx ][ yIdx ] is equal to FALSE, for x = xSb − 1..xSb + 2, y = ySb − 1.. ySb + 2, the prediction sample values of the current subblock are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 +	(8-766)
	predSamplesL1[ x + 1 ][ y + 1 ] )  >>  shift4 )

- Otherwise (cIdx is not equal to 0 or bdofUtilizationFlag[ xIdx ][ yIdx ] is equal to TRUE), the prediction sample values of the current subblock are derived as follows:

  - For x =xSb − 1..xSb + 4, y = ySb − 1..ySb + 4, the following ordered steps apply:

- The locations ( hx, vy ) for each of the corresponding sample locations ( x, y ) inside the prediction sample arrays are derived as follows:

hx = Clip3( 1, nCbW, x )		(8-767)

vy = Clip3( 1, nCbH, y )		(8-768)

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[ x ][ y ]  =  (predSamplesL0[ hx + 1 ][vy] − predSampleL0[ hx − 1 ][ vy ] ) >> shift1	(8-769)

gradientVL0[ x ][ y ]  =   (predSampleL0[ hx ][ vy + 1 ] − predSampleL0[ hx ][vy − 1 ] ) >> shift1	(8-770)

gradientHL1[ x ][ y ]  =  (predSamplesL1[ hx + 1 ][vy] − predSampleL1[ hx − 1 ][ vy ] ) >> shift1	(8-771)

gradientVL1[ x ][ y ]  =   (predSampleL1[ hx ][ vy + 1 ] − predSampleL1[ hx ][vy − 1 ] ) >> shift1	(8-772)

- If cIdx is equal to 0, the variables diff[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows:

diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> shift2 ) − ( predSamplesL1[ hx ][ vy ] >> shift2 )	(8-773)

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> shift3	(8-774)

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> shift3	(8-775)

  - If cIdx is equal to 0, the variables sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as follows:

sGx2 = ij( tempH[ xSb + i ][ ySb + j ] * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-776)

sGy2 = ij( tempV[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-777)

sGxGy = ij( tempH[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j  −1..4	(8-778)

sGxGym = sGxGy >> 12		(8-779)

sGxGys =  sGxGy & ( ( 1 << 12 ) − 1 )		(8-780)

sGxdI = ij( −tempH[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-781)

sGydI = ij( −tempV[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-782)

  - If cIdx is equal to 0, the horizontal and vertical motion offset of the current subblock are derived as:

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres,	(8-783)
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − 	(8-784)
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGx2 ) ) )  :  0

bdofMo [ xIdx ] [ yIdx ] [ 0 ] = vx

bdofMo [ xIdx ] [ yIdx ] [ 1 ] = vy

  - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows:

If cIdx not equal to 0, the following applies:

vx = bdofMo [ x / 2] [ y / 2 ][ 0 ]

vy = bdofMo [ x / 2] [ y / 2 ][ 1 ]

bdofOffset = Round( ( vx * ( gradientHL1[ x + 1 ][ y + 1 ] − gradientHL0[ x + 1 ][ y + 1 ] ) ) >> 1 )	(8-785)
	+ Round( ( vy * (gradientVL1[ x + 1 ][ y + 1 ] − gradientVL0[ x + 1 ][ y + 1 ] ) ) >> 1 )

[Ed. (JC): Round() operation is defined for float input. The Round() operation seems redundant here since the input is an integer value. To be confirmed by the proponent]

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ]  + offset4 +	(8-786)
	predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset )  >>  shift4 )