| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0577 |
| --- | --- |

| Title: | Non-CE9: Bi-directional Optical Flow (BDOF) for Chroma Components |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen Yuwen He 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4855 Wei.Chen@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

The bi-directional optical flow (BDOF) tool is included in VTM-5. This contribution proposes to extend the BDOF to chroma components. The reported simulation result shows that comparing to VTM-5.0, average BD rate savings for RA are {-0.02%, -0.39%, -0.49%}.

# Introduction

The bi-directional optical flow (BDOF) tool is included in VTM-5. BDOF is used to refine the bi-prediction signal of a CU at the 4×4 sub-block level. However, BDOF is only applied to luma component. This contribution proposes to apply BDOF to both luma and chroma components.

# Proposed algorithm

The current VVC adopted the bi-directional optical flow (BDOF) to refine the bi-prediction signal of a CU at the 4×4 sub-block level. For each 4×4 sub-block, a motion refinement vx,vy is calculated by minimizing the difference between the L0 and L1 prediction samples. The motion refinement is then used to adjust the bi-predicted sample values in the 4x4 sub-block. BDOF is however only applied to the luma component.

To align the operation on both luma and chroma components, we propose to extend the BDOF to chroma as well. In order to reduce the computation complexity, the motion refinement already computed for luma component is reused on applying optical flow calculation for chroma component. Since a 4x4 sub-block in chroma component is corresponding to an 8x8 sub-block in luma component, the motion refinement values calculated for 4 4x4 sub-blocks in luma component are halved before applying to a 4x4 sub-block in chroma components.

# Simulation results

The contribution has been implemented on top of VTM-5.0. The test condition described in JVET-N1010 [1] used in the simulation. The result reported shows that the BD saving for random access configuration are {-0.XX%, -0.XX%, -0.XX%}.

Table 1.  Applying BDOF on both luma and chroma components

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.31% | -0.43% | 101% | 101% |
| Class A2 | -0.08% | -0.61% | -0.77% | 101% | 101% |
| Class B | 0.01% | -0.34% | -0.41% | 101% | 102% |
| Class C | -0.02% | -0.33% | -0.44% | 101% | 101% |
| Class E | - | - | - | - | - |
| Overall | -0.02% | -0.39% | -0.49% | 101% | 101% |
| Class D | 0.00% | -0.49% | -0.52% | 100% | 99% |
| Class F | 0.00% | -0.15% | -0.16% | 101% | 100% |

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).