| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0578 |
| --- | --- |

| Title: | Non-CE5: Long-tap deblocking filter on vertical tile boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Sangheon Lee Junghak Nam Jaehyun Lee Seunghwan Kim | Tel: Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VTM-5.0, long-tap deblock filtering is disabled on horizontal CTU boundary to reduce line buffer size. Hence, 4-row samples need to be stored in line buffer as the same with HEVC. However, 8-coloumn samples are still required to be stored in vertical line buffer when a picture is sliced to multiple tiles and long-tap deblocking filter is enabled for the vertical edge. In order to prevent vertical line buffer increasing over HEVC, the proposed method disallows the long-tap deblock filtering when in-loop filter is enabled across the vertical tile boundary. From viewing test results, any subjective quality change is not observed at the tile boundary.

Introduction

In VTM-5.0, long-tap deblocking filter requires maximum 8 samples for luma and 4 samples for chroma. In order to reduce line buffer requirement, long-tap de-block filter is disallowed at horizontal CTU boundary during horizontal edge filtering. However long-tap filter is still used for vertical edge filtering process even though edge is located in tile boundary and in-loop filter is enabled for across the tile boundary. As shown in figure 1, when a picture is split to multiple tiles. vertical line buffer is required to store information of neighboring tiles, e.g. motion information, block size, transform information, CbF and prediction mode. From this reason, vertical line buffer can be increased more than horizontal line buffer in some cases.

Figure 1, Required line buffer for multiple tile split picture decoding

Proposed method

In this proposal, long-tap deblocking filter is simply disabled at edge which is located at the vertical tile boundary. From this proposal, reduced number of samples are required for line buffer as below:

Table 1. Required sample column for line buffer

| component | VTM 5.0 | proposed |
| --- | --- | --- |
| luma | 8 | 4 |
| chroma | 4 | 2 |

Experimental result

Bitstream generation and subjective test are achieved using sequences which are used for subjective viewing of CE5 under the tile splitting condition as below:

Figure 2. Tile boundary visual quality of reconstructed picture by proposed method (KristenAndSara QP32)

Table 2. Condition for tile split

TileUniformSpacing : 1 NumTileColumnsMinus1 : 1 NumTileRowsMinus1 : 0

Table 3. Sequences and condition for viewing test at CE5-2 category

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Redkayak | 1920x1080p | 30fps | 300 | RA | 32, 37 |
| 02 | Cactus | 1920x1080p | 50fps | 500 | LDB | 32, 37 |
| 03 | FourPeople | 1280x720p | 60fps | 600 | LDP | 32, 37 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 32, 37 |
| 05 | Kimono YCbCr 4:4:4 | 1920x1080p | 24fps | 240 | RA | 32, 37 |
| 06 | EBULupoAndCandleLight YCbCr 4:4:4 | 1920x1080p | 50fps | 500 | LDB | 32, 37 |

Conclusion

Disabling long-tap deblocking filter at vertical tile boundary is proposed. From the subjective test, no noticeable visual quality change is observed. It is recommended to include the proposed restriction into VVC to reduce line buffer size.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).