| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0578 |
| --- | --- |

| Title: | Non-CE5: Long-tap deblocking filter on vertical tile boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Sangheon Lee Junghak Nam Jaehyun Lee Seunghwan Kim | Tel: Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VTM-5.0, long-tap deblock filtering is disabled on horizontal CTU boundary to reduce line buffer size. Hence, 4-row samples need to be stored in line buffer as the same with HEVC. However, 8-coloumn samples are still required to be stored in vertical line buffer when a picture is sliced to multiple tiles and long-tap deblocking filter is enabled for the vertical edge. In order to prevent vertical line buffer increasing over HEVC, the proposed method disallows the long-tap deblock filtering when in-loop filter is enabled across the vertical tile boundary. From viewing test results, any subjective quality change is not observed at the tile boundary.

Introduction

In VTM-5.0, long-tap deblocking filter requires maximum 8 samples for luma and 4 samples for chroma. In order to reduce line buffer requirement, long-tap de-block filter is disallowed at horizontal CTU boundary during horizontal edge filtering. However long-tap filter is still used for vertical edge filtering process even though edge is located in tile boundary and in-loop filter is enabled for across the tile boundary. As shown in figure 1, when a picture is split to multiple tiles. vertical line buffer is required to store information of neighboring tiles, e.g. motion information, block size, transform information, CbF and prediction mode. From this reason, vertical line buffer can be increased more than horizontal line buffer in some cases.

Figure 1, Required line buffer for multiple tile split picture decoding

Proposed method

In this proposal, long-tap deblocking filter is simply disabled at edge which is located at the vertical tile boundary. From this proposal, reduced number of samples are required for line buffer as below:

Table 1. Required sample column for line buffer

| component | VTM 5.0 | proposed |
| --- | --- | --- |
| luma | 8 | 4 |
| chroma | 4 | 2 |

Experimental result

Bitstream generation and subjective test are achieved using sequences which are used for subjective viewing of CE5 under the tile splitting condition as below:

Figure 2. Tile boundary visual quality of reconstructed picture by proposed method (KristenAndSara QP32)

Table 2. Condition for tile split

TileUniformSpacing : 1 NumTileColumnsMinus1 : 1 NumTileRowsMinus1 : 0

Table 3. Sequences and condition for viewing test at CE5-2 category

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Redkayak | 1920x1080p | 30fps | 300 | RA | 32, 37 |
| 02 | Cactus | 1920x1080p | 50fps | 500 | LDB | 32, 37 |
| 03 | FourPeople | 1280x720p | 60fps | 600 | LDP | 32, 37 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 32, 37 |
| 05 | Kimono YCbCr 4:4:4 | 1920x1080p | 24fps | 240 | RA | 32, 37 |
| 06 | EBULupoAndCandleLight YCbCr 4:4:4 | 1920x1080p | 50fps | 500 | LDB | 32, 37 |

The md5 which is achieved decoded yuv are below :

| ClassB_Cactus_32.yuv ClassB_Cactus_37.yuv ClassB_EBULupoCandlelight_32.yuv ClassB_EBULupoCandlelight_37.yuv ClassE_FourPeople_32.yuv ClassE_FourPeople_37.yuv ClassE_KristenAndSara_32.yuv ClassE_KristenAndSara_37.yuv ClassB_Kimono_32.yuv ClassB_Kimono_37.yuv ClassB_Redkayak_32.yuv ClassB_Redkayak_37.yuv | 9e541bc7a2b68e51f4b9f4ed9a2f933b 28244bbd96a18b4d473e73a40775fcf4 598f56521c0489d53d27a0b832926fd5 a5baa0c311e82ed2adc219fb02f18860 cbcf2a478abd769548f08f79af0c1418 658a90087d40b9454a15bd452a11ab7d f6998de28af0cbabd0c02f2788d65b6b 33fcf7aa0b841cb463d69d5ab1ec84d7 2fb999953d4bcf4fed2b5fbfcac02dc2 37b7a7d1618e56871d42e95a8f180414 1f26c1fd5cb6ca88435c6c205e17e01d 6bbc3e05d6a1f627581a5f4c86e7851b |
| --- | --- |

Conclusion

Disabling long-tap deblocking filter at vertical tile boundary is proposed. From the subjective test, no noticeable visual quality change is observed. It is recommended to include the proposed restriction into VVC to reduce line buffer size.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).