| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0579 |
| --- | --- |

| Title: | Non-CE4 : SMVD processing when mvd_l1_zero_flag is set equal to 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Naeri Park Junghak Nam Seunghwan Kim Jaehyun Lim | Tel: Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This proposal points out a mismatch issue between draft text and software implementation. Specifically, in VTM 5.0, sym_mvd_flag is signalled regardless of mvd_l1_zero_flag. However, in draft text, slice level constraint (i.e., mvd_l1_zero_flag) has higher priority than CU level constraint (sym_mvd_flag) so that mvd for reference list 1 is set to 0 when mvd_l1_zero_flag is equal to 1. Therefore, in this proposal, two candidate methods are suggested to be considered as follows:

- Change draft spec to align with SW

- Change SW to align with draft spec

Problem Statement

In VTM 5.0 sym_mvd_flag is signalled regardless of mvd_l1_zero_flag. Therefore, in case 4, sym_mvd_flag is redundantly signalled even though SMVD is not allowed in this case.

|  | mvd_l1_zero_flag | sym_mvd_flag | behavior |
| --- | --- | --- | --- |
| Case 1 | 0 | 0 | No SMVD |
| Case 2 | 1 | 0 | No SMVD |
| Case 3 | 0 | 1 | SMVD |
| Case 4 | 1 | 1 | No SMVD |

Proposed method

Proposed method 1 : change draft spec to align with SW

First suggestion is to change draft spec in order to be aligned with SW. In this case, SMVD is allowed regardless of mvd_l1_zero_flag. This means coding unit level syntax sym_mvd_flag has high priority than slice level constraint.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( cbWidth = = 4 && cbHeight = = 4 && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && sps_ibc_enabled_flag && ( cbWidth != 128 ¦ ¦ cbHeight != 128 ) ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( sps_pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( cbWidth <= 32 && cbHeight <= 32 ) |  |
| intra_bdpcm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_bdpcm_flag[ x0 ][ y0 ] ) |  |
| intra_bdpcm_dir_flag[ x0 ][ y0 ] | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) { |  |
| intra_mip_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_mip_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_mip_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| general_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( general_merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| } else if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| mvd_coding( x0, y0, 0, 0 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_amvr_enabled_flag && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ) ) { |  |
| amvr_precision_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( sps_smvd_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && !inter_affine_flag[ x0 ][ y0 ] && RefIdxSymL0 > −1 && RefIdxSymL1 > −1 ) |  |
| sym_mvd_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( NumRefIdxActive[ 0 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( NumRefIdxActive[ 1 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( sym_mvd_flag[ x0 ][ y0 ] ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = −MvdL0[ x0 ][ y0 ][ 0 ] |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = −MvdL0[ x0 ][ y0 ][ 1 ] |  |
| } else { |  |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0][ y0 ] == PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( x0, y0, 1, 0 ) | ae(v) |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( ( sps_amvr_enabled_flag && inter_affine_flag[ x0 ][ y0 ] = = 0 && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 1 ] != 0 ) ) ¦ ¦ ( sps_affine_amvr_enabled_flag && inter_affine_flag[ x0 ][ y0 ] = = 1 && ( MvdCpL0[ x0 ][ y0 ][ 0 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 0 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 0 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 0 ] [ 1 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 1 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 1 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 1 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 1 ] [ 1 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 2 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 2 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 2 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 2 ] [ 1 ] != 0 ) ) { |  |
| amvr_flag[ x0 ][ y0 ] | ae(v) |
| if( amvr_flag[ x0 ][ y0 ] ) |  |
| amvr_precision_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( sps_bcw_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && luma_weight_l0_flag[ ref_idx_l0 [ x0 ][ y0 ] ] = = 0 && luma_weight_l1_flag[ ref_idx_l1 [ x0 ][ y0 ] ] = = 0 && chroma_weight_l0_flag[ ref_idx_l0 [ x0 ][ y0 ] ] = = 0 && chroma_weight_l1_flag[ ref_idx_l1 [ x0 ][ y0 ] ] = = 0 && cbWidth * cbHeight >= 256 ) |  |
| bcw_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && general_merge_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| if( cbWidth <= MaxSbtSize && cbHeight <= MaxSbtSize ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ¦ ¦ allowSbtVerQ ¦ ¦ allowSbtHorQ ) |  |
| cu_sbt_flag | ae(v) |
| } |  |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_pos_flag | ae(v) |
| } |  |
| } |  |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

mvd_l1_zero_flag equal to 1 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is not parsed and MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to as following:

- if sym_mvd_flag[x0][y0] is equal to 0

MvdL1[ x0 ][ y0 ] is set equal to 0 for compIdx = 0..1.

- otherwise

MvdL1[ x0 ][ y0 ][ 0 ] = ( -MvdL0[ x0 ][ y0 ][ 0 ])

MvdL1[ x0 ][ y0 ][ 1 ] = ( -MvdL0[ x0 ][ y0 ][ 1 ])

mvd_l1_zero_flag equal to 0 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is parsed.

Proposed method 2 : change SW to align with draft spec

In the proposed method 2, mvd for reference list 1 is set to 0 when mvd_l1_zero_flag is equal to 1 even though sym_mvd_flag is set equal to 1. This means that mvd_l1_zero_flag which is signaled in slice header has higher priority than sym_mvd_flag in coding unit level.

Experimental result

The experiment result are achieved for the proposed method 2.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 103% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 98% |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).