| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0582 |
| --- | --- |

| Title: | Non-CE5 : Deblock filtering process for BDPCM block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Sunmi Yoo Seunghwan Kim Jaehyun Lim | Tel: Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

De-block filtering process for BDPCM is proposed in this contribution. In VTM-5.0, boundary strength is set equal to 0 only when both blocks across an edge are coded as BDPCM so that filtering is disabled. Considering that BDPCM is applied for transform skip mode and residual pixels are predicted by sample unit based on DPCM, the deblocking process for BDPCM needs to be considered as PCM block rather than intra coded block where boundary strength equal 2 is applied.. Therefore, in this proposal, no deblocking filtering for BDPCM block is suggested so that DPCM coded transform skip block is not to be deblocked as normal intra block. It is reported that BD-rate changes of the proposed scheme 2 is observed to be 0.01% / -0.02% in Class F and TGM in AI, 0.01 / -0.04% in Class F and TGM in RA, and X.XX / X.XX% in Class F and TGM in LDB , respectively.

Introduction

In VTM-5.0, boundary strength is set equal to 0 when both neighboring blocks of an edge are coded as BDPCM as shown in Fig. 1. From this BS decision, filtering is disabled when only both blocks are BDPCM.

Figure 1. Boundary strength decision of VTM-5.0

Proposed method

Proposed method 1

The proposed method 1 suggests a fixing mismatch between SW and SPEC. Therefore in single tree structure, boundary strength for chroma component is set equal to 0 when both blocks are BDPCM even though BDPCM is not applied for chroma component. The proposed method is that boundary strength is set as Figure 2.

Figure 2. Boundary strength decision of proposed method 1

Proposed method 2

In the proposed method 2, it is suggested that filtering is skipped for BDPCM coded block based on the same method which is being applied for PCM and TrQuantBypassblock. Specifically, suggested method apply normal deblocking process and replace the deblocked pixels with unfiltered samples as shown in Fig. 3.

Figure 3. Filtering process of proposed method 2

- Filtering process for a luma sample using short filters Inputs to this process are: - the sample values pi and qi with i = 0..3, - the locations of pi and qi, ( xPi, yPi ) and ( xQi, yQi ) with i = 0..2, - a variable dE, - the variables dEp and dEq containing decisions to filter samples p1 and q1, respectively, - a variable tC. Outputs of this process are: - the number of filtered samples nDp and nDq, - the filtered sample values pi′ and qj′ with i = 0..nDp − 1, j = 0..nDq − 1. Depending on the value of dE, the following applies: - If the variable dE is equal to 2, nDp and nDq are both set equal to 3 and the following strong filtering applies: p0′ = Clip3( p0 − 3 * tC, p0 + 3 * tC, ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 ) >> 3 ) (8-1131) p1′ = Clip3( p1 − 2 * tC, p1 + 2 * tC, ( p2 + p1 + p0 + q0 + 2 ) >> 2 ) (8-1132) p2′ = Clip3( p2 − 1 * tC, p2 + 1*tC, ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 ) >> 3 ) (8-1133) q0′ = Clip3( q0 − 3 * tC, q0 + 3 * tC, ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 ) >> 3 ) (8-1134) q1′ = Clip3( q1 − 2 * tC, q1 + 2 * tC, ( p0 + q0 + q1 + q2 + 2 ) >> 2 ) (8-1135) q2′= Clip3( q2 − 1 * tC, q2 + 1 * tC, ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 ) >> 3 ) (8-1136) - Otherwise, nDp and nDq are set both equal to 0 and the following weak filtering applies: - The following applies: = ( 9 * ( q0 − p0 ) − 3 * ( q1 − p1 ) + 8 ) >> 4 (8-1137) - When Abs() is less than tC * 10, the following ordered steps apply: - The filtered sample values p0′ and q0′ are specified as follows: = Clip3( −tC, tC, ) (8-1138) p0′ = Clip1Y( p0 + ) (8-1139) q0′ = Clip1Y( q0 − ) (8-1140) - When dEp is equal to 1, the filtered sample value p1′ is specified as follows: p = Clip3( −( tC >> 1 ), tC >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) − p1 + ) >> 1 ) (8-1141) p1′ = Clip1Y( p1 + p ) (8-1142) - When dEq is equal to 1, the filtered sample value q1′ is specified as follows: q = Clip3( −( tC >> 1 ), tC >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) − q1 − ) >> 1 ) (8-1143) q1′ = Clip1Y( q1 + q ) (8-1144) - nDp is set equal to dEp + 1 and nDq is set equal to dEq + 1. When nDp is greater than 0 and one or more of the following conditions are true, nDp is set equal to 0: - pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xP0 ][ yP0 ] is equal to 1. - cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1. - intra_bdpcm_flag[ xP0 ][ yP0 ] is equal to 1. When nDq is greater than 0 and one or more of the following conditions are true, nDq is set equal to 0: - pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xQ0 ][ yQ0 ] is equal to 1. - cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1. - intra_bdpcm_flag[ xQ0 ][ yQ0 ] is equal to 1. - 8.8.3.6.7 Filtering process for a luma sample using long filters Inputs to this process are: - the variables maxFilterLengthP and maxFilterLengthQ, - the sample values pi and qj with i = 0..maxFilterLengthP and j = 0..maxFilterLengthQ, - the locations of pi and qj, ( xPi, yPi ) and ( xQj, yQj ) with i = 0..maxFilterLengthP − 1 and j = 0..maxFilterLengthQ − 1, - a variable tC. Outputs of this process are: - the filtered sample values pi′ and qj′ with i = 0..maxFilterLengthP − 1, j = 0..maxFilterLenghtQ − 1. The variable refMiddle is derived as follows: - If maxFilterLengthP is equal to maxFilterLengthQ and maxFilterLengthP is equal to 5, the following applies: refMiddle = ( p4 + p3 + 2* ( p2 + p1 + p0 + q0 + q1 + q2 ) + q3 + q4 + 8) >> 4 (8-1145) - Otherwise, if maxFilterLengthP is equal to maxFilterLengthQ and maxFilterLengthP is not equal to 5, the following applies: refMiddle = ( p6 + p5 + p4 + p3 + p2 + p1 + 2* ( p0 + q0 ) + q1 + q2 + q3 + q4 + q5 + q6 + 8 ) >> 4 (8-1146) - Otherwise, if one of the following conditions are true, - maxFilterLengthQ is equal to 7 and maxFilterLengthP is equal to 5, - maxFilterLengthQ is equal to 5 and maxFilterLengthP is equal to 7, the following applies: refMiddle = ( p4 + p3 + 2* ( p2 + p1 + p0 + q0 + q1 + q2 ) + q3 + q4 + 8 ) >> 4 (8-1147) - Otherwise, if one of the following conditions are true, - maxFilterLengthQ is equal to 5 and maxFilterLengthP is equal to 3, - maxFilterLengthQ is equal to 3 and maxFilterLengthP is equal to 5, the following applies: refMiddle = ( p3 + p2 + p1 + p0 + q0 + q1 + q2 + q3 + 4) >> 3 (8-1148) - Otherwise, if maxFilterLengthQ is equal to 7 and maxFilterLengthP is equal to 3, the following applies: refMiddle = ( 2 * ( p2 + p1 + p0 + q0 ) + p0 + p1 + q1 + q2 + q3 + q4 + q5 + q6 + 8 ) >> 4 (8-1149) - Otherwise, the following applies: refMiddle = ( p6 + p5 + p4 + p3 + p2 + p1 + 2*( q2 + q1 + q0 + p0) + q0 + q1 + 8 ) >> 4 (8-1150) The variables refP and refQ are derived as follows: refP = ( pmaxFilterLengtP + pmaxFilterLengthP-1 + 1 ) >> 1 (8-1151) refQ = ( qmaxFilterLengtQ + qmaxFilterLengthQ-1 + 1 ) >> 1 (8-1152) The variables fi and tCPDi are defined as follows: - If maxFilterLengthP is equal to 7, the following applies: f0..6 = { 59, 50, 41, 32, 23, 14, 5 } (8-1153) tCPD0..6 = { 6, 5, 4, 3, 2, 1, 1 } (8-1154) - Otherwise, if maxFilterLengthP is equal to 5, the following applies: f0..4 = { 58, 45, 32, 19, 6 } (8-1155) tCPD0..4 = { 6, 5, 4, 3, 2 } (8-1156) - Otherwise, the following applies: f0..2 = { 53, 32, 11 } (8-1157) tCPD0..2 = { 6, 4, 2} (8-1158) The variables gj and tCQDj are defined as follows: - If maxFilterLengthQ is equal to 7, the following applies: g0..6 = { 59, 50, 41, 32, 23, 14, 5 } (8-1159) tCQD0..6 = { 6, 5, 4, 3, 2, 1, 1 } (8-1160) - Otherwise, if maxFilterLengthQ is equal to 5, the following applies: g0..4 = { 58, 45, 32, 19, 6 } (8-1161) tCQD0..4 = { 6, 5, 4, 3, 2 } (8-1162) - Otherwise, the following applies: g0..2 = { 53, 32, 11 } (8-1163) tCQD0..2 = { 6, 4, 2 } (8-1164) The filtered sample values pi′ and qj′ with i = 0..maxFilterLengthP − 1 and j = 0..maxFilterLengthQ − 1 are derived as follows: pi′ = Clip3( pi − ( tC*tCPDi ) >> 1, pi + ( tC*tCPDi ) >> 1, ( refMiddle*fi + refP*( 64 − fi ) + 32) >> 6 ) (8-1165) qj′ = Clip3( qj − ( tC*tCQDj ) >> 1, qj + ( tC*tCQDj ) >> 1, ( refMiddle*gj + refQ*(64 − gj ) + 32) >> 6 ) (8-1166) When one or more of the following conditions are true, the filtered sample value, p i′ is substituted by the corresponding input sample value p i with i = 0..maxFilterLengthP − 1: - pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xPi ][ yPi ] is equal to 1. - cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample pi is equal to 1. - intra_bdpcm_flag[ xP0 ][ yP0 ] is equal to 1. When one or more of the following conditions are true, the filtered sample value, q i′ is substituted by the corresponding input sample value q j with j = 0..maxFilterLengthQ − 1: - pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xQi ][ yQi ] is equal to 1. - cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample qi is equal to 1. - intra_bdpcm_flag[ xQ0 ][ yQ0 ] is equal to 1.]

Experimental result

Proposed method 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 95% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 97% |
| SCC | 0.00% | 0.00% | 0.00% | 102% | 98% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F | 0.00% | 0.01% | 0.00% | 100% | 101% |
| SCC | -0.02% | 0.06% | 0.05% | 101% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class F | -0.01% | -0.26% | -0.29% | 103% | 98% |
| SCC | -0.05% | -0.09% | -0.09% | 101% | 103% |

Proposed method 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class F | 0.01% | 0.00% | 0.00% | 102% | 100% |
| SCC | -0.02% | 0.00% | 0.00% | 101% | 101% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.00% | 0.00% | 105% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 107% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class F | 0.01% | 0.05% | 0.02% | 100% | 100% |
| SCC | -0.04% | 0.02% | -0.01% | 102% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.00% | 0.00% | 103% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 104% | 106% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| SCC | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).