| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0585 |
| --- | --- |

| Title: | Non-CE4 : Merge syntax for removing redundant syntax signaling or fall back merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Naeri Park Junghak Nam Jaehyun Lim Seunghwan Kim | Tel: Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

Merge data syntax in current VVC is being signalled redundantly and has potential problem with decoder crash for non-common test conditions. In this contribution, two solutions are proposed to remove the above mentioned issues.

Method 1: propose updated conditions for each merge syntax signaling to avoid redundant signaling and invalid (undefined) mode

Method 2: fall back merge mode is defined to avoid undefined mode as follows.

a. Merge candidate index 0 without syntax signaling

b. Zero motion without syntax signaling

The method 1 shows -0.20% and -0.22% BD-rate changes for RA and LDB respectively when triangle prediction is disabled. The experimental results of fall back mode by the method 2-a and method 2-b show 0.00%, 0.00% and 0.00% BD-rate change over VTM-5.0.

Introduction

The regular merge flag was adopted in the previous meeting. By this proposal [1], merge mode can cause redundant signaling and decoder crash, because the default merge mode does not exist from the adopted change. In order to prevent these problems, two alternative suggestions are proposed in this contribution.

Proposed method

Method 1

In the method 1, merge mode availability is considered for each merge tool syntax signaling.

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(MMVDAllowed ¦¦ MergeSubBlockAllowed ¦¦ MergeCIIPAllowed ¦¦ MergeTriangleAllowed) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(MMVDAllowed &&(MergeSubBlockAllowed ¦¦ MergeCIIPAllowed ¦¦ MergeTriangleAllowed)) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(MergeSubBlockAllowed && ( MergeCIIPAllowed ¦¦ MergeTriangleAllowed ) ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(MergeCIIPAllowed && MergeTriangleAllowed) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

general_merge_flag[ x0 ][ y0 ] specifies whether the inter prediction parameters for the current coding unit are inferred from a neighbouring inter-predicted partition. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When general_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If cu_skip_flag[ x0 ][ y0 ] is equal to 1, general_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1.

- Otherwise, general_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

If all of the the following conditions are true, the variable MMVDAllowed is set equal to true.

- general_merge_flag[ x0 ][ y0 ] is equal to 1

- sps_mmvd_enabled_flag is equal to 1

If all of the the following conditions are true, the variable MergeSubblockAllowed is set equal to true.

- general_merge_flag[ x0 ][ y0 ] is equal to 1

- MaxNumSubblockMergeCand > 0

- cbWidth is greater than or equal to 8

- cbHeight is greater than or equal to 8

If all of the the following conditions are true, the variable MergeCIIPAllowed is set equal to true.

- general_merge_flag[ x0 ][ y0 ] is equal to 1

- sps_ciip_enabled_Flag is equal to 1

- cu_skip_flag[ x0 ][ y0 ] is equal to 0

- cbWidth * cbHeight is greater than or equal to 64

- cbWidth is smaller than 128

- cbHeight is smaller than 128

If all of the the following conditions are true, MergeTriangleAllowed is set equal to true.

- general_merge_flag[ x0 ][ y0 ] is equal to 1

- sps_triangle_enalbed_Flag is equal to 1.

- slice_type is equal to B

- NaxNumTriangleMergeCand is greater than or equal to 2

- cbWidth * cbHeight is greater than or equal to 64

Method 2-a

The method 2-a suggests fall back merge mode. This fall back merge mode is set to regular merge mode with candidate index 0 without any syntax signaling. merge_idx is inferred and also motion information is derived by regular merge mode in decoding process when none of the merge mode is selected.

merge_idx[ x0 ][ y0 ] specifies the merging candidate index of the merging candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When merge_idx[ x0 ][ y0 ] is not present, it is inferred as follows: - If mmvd_merge_flag[ x0 ][ y0 ] is equal to 1, merge_idx[ x0 ][ y0 ] is inferred to be equal to mmvd_cand_flag[ x0 ][ y0 ]. - Otherwise (mmvd_merge_flag[ x0 ][ y0 ] is equal to 0), merge_idx[ x0 ][ y0 ] is inferred to be equal to 0.

Method 2-b

The method 2-b suggests fall back merge mode which is zero motion information. From this fall back mode, prediction samples can be derived by zero motion without any syntax signaling.

Derivation process for motion vector components and reference indices Inputs to this process are: - a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, - a variable cbWidth specifying the width of the current coding block in luma samples, - a variable cbHeight specifying the height of the current coding block in luma samples. Outputs of this process are: - the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], - the reference indices refIdxL0 and refIdxL1, - the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], - the bi-prediction weight index bcwIdx. Let the variable LX be RefPicList[ X ], with X being 0 or 1, of the current picture. For the derivation of the variables mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], refIdxL0 and refIdxL1, as well as predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], the following applies: - If general_merge_flag[ xCb ][ yCb ] is equal to 1 and all of the regular_merge_flag[ x0 ][ y0 ], mmvd_merge_flag[ x0 ][ y0 ], merge_subblock_flag[ x0 ][ y0 ], ciip_flag[ x0 ][ y0 ] and MergeTriangleFlag[ x0 ][ y0 ] are equal to 0, the following applies: - mvL0[ 0 ][ 0 ][ 0 ] = 0 - mvL0[ 0 ][ 0 ][ 1 ] = 0 - mvL1[ 0 ][ 0 ][ 0 ] = 0 - mvL1[ 0 ][ 0 ][ 1 ] = 0 - refIdxL0 = 0 - refIdxL1 = (slice_type = = B) ? 0 : −1 - predFlagL0[ 0 ][ 0 ] = 1 - predFlagL1[ 0 ][ 0 ] = (slice_type = = B ) ? 1 : 0 - Otherwise If general_merge_flag[ xCb ][ yCb ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in clause 8.5.2.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight inputs, and the output being the luma motion vectors mvL0[ 0 ][ 0 ], mvL1[ 0 ][ 0 ], the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], the bi-prediction weight index bcwIdx and the merging candidate list mergeCandList.

Experimental result

Anchor and the method 1 is tested under the CTC with TPM off. The experimental results are as follows:

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 with TPM off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.28% | -0.28% | 100% | 99% |
| Class A2 | -0.19% | -0.29% | -0.23% | 100% | 100% |
| Class B | -0.18% | -0.16% | -0.12% | 99% | 100% |
| Class C | -0.22% | -0.18% | -0.20% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.22% | -0.20% | 99% | 100% |
| Class D | -0.19% | -0.39% | -0.25% | 100% | 100% |
| Class F | -0.14% | -0.12% | -0.19% | 101% | 101% |
|  | Low Delay B |  |  |  |  |
|  | Over VTM-5.0 with TPM off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.35% | -0.60% | 100% | 100% |
| Class C | -0.20% | -0.43% | -0.30% | 101% | 99% |
| Class E | -0.35% | -0.14% | -0.04% | 101% | 99% |
| Overall | -0.22% | -0.32% | -0.36% | 100% | 100% |
| Class D | -0.28% | -0.59% | -0.85% | 100% | 100% |
| Class F | -0.03% | -0.90% | -1.29% | 101% | 99% |

Reference

- Y.-W. Chen, X. Wang, “CE4: Regular merge flag coding (CE4-1.2.a and CE4-1.2.b),” JVET-N0324, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).