| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0586 |
| --- | --- |

| Title: | Non-CE4: Redundancy removal in merge modes signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Hong-Jheng Jhu, Shuiming Ye, Xianglin Wang | Email: | yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In current VVC, there is syntax redundancy in signaling merge related modes when only a subset of the merge related modes is available. In one example, when triangle mode is disabled by its SPS flag, the CIIP flag is still signaled into the bitstream to indicate whether the current CU is CIIP or triangle mode, which is redundant. Besides, such signaling redundancy may lead to undefined cases with all merge related mode flags parsed as zero. In this proposal, it is proposed to remove the signaling redundancy by adjusting the codeword binarization according to those available merge related modes, so that the undefined cases above are completely avoided. The reported results show that the proposed modifications bring -0.1% to -0.3% BD-rate savings under different test conditions where only a subset of the merge related modes is available.

# Introduction

In current VVC, merge related mode flags are signaled in a cascading order:

regular merge  MMVD  subblock merge  CIIP  triangle merge

Moreover, the flag of the last mode (i.e. triangle mode) is not signaled but inferred from the values of all the previous flags. The signaling scheme is basically identical to assigning the truncated unary codewords to the merge related modes as shown in Table 1.

Table 1 Codewords of the merge related modes in VVC

However, there is signaling redundancy in VTM-5.0 for the cases when only subset of the merge related modes is enabled. Specifically, when regular merge, MMVD, subblock merge or CIIP becomes the last mode in the cascading signaling order, current VVC still signal one flag for the last mode. In one example as shown in Table 2, when triangle mode is disabled by the SPS flag and the other modes are enabled, the CIIP becomes the last mode. According to current design, in this case the CIIP flag is still signaled into the bitstream to indicate whether the current CU is CIIP or triangle mode, which is redundant.

In addition, such signaling redundancy may also lead to undefined cases with all merge related mode flags parsed as zero, as exemplified in Table2.

Table 2 Codewords of the merge related modes when triangle mode is disabled in VVC

# Proposed method to fix the issue

In this contribution, it is proposed that the corresponding codeword binarization is adjusted according to the available merge related modes, so that the issue above can be completely solved.

Specifically, four boolean variables MMVDAllowed, MergeSubblockAllowed, MergeCIIPAllowed and MergeTriangleAllowed, indicating whether each merge mode is allowed for a current coding block, are derived according to the conditions specified in current VVC as follows:

MMVDAllowed = sps_MMVD_enabled_flag;

MergeSubblockAllowed = MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8;

MergeCIIPAllowed = sps_ciip_enabled_flag && cu_skip_flag [ x0 ][ y0 ] = = 0 && (cbWidth*cbHeight) >= 64 && cbWidth <128 && cbWidth <128;

MergeTriangleAllowed = sps_triangle_enabled_flag && MaxNumTriangleMergeCand >= 2 && (cbWidth*cbHeight) >=64;

The flag of each merge-related mode is signaled in the bitstream only when the corresponding condition is true and it is not the last merge mode. The proposed syntax changes are indicated below.

#### Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(MMVDAllowed ¦¦ MergeSubblockAllowed ¦¦ MergeCIIPAllowed ¦¦ MergeTriangleAllowed) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(MMVDAllowed && (MergeSubblockAllowed ¦¦ MergeCIIPAllowed ¦¦ MergeTriangleAllowed)) |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(MergeSubblockAllowed && (MergeCIIPAllowed ¦¦ MergeTriangleAllowed)) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MergeCIIPAllowed && MergeTriangleAllowed) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

In case a certain merge mode is allowed but the corresponding flag is not signaled, the value of the corresponding flag is inferred. If the previously signaled merge mode flags for the current CU are all false, the value of the corresponding flag is inferred as true. Otherwise, the value is inferred as false.

Taking the case when triangle mode is disabled by its SPS flag and the other modes are enabled as an example, the codeword for each mode according to the proposed scheme is illustrated in Table 3. In this case, CIIP mode is allowed but the CIIP flag is not signaled. The value of CIIP flag is inferred as 1 if the signaled Regular merge, MMVD and subblock flags are all 0. Otherwise the value of CIIP flag is inferred as 0.

Table 3 Proposed codewords of the merge related modes when triangle mode is disabled

# Results

The proposed modification is integrated into VTM-5.0 [2] and tested under the 4 different conditions against the VTM-5.0 as shown in Table 4. More detailed results are available in the attached Excel files. With the syntax redundancy removed, the proposed modifications not only prevent signaling resulting in un-defined cases, but also bring -0.1% to -0.3% BD-rate saving under these test conditions.

Table 4. The BD-rate saving of the proposed modifications against VTM-5.0

| Conditions | Regular:On MMVD:On SubBlk:On CIIP:On Triangle:Off | Regular:On MMVD:On SubBlk:On CIIP: Off Triangle: Off | Regular:On MMVD:On SubBlk: Off CIIP: Off Triangle: Off | Regular:On MMVD: Off SubBlk: Off CIIP: Off Triangle: Off |
| --- | --- | --- | --- | --- |
| RA | -0.20% | -0.27% | -0.08% | -0.11% |
| LDB | -0.22% | -0.32% | -0.18% | -0.23% |

# Conclusion

In this proposal, we propose modifications to the signaling of merge related modes. The proposed scheme solves the signaling redundancy issue, and also may bring -0.1% to -0.3% BD-rate saving in case only a subset of the merge related modes is available. It is recommended to adopt the proposed modifications into VVC.

# Reference

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-M1010, January 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).